The VPLOT Combination User's, Installation, and Reference Manual

for Version 2.0

by

M. Gooley

Millikin University Computer Center

Millikin University

Decatur, Illinois 62522

Copyright (C) 1981 by M.  Gooley.  This manual and the  software

it describes are distributed without cost, and may be reproduced

only with the inclusion of this copyright statement.  The author

assumes no responsibility for the accuracy of this manual or the

performance of the software.

VPLOT Guide                                                       Page 2

I.  What is VPLOT?

VPLOT is a package  of  VAX-11  FORTRAN  and  VAX-11  MACRO

routines  that  enables a PRINTRONIX 300 printer to mimic a drum

plotter (the Calcomp Model  565,  according  to  my  circa  1969

information).  VPLOT includes these files:

VPLOT.FOR:    Imitations of CalComp's Basic Software:

Subroutines and Entries:

PLOTS - initialisation and output

PLOT - single lines, pen motions, output

FACTOR - changes scale of plot

WHERE - tells user's program the current "pen"

location and scaling-factor

SYMBOL - draws single symbols or character strings

NUMBER - draws numbers of given precision

SCALE - scales data arrays to axes

AXIS - draws numbered, titled axes

LINE - draws line between data points given in two

arrays

[plus sundry sub-subroutines and functions]

Copies of bits of Calcomp's Functional Software:

Subroutines:

CIRCL - draws circles, Archimedean spirals, and

circular arcs

ELIPS - draws ellipses and elliptical arcs

GRID - draws rectangular grids which are guaranteed

to cause eyestrain

POLY - draws regular polygons and stars

DASHP - resembles PLOT but draws dashed lines

DASHL - resembles LINE but draws dashed lines

RECT - draws rectangles

POLAR - resembles LINE but works in polar

coordinates

[plus sundry sub-subroutines and functions]

MACPLOT.MAR: Three VAX-11 MACRO routines:

INITIAL - INITIALisation; also sets up a table to

store addresses of parts of a huge phantom matrix.

LOAD - loads data into the phantom matrix bit by

bit, enlarging the matrix when necessary.

DUMP - converts matrix data to the characters that

the PRINTRONIX understands.

[More about the "phantom matrix" later in part II.]

CHARGEN.DAT:   A  data  file,  about   five   blocks   long,

containing CHARacter GENerator DATa for SYMBOL.

Other assorted VAX-11 FORTRAN, BASIC,  and  PASCAL  programs

and  subroutines:   illustrations  for  this manual,

demonstrations of  how  to  call  VPLOT  from  other

languages, useless pictures.

VPLOT.DOC:  This manual.

VPLOT Guide                                                       Page 3

II.  How does it work?

A PRINTRONIX P300 can print 132 columns, each consisting of

six dots, over 13.2 inches--792 dots across the width of 14-inch

paper at a density of 60 dots per inch.  Dot density parallel to

the  length of the paper is higher:  72 dots per inch.  Dots are

printed in horizontal rows;  each such  row  is  produced  by  a

string  of  characters sent to the printer in the usual way, but

followed by the ASCII character of value 5 [ENQ].  This puts the

printer into "Plot Mode";  each character is then interpreted as

six dots corresponding  to  its  least  significant  bits;   the

seventh  bit  must  be a one.  For example, the character "A" is

01000001, or five "blanks" followed by a dot;  "C" is  01000011,

four  blanks  followed  by  two  dots;   and "P" is 01010000:  a

blank, a dot, then four more blanks.  Note  that  the  bits  are

interpreted  in  order  from  most-  to least- significant.  The

"legal" characters in Plot Mode, then, have decimal values  from

64  (or  "@",  which  serves  as  a  blank) to 127 (the <DELETE>

character,  which  draws  a  six-dot  horizontal  line   [DELETE

presents  a  problem.   More  on  that later.]).  See PRINTRONIX

Application Note No.  102370, a booklet entitled "Using the Plot

Mode", for general information.

How are the proper character strings produced?  A record of

pen  movements  must  be  kept,  and, when the plot is complete,

translated into a series of strings.  One method is to store the

starting  and  ending  coordinates  of  each  line "drawn" by an

imaginary "pen", putting them into an array or data file;   this

saves  memory  (eight bytes can represent one line) but is slow:

essentially, the entire record must be searched to  create  just

one character string.  At the other extreme, a huge matrix could

provide an electronic simulacrum of a sheet of paper.  with  one

bit  representing  one  dot  (or  one character six dots).  This

seems wasteful, even with the "unlimited" virtual memory of  the

VAX.

VPLOT uses neither method.  It divides the plot  area  into

small  blocks, 24 dots wide and 40 long (120 bytes apiece), each

representing a 2/5" by 5/9" area of paper.  The user's  call  to

the  initialisation  routine  PLOTS  gives  VPLOT the length and

width of the proposed plot and thereby  the  maximum  number  of

blocks  needed.   The  MACRO  routine INITIAL calls the run-time

procedure  LIB$GET VM  (GET  Virtual  Memory.   See  the  VAX-11                          _

Run-Time  Library  Reference  Manual  for details) and obtains a

longword (four bytes, the amount of memory a VAX needs to  store

an address) of virtual memory for each block.  Later in the run,

various subroutines call LOAD, asking it  to  load  a  bit  into

what,  as  far  as  they  are  concerned,  is  a giant matrix of

individually addressable bits.  LOAD determines what  block  the

bit  would be in and checks its corresponding longword;  if this

longword has the value zero, the block does not yet  exist,  and

to  create  it, LOAD calls LIB$GET VM for another 120-byte block                                          _

of memory, storing the lowest address of this new block  in  the

longword.   Then LOAD writes a one into the appropriate bit.  In

this way, sections of the matrix that are  not  used  are  never

created.   Finally,  DUMP  translates  the blocks into character

strings;  each three-byte row of  a  block,  then,  yields  four

characters,  and each block contributes to forty dot-rows.  When

VPLOT Guide                                                       Page 4

DUMP is finished with a block, it returns  it  to  the  bank  of

virtual  memory  using LIB$FREE VM.  In this way, virtual memory                                       _

is used for as short a time as possible.

III.  Necessities, Conventions, &c.

The device handler for your  PRINTRONIX  must  not  convert

lower- to uppercase letters, as this erases the first dot of the

six each character represents;  it must also  consider  <DELETE>

(decimal  127)  a symbol and not a command to erase the previous

character.  Millikin's handler doesn't do  the  latter,  but  is

tricked  into  passing  <DELETE>  by a set eighth bit.  In other

words, DUMP outputs the character as decimal 255, or a  byte  of

all  ones  (11111111),  rather  than 127 (or 01111111).  If your

handler works properly without this subterfuge,  you  can  speed

the  program  by  removing several clearly marked lines in DUMP;

if the trick fails, check your handler board  and  its  manuals.

The  board  should  have  wire  jumpers  or  DIP  switches whose

resetting will enable it to pass <DELETE>.

The maximum width of a VPLOT plot is 13.2 inches;  however,

plotting in the last tenth of an inch is not recommended, making

the actual width 13.1 inches.  The maximum length depends on the

sparseness of the plot, your VAX's memory and disk space, and so

on.  A far-from-sparse plot 13 inches  wide  and  100  long  ran

without  incident  on  Millikin's  VAX-11/780, which is equipped

with 2 MB of memory and two RMO3 disks.   Plotting  outside  the

space  you have declared will not result in run-time errors, or,

for that matter, lines--it's merely a waste of time.

In the following descriptions of the subroutines, parameter

names  starting  with  the  letters I through N denote INTEGER*4

variables;  all other parameters  are  REAL*4.   Use  conversion

functions  such  as  JNINT  and  SNGL when necessary.  Also note

that, by convention, the Y-axis of a plot  is  parallel  to  the

width  of  the paper and the X-axis to its length;  zero degrees

points along the X-axis into the paper box.

VPLOT Guide                                                       Page 5

IV.  The "Basic Software" Routines

PLOTS:

Since PLOTS initializes the  entire  package,  it  must  be

called before any other VPLOT routine, and after the output call

to PLOT if plotting is to continue in a new field.  PLOTS clears

certain variables and calls INITIAL to set up the phantom matrix

and the MACRO routines.  The call is:

CALL PLOTS (XLONG, YWIDE, LDEV)

XLONG is the length of the plot in inches.  The

complexity  of your plot and the memory size of your

VAX will place limits on it (see part III.).

YWIDE is the width of the plot in inches.   The

maximum  value  is  13.2;   anything  larger will be

truncated.

LDEV is the number  of  the  output  device  or

file,  generally  a file named FORnnn.DAT, where nnn

is a number between 2 and 999, with certain  numbers

reserved (5,6, etc.) for system files and 1 reserved

for the data file CHARGEN.DAT.  For example, an LDEV

of 3 would yield the file FOR003.DAT.

PLOT:

PLOT gives the  calling  program  direct  control  over  an

imaginary  pen;   it lets the program "move" the pen, "raise" or

"lower" it (a raised pen draws no lines), change the location of

the  origin  (the point from which all pen motions are defined),

and ask to stop plotting and output a data file.   The  call  to

PLOT has three arguments:

CALL PLOTS (XPAGE, YPAGE, IPEN)

XPAGE and YPAGE are the X-  and  Y-coordinates,

in  inches from the current origin, to which the pen

is to be moved.  Initially, the  origin  is  in  the

upper  left-hand  corner  of  the  paper, but may be

moved to any point in or outside of the plot area.

IPEN:  If IPEN is 2 or  -2,  the  pen  is  down

during  movement  and draws a line.  If IPEN is 3 or

-3, the pen is  up  during  movement.   If  IPEN  is

negative,  the  end-position given by (XPAGE, YPAGE)

becomes the new origin:  its coordinates are set  to

(0.0, 0.0) and it is the reference point for further

pen motions.  If IPEN equals 999, the output file is

loaded  with  the  results  of the plot so far.  The

"999" call must be  either  the  last  call  of  the

program or be followed by a new call to PLOTS.  Note

that this new call may ask for a  plotting  area  of

different size or for a new output file.

VPLOT Guide                                                       Page 6

FACTOR:

FACTOR changes the size of the  plot;   all  pen  movements

after  a  call  to  it are multiplied by its argument.  Repeated

calls are cumulative;  for instance, a call with "2.0"  followed

by  one  with  "3.0" will result in a magnification of six.  The

FACTOR call:

CALL FACTOR (FACT)

If FACT is 2.0, all subsequent pen motions will

be  twice  as  large  as  before;   if  FACT is 3.0,

thrice, &c.  A FACT of 1.0 returns plotting  to  its

normal  size.   While  a  program is being debugged,

much time and paper can be saved  by  following  the

initial PLOTS call with a FACTOR call with FACT less

than 1.0, which can be removed later.

WHERE:

The WHERE entry sets the arguments in the call to the pen's

current X, Y coordinates and the present scaling-factor:

CALL WHERE (RXPAGE, RYPAGE, RFACT)

RXPAGE and RYPAGE are variables  that  will  be

set to the pen's present coordinates.

RFACT is set to the current scaling factor (1.0

if FACTOR has not been called).

NOTE:  a call to WHERE made after a  call

to  SYMBOL  might  not indicate where the next

character  should   be   plotted.    Use   the

"continuation feature" of SYMBOL (which see).

VPLOT Guide                                                       Page 7

SYMBOL:

This routine writes symbols  of  almost  any  size  at  any

angle.  The call looks like this:

CALL SYMBOL (XPAGE, YPAGE, HEIGHT, IBCD, ANGLE, NCHAR)

XPAGE and YPAGE are the coordinates, in inches,

of  the centre of the first character to be plotted.

To continue annotation from the  position  at  which

the  last  annotation ended, give XPAGE and/or YPAGE

as  999.0;   this  may  be  done   with   X   or   Y

independently.  (Calling WHERE and then using RXPAGE

and RYPAGE in another call to SYMBOL might not yield

the  same  results as using 999.0, especially if the

previous call to SYMBOL had a  negative  NCHAR  [see

below].)

HEIGHT  is  the  height,  in  inches,  of   the

characters  to  be  plotted.   For  best results, it

should be at least 0.1, and a multiple of 0.1  (0.1,

0.2,   0.3,   &c.),  but  any  number  over  0.1  is

acceptable.   Character  width  (including  spacing)

equals character height;  therefore, a ten-character

string of 0.2" letters should be about 2.0" wide.

NOTE:    Plotting   small   (under   .2")

characters  at  angles which are not multiples

of  90  degrees   is   not   suggested:    the

dot-matrix   of   the   PRINTRONIX  lacks  the

resolution  needed  for  legibility   at   odd

angles.  Even symbols as large as 0.3" or 0.4"

may have a rather shaky appearance.  Use  your

own discretion.

IBCD is the text to be plotted.  This should be

a character string, such as 'ECCLES', or a character

string variable;  in the latter  case,  a  substring

such  as  CSTRNG(2:7)  may  be  used rather than the

whole of CSTRNG (see  the  VAX-11  FORTRAN  Language

Reference   Manual   for   more  about  substrings).

Hollerith constants are not allowed.

There are a number  of  centred  plot  symbols,

numbered  from  0  to  17,  that may be used to mark

points of a  graph;   to  plot  one  of  these,  use

CHAR(n)  for  IBCD,  where  n is the symbol's number

(see the text on NCHAR, below).

ANGLE is the angle, in degrees from the X-axis,

at  which  the string is to be plotted.  If ANGLE is

0.0, the character(s) will be drawn  right  side  up

and parallel to the X-axis.

VPLOT Guide                                                       Page 8

NCHAR is, in CalComp software,  the  number  of

characters  of  IBCD  that  are  to be plotted;  for

instance,  an  NCHAR  of  5  would  plot  only  five

characters  of  a  ten character string.  VPLOT uses

VAX-11 FORTRAN's implicit passing of string lengths;

therefore,  for  drawing strings, NCHAR need only be

positive.  For one  of  the  special  numbered  plot

symbols  (or any symbol being used to mark a point),

NCHAR must be negative.  If NCHAR  is  -1,  the  pen

moves  to  the  given location and draws the symbol;

if NCHAR is -2, the pen is down during the move  and

draws a line to the point as well.

NOTE:  After a call  in  which  NCHAR  is

negative,  the  pen is at XPAGE, YPAGE;  after

one with a positive NCHAR, it's in the  centre

of a nonexistent "next character."

NUMBER:

NUMBER converts a floating point  ("REAL*4")  number  to  a

form  that  is  plotted in what resembles FORTRAN F-format.  The

calling sequence:

CALL NUMBER (XPAGE, YPAGE, HEIGHT, FPN, ANGLE, NDEC)

XPAGE, YPAGE, HEIGHT, & ANGLE are the  same  as

in  SYMBOL;  the "999" continuation feature may also

be used.

FPN is the floating point number to be plotted;

it  must be REAL*4.  If it isn't, use functions such

as SNGL, FLOATI, and FLOATJ to convert it.

NDEC controls  the  precision  of  the  plotted

number.  If NDEC > 0, NDEC digits will be plotted to

the right of the decimal point--rounded, of  course.

For  example,  the  number  -0.12345678 E03, with an

NDEC of 2, would be plotted as -123.46.

If NDEC = 0, only the number's integer  portion

(rounded) and a decimal point are plotted;  the same

holds for NDEC = 1, except that there's  no  decimal

point.   If  NDEC  < -1, |NDEC|-1 (absolute value of

NDEC, minus  one)  digits  are  truncated  from  the

integer portion (still rounded).

NOTE:  |NDEC| should not exceed  9;   the

maximum  number  of  plotted  characters  in a

number will not  exceed  20.   REAL*4  numbers

which  cannot  be  plotted  in  the  requested

format will cause overflow:  the  plotting  of

ten asterisks.  The call is roughly equivalent

to an F20.<NDEC> format, with  leading  blanks

suppressed;   in  fact,  NUMBER  uses the same

Run-Time Library routine as does FORTRAN.

VPLOT Guide                                                       Page 9

SCALE:

Data to be plotted on a pair of axes can be  stored  as  as

two  arrays,  one  of X-values and one of Y-values.  Rarely will

these X and Y values correspond to the length of  an  axis,  and

the user may find it troublesome (or impossible, if the range of

his data isn't known beforehand) to provide conversion factors.

Enter SCALE, which scans  data  in  arrays,  determining  a

starting  value  for  the  axis  (a  minimum  or  maximum) and a

units-per-inch scaling factor  (positive  or  negative).   These

ensure  that  each  labeled mark on an axis is correctly placed,

and that the data points, when plotted by the LINE routine, will

fit  in  a given area.  SCALE stores these two values at the end

of an array.

DELTAV, the scaling factor, is in units/inch of  axis,  but

is adjusted so that it is 1, 2, 4, 5, or 8 x10^n.  For instance,

given an array with values ranging from 1250 to 3800 which is to

be  plotted over a ten-inch axis, SCALE would first calculate an

"unadjusted" factor of (3800-1250)/10 = 255 units per inch;   it

would  then  assign the largest even value to DELTAV:  400 units

per inch.

FIRSTV, the starting value, is a multiple of DELTAV that is

equal to or outside the limits of the data in the array.  In the

example above, a minimum FIRSTV would be 1200;   a  maximum  one

would be 4000.  (Note that with a FIRSTV of 1200, the far end of

the axis would be 5200, and the last 3.5" of the axis  would  be

wasted.   SCALE excels at such paper-wasting, and this behaviour

probably is not peculiar to its VPLOT version.   Often  all  the

data  points of a graph will be crammed into the first fourth of

an axis.  You have been warned!)   The calling sequence:

CALL SCALE (ARRAY, AXLEN, NPTS, INC)

ARRAY  is  the  array  of  data  points  to  be

examined.

AXLEN is the length of the axis  to  which  the

data is being scaled;  it must be greater than 1.0.

NPTS is the number of data points which are  to

be  examined.  This is not necessarily the number of

points in the array;  the array will generally  have

NPTS*|INC| points in it, plus several more locations

for scaling-factors (see below).

INC's  magnitude  is  used  by  SCALE  as   the

increment with which to select values from the array

for scanning.  Usually INC = 1;  if  it  is  2,  for

instance,  every  other  location  of  the  array is

examined.

If  INC  is  positive,  FIRSTV  approximates  a

minimum and DELTAV is positive;  if INC is negative,

FIRSTV is a maximum and DELTAV is also negative.

VPLOT Guide                                                      Page 10

If INC = 1 or -1, the array must  be  at  least

two elements larger than it would need to be for the

data alone.  In general, FIRSTV is stored  at  ARRAY

(NPTS*INC+1) and DELTAV at ARRAY (NPTS*INC+INC+1).

Generally, SCALE  is  called  to  examine  each

array whose data will be plotted;  if the user knows

the range of his data values he need not call  SCALE

for  an array, so long as he supplies an appropriate

FIRSTV and DELTAV for AXIS and LINE.

AXIS:

Most graphs require labelled axes;  this  routine  draws  a

line  at  any angle, divides it into inch-long segments with tic

marks, labels the marks, and writes a centred title.  When  both

X-  and  Y-axes  are  needed, AXIS is called separately for each

one.  The call has eight arguments:

CALL AXIS (XPAGE, YPAGE, IBCD, NCHAR, AXLEN, ANGLE, FIRSTV,

DELTAV)

XPAGE and YPAGE are the coordinates, in inches,

of  the axis line's starting point.  The entire line

and its terminal ends should be  at  least  one-half

inch  from  any edge of the paper to allow space for

labels and titles.  Usually the X-  and  Y-axes  are

joined  at the origin of the graph (0.0,0.0), though

other points may be used instead.  (See note at  end

of LINE for the correct use with that routine.)

IBCD is the title, which is centred and  placed

parallel  to  the  axis line.  The characters have a

fixed height of  about  0.2"  (five  characters  per

inch).  See SYMBOL for more about IBCD.

NCHAR, in Calcomp software, gives the number of

characters in the axis title, but its exact value is

not important to VPLOT.  If NCHAR is  positive,  all

axis  labels  (tic  marks, numbers, title) appear on

the positive (counterclockwise) side  of  the  axis,

which  is  usual  for the Y-axis;  if it's negative,

the labels will be on the negative side (X-axis).

AXLEN is  the  length  of  the  axis  line,  in

inches.

ANGLE is the angle at which the axis is  to  be

drawn,  normally  zero degrees for the X-axis and 90

degrees for the Y-axis.

NOTE:  Plotting axes at angles which  are

not  multiples  of  90  degrees  may result in

illegible tic-mark numbers.

VPLOT Guide                                                      Page 11

FIRSTV  is  the  starting  value  (minimum   or

maximum)  which  will  appear at (XPAGE, YPAGE).  It

may   be   computed   by   SCALE   and   stored   at

ARRAY(NPTS*INC+1),  or may be determined by the user

and stored anywhere.  This number  and  every  scale

value along the axis is drawn with two digits to the

right of the decimal point.  Since the digit size is

fixed  at about 0.1" (about ten characters per inch)

and one scale value appears every inch, no more than

six  digits  and a sign should appear to the left of

the decimal point.

DELTAV represents the number of data units  per

inch  of axis.  This value (increment or decrement),

which  is  added  to  FIRSTV  for  each   succeeding

one-inch  division  of  the axis, may be computed by

SCALE    and     stored     beyond     FIRSTV     at

ARRAY(NPTS*INC+INC+1), or determined by the user and

stored anywhere.  In order to use a standard format,

the size of DELTAV is adjusted to be between 100 and

0.01.  As a result, the decimal point may be shifted

left  or  right  in  the scale values as drawn;  the

axis title will then be followed by "10^n," where  n

is a power-of-ten adjustment factor.

LINE:

The LINE subroutine produces a line plot of pairs  of  data

points  given  in  two arrays (X and Y).  LINE computes the page

coordinates of each point according to its X- and  Y-values  and

scaling  factors;  the data points may be represented by centred

symbols, and/or lines which connect them.

The scaling factors FIRSTV and DELTAV must be at the end of

each  array;   if they have not been computed by SCALE they must

be supplied by the user.

CALL LINE (XARRAY,YARRAY,NPTS,INC,LINTYP,INTEQ)

XARRAY and YARRAY contain abscissa and ordinate

values, respectively, and the X and Y scaling values

(FIRSTV and DELTAV for each dimension).   These  two

arrays  must  be  one-dimensional  and have the same

number of elements.

NPTS is the number of data points to  be  taken

from   each  of  the  above  arrays;   this  is  not

necessarily the number of data points in each  array

(see  SCALE).   NPTS  should be the same as the NPTS

used for any previous call to SCALE.

INC is the increment that LINE is  to  use  for

gathering  data  from  the  two arrays, as described

earlier for SCALE.

NOTE:  For most applications, INC  is  1;

i.e.,  every  point  in  the  arrays is a data

point, and each array has (NPTS+2)  locations,

the last two containing FIRSTV and DELTAV.

VPLOT Guide                                                      Page 12

LINTYP determines what sort of a line  will  be

drawn  through  the  data  points.  The magnitude of

LINTYP  determines  the  frequency  of  the  plotted

symbols;   for  instance,  if  LINTYP=+4  or  -4,  a

special symbol  is  plotted  at  every  fourth  data

point.

If  LINTYP=0,  the  points  are  connected   by

straight lines but no symbols are plotted.

If LINTYP>0, a  straight  line  connects  every

data point gathered using INC (there will be NPTS of

them).  The pen is up while moving to the first data

point.

If LINTYP<0, no  connecting  lines  are  drawn;

only symbols are plotted.

INTEQ is the integer equivalent of the  special

plotting  symbol  which  is to be drawn (centred) at

each data point.   INTEQ  is  usually  zero  through

eighteen,  but  may be any other value corresponding

to a symbol (64 through 127);  however,  the  latter

symbols are not always centred about the data point.

One sample program included  with  VPLOT  plots  its

complete character set;  refer to its output.

NOTE:  When plotting lines on a  pair  of

axes,  be  certain  that the starting point of

both axes  (the  "XPAGE,YPAGE"  given  in  the

calls  to  AXIS)  is  set to (0.0,0.0);  if it

isn't, the axis labels will  not  reflect  the

actual  values  of  the  points.  For a single

X-axis, the starting point's X-coordinate must

be  0.0;   for a single Y, the Y- must be 0.0.

The starting-point may be reset  to  (0.0,0.0)

by  this  call:    CALL PLOT (X,Y,-3), where X

and Y are the coordinates of the point.

VPLOT Guide                                                      Page 13

V.  The "Functional Software" Routines

(The descriptions of these routines are rather sketchy.   I

assume  that  you are already familiar with the "Basic Software"

set.)

CIRCL:

CIRCL draws circular arcs  from  a  given  starting  point.

These arcs may form circles and Archimedean spirals.

CALL CIRCL (XPAGE,YPAGE,THO,THF,RO,RF,DI)

XPAGE and YPAGE are the coordinates, in inches,

of the starting point.

THO is the angle from the X-axis,  measured  in

degrees along a circle of radius RO, of the start of

the arc.

THF is the angle from the X-axis,  measured  in

degrees  along  a circle of radius RF, of the end of

the arc.

RO and RF are the starting  and  ending  radii,

respectively, of the arc.

DI:  If DI is 0.0, a solid arc is drawn;  if DI

is 0.5, a dashed arc is drawn.

To draw a circle, set both RO and RF equal to the radius in

inches,  and  make THO 360 degrees smaller (or larger) than THF.

To draw an Archimedean spiral, set RO=0.0, RF=the final  radius,

THO=0.0,  and THF=some large number of degrees.  THO and THF may

be  positive  or  negative;   if  THO<THF,  the  arc  is   drawn

counterclockwise,  else  clockwise.   If  THO=THF, or nearly so,

errors may  occur.   Since  each  dot  of  an  arc  is  computed

separately,  CIRCL  is  rather wasteful of CPU time;  also, some

dots of a circle may seem superfluous.

DASHL:

DASHL is much like LINE except that it draws a dashed  line

(the  dashes  are  about  0.1"  long)  instead  of a unbroken or

nonexistent one and it does not plot centred symbols at  points.

Consequently, its call is shorter:

CALL DASHL (XARRAY,YARRAY,NPTS,INC)

XARRAY and YARRAY are the abscissa and ordinate

arrays for the points to be plotted.

NPTS is the number of data points to be plotted

(normally,  this  is  the same as the number of data

points in the array.  See SCALE and LINE).

VPLOT Guide                                                      Page 14

INC is the increment between the  point  values

in the arrays;  usually INC=1.

DASHP:

This routine draws a dashed line from the  "pen's"  current

position  to a specified one.  If the line is not at least twice

as long as the given dash length, dashes half the length of  the

line result.  DASHP is rather like PLOT:

CALL DASHP (XPAGE,YPAGE,DASH)

XPAGE,YPAGE are the coordinates of the point to

which the dashedline will be drawn.

DASH is the length of each dash and  the  space

between  dashes.   It should be greater than 1/60 of

an inch.

ELIPS:

ELIPS draws ellipses  or  elliptical  arcs.   It  resembles

CIRCL,  but  is  even slower.  Large plots containing many large

ellipses will consume vast amounts of CPU time;  several library

functions  (SIN,  COS,  etc.),  must be called for each dot (the

mathematical theory behind the method is correct, but  not  very

practical).  Nevertheless:

CALL ELIPS (XPAGE,YPAGE,RMAJ,RMIN,ANGLE,THO,THF,IPEN)

XPAGE and YPAGE  are  the  coordinates  of  the

starting point of the ellipse or arc.

RMAJ and RMIN are the lengths  (as  always,  in

inches)   of   the  semimajor  and  semiminor  axes,                                        ____            ____

respectively.

ANGLE is  the  angle  of  the  major  axis,  in

degrees.

THO and THF are the angles, in degrees from the

major  axis,  of  the  arc's  initial  and  terminal

points.

NOTE:  THO and THF are NOT measured  with                                                      ___

respect to the centre of the ellipse, but from

the  focus  on  the  "positive"  side  of  the

ellipse.   Imagine  plotting an entire ellipse

with ANGLE=0.0;  the focus about which  angles

are measured would be the one nearer the paper

box.  This is not the  same  as  the  original

CalComp routine, unfortunately.

IPEN is the same as in PLOT.

VPLOT Guide                                                      Page 15

GRID:

GRID draws linear grids--customized "graph paper" which  is

guaranteed  to cause eyestrain.  It may be used with SCALE, AXIS

and LINE to make graphs.

CALL GRID (XPAGE,YPAGE,DELTAX,DELTAY,NXSP,NYSP)

XPAGE and YPAGE are the coordinates, in inches,

of the grid's lower left corner.

DELTAX is the distance between  grid  lines  in

the X direction.

DELTAY is the distance between  gris  lines  in

the Y direction.

NXSP and NYSP are the numbers of spaces between

lines in the X and Y directions, respectively.

NOTE:  NXSP+1 lines are drawn  in  the  X

direction, NYSP+1 in the Y.

POLY:

This routine draws equilateral polygons and stars.

CALL POLY (XPAGE,YPAGE,SLEN,SN,ANGLE)

XPAGE and YPAGE  are  the  coordinates  of  the

starting point.

SLEN is the length of each side of the polygon.

SN is the number of sides in the  polygon;   if

SN  is  negative,  a  star  of  SN  points  is drawn

instead.

ANGLE is the angle, in degrees,  of  the  first

side of the polygon.

RECT:

RECT draws rectangles of any reasonable length and width at

any angle.  The call is:

CALL RECT (XPAGE,YPAGE,HEIGHT,WIDTH,ANGLE,IPEN)

XPAGE  and  YPAGE  give  the  position  of  the

rectangle's lower left corner (before rotation).

HEIGHT and WIDTH are self-explanatory.

ANGLE  is  the  angle,  in  degrees,   of   the

rectangle's  base  (the  rectangle is rotated around

XPAGE, YPAGE).

VPLOT Guide                                                      Page 16

IPEN is the same as for PLOT.

POLAR:

POLAR is much like LINE, except  that  it  works  in  polar

coordinates,  plotting  a radial variable against an angular one

(given in radians), and can do its own scaling.  The call is:

CALL POLAR (RADAR,ANGAR,NPTS,INC,LINTYP,INTEQ,RMAX,DR)

RADAR and ANGAR are the arrays  containing  the

radial and angular values, respectively.

NPTS is the number of data points to be plotted

(recall the information given in SCALE and LINE).

INC is the increment between data values in the

arrays, as in LINE.

LINTYP is also the same as in LINE.

INTEQ:  See LINE.

RMAX is the maximum  radius  for  the  plotting

area that the polar plot will occupy.  If RMAX<=0.0,

DR is used as a scale factor.

NOTE:  In the original CalComp version of

POLAR,  RMAX  was  in  page inches:  inches of                                                 ____ ______

paper, which are not influenced  by  calls  to

FACTOR.   In VPLOT, RMAX is in inches, and its

value will  be  multiplied  by  FACTOR  before

being  used.   The  same  holds for DR:  it is

units of data per inch rather  than  per  page

inch.

DR is the scale factor;  POLAR computes  it  if

RMAX>0.0;  otherwise the user must supply it.

VPLOT Guide                                                      Page 17

VI.  How to compile, link and  run  VPLOT  with  VAX-11  FORTRAN

programs:

$ FORTRAN MAIN1

$ FORTRAN VPLOT

$ MACRO MACPLOT

$ LINK MAIN1+VPLOT+MACPLOT

$ RUN MAIN1

VPLOT and MACPLOT may be compiled beforehand and placed  in

a  system  directory  as object files;  a short command language

program could be used to link them with the main program and run

the result.  The file CHARGEN.DAT is the character-generator for

SYMBOL;  if it is also placed in a directory such  as  [SYSEXE],

the  OPEN  statement  in  the SYMBOL subroutine of VPLOT must be

modified.  At present it looks like this:

OPEN (UNIT=1,NAME='CHARGEN.DAT',TYPE='OLD',READONLY)

Replace 'CHARGEN.DAT' with  'DRA0:[SYSEXE]CHARGEN.DAT'  (or

whatever directory and device names are appropriate).

VII.    Printing the output file:

On the Millikin system, the command is:

PRINT/NOFEED/DEV=LPB0: FORnnn.DAT

where nnn is the number of the output file as  given  to  PLOTS.

An  ASSIGN  statement may be used to associate some such FORTRAN

data file with the PRINTRONIX (which, as you can tell  from  the

above,  is LPB0:  on Millikin's VAX);  in this way, no permanent

data file is created and the information is sent directly to the

printer.  It's all a matter of taste and convenience.

Printing VPLOT output files on  the  screen  using  a  TYPE

command  allows one to see the approximate length and width of a

plot.  These files  contain  some  of  the  most  moronic  prose

imaginable:

@@@@@@@@@@@@@@@@@@@@@qiOc@jxfF@shI@aAy@@qMpGg@a@O@p

@@@@@@@@@@@@@@@@@@@f@a@AuP@bjIe@sqFFf@mN@@@Ggiarx@zl@k

@@@@@@@@@@@@@@@@gT@@@@@@fgPz@@@@@@GiMq@@@@eCc@@@@@@@@@h@@fgT

@@@@@@@@@@@@@@@yi@@@@whUSSjkOap@@F@x

@

@

@

Those single "@" characters usually represent blank  lines;

if  these  occur  at the beginning or end of a plot, they may be

edited out using EDT or SOS to make the plot shorter.  Text  may

be  edited  in with plot files, but may not occupy the same line

as plot data.  Be certain to  leave  at  least  one  blank  line

between  data  and  text, or the adjacent lines of text may have

broken letters.

VPLOT Guide                                                      Page 18

VIII.  Some final words:

O, tish ill done, tish ill done!  By my hand, tish ill done!

--(Henry the Fifth, Act III, Sc. II, line 91)

The VPLOT package has become an albatross around  my  neck.

I  have  been  working  on  it sporadically for something over a

year, with most programming being done in early August 1980  and

January  1981.   At  other  times  I  have  tried to improve the

plotter's performance, test for bugs (some were turning up  only

days  ago  and I'm certain that there are a few left), and write

documentation.   This  manual  is  based  on  the  same  CalComp

literature that I used in designing my routines;  in places this

Guide  reads  depressingly  (and  plagiaristically!)  like   the

CalComp material.  Peccavi.

Although I thought up the matrix-storage method  for  VPLOT

independently,  it seems that I was anticipated;  Hoare mentions

it in Structured Programming, and gives credit for the invention              __________ ___________

to  Dijkstra.  While I'm on the subject of acknowledgements, I'd

like to thank the Mathematics faculty at Millikin for much  help

and  encouragement,  especially  Dr.   Thomas Wallgren, who gave

valuable suggestions for CIRCL, ELIPS, and POLY;  the  staff  of

the  Computer  Center,  especially  center  director  Dr.   John

Arrison and programmer/analyst Mr.   Alan  Smith;   the  student

operators,  notably  Mr.   Jon  Westcot, who found a way to call

VPLOT from BASIC and who has contributed a short manual and some

BASIC programs to this package;  and of course my parents.

If (when?) you  find  errors  in  VPLOT,  need  help,  have

suggestions  or  needs  for  improvements, modified versions for

other printers or the PDP-11, etc., write:

M. Gooley

c/o Millikin University Computer Center

1184 West Main Street

Decatur, Illinois 62522

The Center's 'phone numbers, at present, are (217)-424-6333  and

-6334, but I can't conceive an emergency grave enough to warrant

a call;  besides, I probably would be in  class  when  the  call

arrived.