[VAXF81.ROSS.REMOTE]AAAREADME.DOC

[remote]areadme.txt

utility:	remote

author:		Jim Sturtevant

Ross Systems, inc.

Palo Alto, California

(415) 856-1100 x310

edit date:	December, 1981

abstract:	utility provides for terminal emulation through

VAX port. In conjunction with a VENTEL 212-plus

auto-dial modem this utility will allow interactive

users to dial out.

setup:		1. link remote with remotel.com

2. set line characteristics for remote ports to be

1200 baud, passall, fullduplex

3. add logical remote_log to point to a sequential

file with world:RW protection

4. add logical REMOTE0 thru REMOTEn to be equated with

the ports which your modem(s) are connected.

5. create logfile with remote.com

more info:	The remote program will begin by attempting to assign

the first available port begining with REMOTE0 till it

finds no more logicals. As it progresses it reports

who's using the busy ports and continues to rotor to

the next avaialable port. Once connected it outputs 2

carriage returns to get the attention of the modem.

At this point REMOTE is connected to the modem.

From this connected point till the end of the session

the user may type a control-F to request a remote

function.

Once connected to a Ventel modem the user can type the

desired phone number and operate as a remote terminal.

features:	.ability to log terminal session to an ondisk log file

.request input from a disk file on the local cpu

.drops DTR (to hangup the modem) due to an exit from

program.

.more efficient use of dial out modems...