;+++

;

; FACILITY

;	OAK RIDGE NATIONAL LABORATORY

;	FUSION ENERGY DIVISION

;	TOKAMAK SECTION

;

; ABSTRACT

;

;	This library contains a collection of procedures

;	designed to support access to the VAX CAMAC system

;	developed for the ORNL Fusion Energy Division VAX-11/780

;	computer system(s). These procedures are designed to

;	conform to VAX linkage conventions and as such should

;	be callable from any VAX supported high-level language.

;	Specifically, this library contains the following procedures:

;

;	CAM$CSTATE

;	-Alter default CAMAC system states for the

;	 on an image by image basis,

;	CAM$ASSIGN

;	-Assign a key value to a specific CAMAC module,

;	CAM$DASSGN

;	-Release a CAMAC module key assignment,

;	CAM$PIO(W)

;	-Perform a single-action CAMAC request(programmed I/O),

;	CAM$QSCAN(W)

;	-Perform a CAMAC Q-Scan mode transaction,

;	CAM$QSTOP(W)

;	-Perform a CAMAC Q-Stop mode transaction,

;	CAM$STOP(W)

;	-Perform a CAMAC Stop mode transaction.

;

;

;	A brief synopsis of each access procedure is included

;	in the preamble for that procedure.

;

; AUTHOR

;	E. T. Blair

;

; REVISION HISTORY

;	V02-000, E.T. Blair,12-AUG-1982

;	Modified CAMAC access procedures performing QIO(W) system services

;	to provide analogous entry points,i.e. CAM$XXX and CAM$XXXW calls

;	with consistent argument lists for both calls. In V01 the determination

;	as to whether a call was wait/proceed mode was based on the argument

;	list parameters specified, however, this approach posed some problems

;	when attempting to perform CAMAC calls from PASCAL.

;

;	V02-001,E.T. Blair,12-AUG-1982

;	Made changes to documentation in several procedure preambles

;	to correct errors and clarify intent. Also, changed argument

;	format for CAM$CSTATE(arguments XSTATE and QSTATE) from

;	bytes to words to accomodate use of existing mask definitions

;	in the CAMAC macro library.

;

;

;	V02-002,E.T. Blair,25-AUG-1982

;	Made change to CAM$CSTATE to force update of the process information

;	maintained in the CIB for each call to CAM$CSTATE. This provides a

;	simple means of updating this information if the process happens to

;	change some privilege, etc. without creating a separate access call

;	to perform the update.

;

;	V02-003, E.T. Blair,8-SEP-1982

;	Incorporated changes to optimize the access procedures. An internal

;	routine was added to generate the CAMAC RTB. Also, modifications to

;	support the parallel highway were added.

;---

;+++

;

;	Procedure:

;	CAM$CSTATE(EFN[,XSTATE][,QSTATE][,MEM])

;

;	Description:

;	This procedure provides a means to define the default

;	values for the parameters specified in the procedure

;	argument list. A description of each parameter, it's

;	function and permissible values is given below:

;

;	Parameter: EFN

;

;	Function:

;	Allows the user to define the default process local event flag to be

;	utilized by the CAMAC access procedures for synchronizing I/O

;	completion on all non-proceed I/O calls(i.e. any call to a CAMAC access

;	procedure that does not include the optional 'proceed I/O' parameters).

;	This value initially defaults to local event flag 0.

;

;	Value:

;	Any legal process local event flag number.

;

;	Parameter: XSTATE

;

;	Function:

;	Allows the user to influence the logic equation used by the

;	CAMAC branch driver in the determination of 'ERROR'/'NORMAL'

;	termination of a programmed I/O(PIO) or a list PIO operation.

;	This value initially defaults to 'CAM$_XI'.

;

;	Values:

;	CAM$XI-Ignores CAMAC X response in the determination of

;		ERROR status from the CAMAC branch driver.

;	CAM$_XE-Opposite of CAM$_XI.

;

;	Parameter: QSTATE

;

;	Function:

;	Allows the user to influence the logic equation used by the

;	CAMAC branch driver in the determination of 'ERROR'/

;	'NORMAL' termination of a programmed I/O(PIO) or a list

;	PIO operation. This value initially defaults to 'CAM$_QDC'.

;

;	Values:

;	CAM$_QDC-Ignores CAMAC Q response in the determination of

;		ERROR status from the CAMAC interface.

;	CAM$_QNE-CAMAC Q response not expected. If received, the CAMAC

;		interface terminates with ERROR status on PIO and list PIO

;		operations.

;	CAM$_QE-CAMAC Q response expected. If not received, the CAMAC

;		interface terminates with ERROR status on PIO and list PIO

;		operations.

;

;	Parameter: MEM

;

;	Function:

;	Allows the user to change the default memory format state for

;	a given image. The memory format used determines whether 16 or

;	24 bit CAMAC data transfer operations are performed. This value

;	initially defaults to '16' bit mode.

;

;	Values:

;	16-Sets default memory format to '16' bit format(1 CAMAC transfer =

;		1 word).

;	24-Sets default memory format to '24' bit format(1 CAMAC transfer =

;		2 words).

;

;	Parameters:

;	EFN- value of the process local event flag to be used by CAMAC

;		access procedures as default,

;	XSTATE- address of a word containing one of the symbolic values

;		CAM$_XI,CAM$_XE,

;	QSTATE- address of a word containing one of the symbolic values

;		CAM$_QDC, CAM$_QE, CAM$_QNE,

;	MEM- address of a byte containing one of the values 16 or 24.

;

;	Return Status:

;	SS$_NORMAL

;		Successful completion of item request.

;	SS$_INSFARG

;		Not enough arguments supplied.

;	SS$_BADPARAM

;		Invalid parameter specified.

;

;---

;+++

;

;	Procedure:

;	CAM$ASSIGN(%DESCR(MODULE_NAME),KEY)

;

;	Description:

;	This procedure performs a CAMAC key(I/O channel)

;	assignment to the specified CAMAC logical module.

;	The logical module name is translated recursively

;	and must eventually translate to a physical module

;	name from the SYSTEM logical name table unless the

;	calling process has SYSNAM privilege. Upon verification

;	of physical module name syntax and UIC access to the

;	module, a VAX I/O channel is assigned to the CAMAC

;	unit specified in the physical module name and the

;	key is encoded for return to the caller.

;

;	Parameters:

;	MODULE_NAME- address of a CHARACTER variable containing an ASCII

;		character string identifying the CAMAC module to be referenced,

;	KEY- address of a longword to receive the encoded key value

;		to be used for subsequent CAMAC I/O calls,

;

;	Return Status:

;	SS$_NORMAL

;		Service normally completed. Key value is valid.

;	SS$_BADPARAM

;		Invalid/illegal parameter was passed on the call.

;	(See SYS$ASSIGN system service for additional status return).

;

;---

;+++

;

;	Procedure:

;	CAM$DASSGN(KEY)

;

;	Description:

;	This procedure releases a CAMAC key assignment previously

;	assigned via the CAM$ASSIGN function call. The VAX I/O

;	channel associated with the specified key value is

;	deassigned when the reference count associated with the

;	CAMAC Support Unit becomes 0.

;

;	Parameters:

;	KEY- address of a longword integer containing the encoded

;		key value.

;

;	Return Status:

;	SS$_NORMAL

;		Request successfully completed.

;	SS$_BADPARAM

;		Illegal/invalid parameter passed on call.

;	See SYS$DEASSGN system service for additional status).

;

;---

;+++

;

;	Procedure:

;

;	CAM$PIO(W)(%DESCR(MODULE_NAME),A,F[,DATA][,MEM]][,IOSB]

;			[,EFN][,AST][,ASTPRM])

;		Or

;

;	CAM$PIO(W)(%DESCR(KEY),A,F,[,DATA][,MEM][,IOSB]

;			[,EFN][,AST][,ASTPRM])

;

;	Description:

;	This procedure performs a single-action CAMAC I/O request.

;	The specified CAMAC module name is translated into the

;	appropriate controller, crate and station addresses.

;	If the keyed access format is used, the key must correspond

;	to a CAMAC module previously connected via the CAM$ASSIGN

;	function call. The CAMAC data buffer is not required for

;	calls performing CAMAC control functions(8 <= F <= 15 or

;	24 <= F <= 31. If the I/O status block parameter is specified,

;	it is used to receive the status from the CAMAC I/O request.

;	If the memory format control parameter is specified,

;	the value determines whether 16 bits of data is transferred

;	(MEM=16) or 24 bits of data is transferred(MEM=24). If

;	not specified, the CIB memory format specifier is used.

;

;	If the proceed mode call(CAM$PIO) is used, control returns

;	 to the caller as soon as the CAMAC request is queued to

;	the system. If the wait mode call(CAM$PIOW) is used, control

;	is returned to the calling program only when the CAMAC I/O

;	is complete.

;

;	The I/O synchronization parameters(EFN, AST and ASTP) are used

;	in the same manner as their counterparts in the QIO and QIOW

;	system services. See the VAX System Services Reference Manual

;	for further discussion if needed.

;

;

;	Parameters:

;	MODULE_NAME- address of a CHARACTER variable containing an ASCII

;		character string identifying the CAMAC module to be referenced,

;	KEY- address of a longword descriptor for the key value

;		previously computed for the CAMAC module via

;		calling the procedure CAM$ASSIGN,

;	A- address of a byte containing CAMAC subaddress,

;	F- address of a byte containing CAMAC function code,

;	DATA- address of a long word containing CAMAC write

;		data or that will receive CAMAC read data. Not

;		required for CAMAC control functions,

;	MEM- address of a byte containing the memory format for

;		the CAMAC transaction(MEM=16; MEM=24). If not

;		specified, the current CIB memory format specifier

;		is used.

;	IOSB- address of a quadword buffer to receive I/O completion

;		status from the CAMAC request.

;

;	Optional I/O Synchronization Parameters:

;	EFN- value specifying the number of a local event flag

;		to be set upon I/O completion. If not specified, the

;		default process event flag from the CAMAC image block

;		is used,

;	AST- address of an AST procedure to receive an I/O completion

;		AST,

;	ASTPRM- address of a longword containing a parameter to

;		be passed to the specified AST procedure.

;

;	Return Status:

;

;---

;+++

;

;	Procedure:

;	CAM$QSCAN(W)(%DESCR(MODULE_NAME),A,F,COUNT,DATA[,MEM][,IOSB]

;		[,EFN],[,AST][,ASTPRM])

;		Or

;	CAM$QSCAN(W)(%DESCR(KEY),A,F,COUNT,DATA[,MEM][,IOSB]

;		[,EFN],[,AST][,ASTPRM])

;

;	Description:

;	This procedure performs a CAMAC subaddress/address

;	scan utilizing the Q-scan technique to control the

;	incrementing of the CAMAC subaddress and address

;	utilized on successive repetitions of the specified

;	CAMAC function code. The scan starts initially at the

;	specified subaddress(A). The initial station number

;	is determined from the specified module name(for direct

;	module reference calls) or from the key value(for keyed

;	access calls). To utilize the keyed access mode, the

;	key value must first be generated by a call to the

;	CAM$ASSIGN function. In all cases, the transaction

;	terminates when the transfer count(COUNT) is exhausted

;	or when an anomolous condition is detected. If the I/O

;	status block parameter is specified, the status of the

;	CAMAC I/O request is returned in specified quadword buffer.

;	If not, a quadword buffer in the CAMAC image block is used.

;

;	If the proceed mode call(CAM$QSCAN) is used, control returns

;	 to the caller as soon as the CAMAC request is queued to

;	the system. If the wait mode call(CAM$QSCANW) is used, control

;	is returned to the calling program only when the CAMAC I/O

;	is complete.

;

;	The I/O synchronization parameters(EFN, AST and ASTP) are used

;	in the same manner as their counterparts in the QIO and QIOW

;	system services. See the VAX System Services Reference Manual

;	for further discussion if needed.

;

;	Parameters:

;	MODULE_NAME- address of a CHARACTER variable containing an ASCII

;		character string identifying the CAMAC module to be referenced,

;	KEY- address of a longword descriptor for the key value

;		previously computed for the CAMAC module via

;		calling the procedure CAM$ASSIGN,

;	A- address of a byte containing CAMAC subaddress,

;	F- address of a byte containing CAMAC function code,

;	DATA- address of data buffer that contains CAMAC write data

;		or that will receive CAMAC read data.

;	COUNT- address of a word containing the CAMAC transfer

;		count(in units of the memory format).

;	MEM- address of a byte containing memory format for the

;		CAMAC transaction(MEM=16; MEM=24). If not specified,

;		the current CIB memory format specifier is used,

;	IOSB- address of a quadword buffer to receive I/O completion

;		status from the CAMAC request.

;

;	Optional I/O Synchronization Parameters:

;	EFN- value specifying the number of a local event flag

;		to be set upon I/O completion. If not specified, the

;		default process event flag from the CAMAC image block

;		is used,

;	AST- address of an AST procedure to receive an I/O completion

;		AST,

;	ASTPRM- address of a longword containing a parameter to

;		be passed to the specified AST procedure.

;

;

;	Return Status:

;

;---

;+++

;

;	Procedure:

;	CAM$QSTOP(W)(%DESCR(MODULE_NAME),A,F,COUNT,DATA[,MEM][,IOSB]

;		[,EFN][,AST][,ASTPRM])

;		Or

;	CAM$QSTOP(W)(%DESCR(KEY),A,F,COUNT,DATA[,MEM][,IOSB]

;		[,EFN][,AST][,ASTPRM])

;

;	Description:

;	This procedure performs a CAMAC block transfer utilizing

;	the Q-stop technique for controlling successive references

;	to a given CAMAC module using a fixed subaddress and

;	function code. A Q-stop transaction terminates when

;	the referenced module returns a Q=0 response or the

;	specified transfer count(COUNT) is exhausted or an

;	anomolous condition is detected. If the I/O status block parameter

;	is specified, the status of the CAMAC I/O operation is

;	returned in that quadword buffer. If not, a quadword

;	in the CAMAC image block is used to receive the status.

;

;	If the proceed mode call(CAM$QSTOP) is used, control returns

;	 to the caller as soon as the CAMAC request is queued to

;	the system. If the wait mode call(CAM$QSTOPW) is used, control

;	is returned to the calling program only when the CAMAC I/O

;	is complete.

;

;	The I/O synchronization parameters(EFN, AST and ASTP) are used

;	in the same manner as their counterparts in the QIO and QIOW

;	system services. See the VAX System Services Reference Manual

;	for further discussion if needed.

;

;	Parameters:

;	MODULE_NAME- address of a CHARACTER variable containing an ASCII

;	character string identifying the CAMAC module to be referenced,

;	KEY- address of a longword descriptor for the key value

;		previously computed for the CAMAC module via

;		calling the procedure CAM$ASSIGN,

;	A- address of a byte containing CAMAC subaddress,

;	F- address of a byte containing CAMAC function code,

;	DATA- address of data buffer that contains CAMAC write data

;		or that will receive CAMAC read data.

;	COUNT- address of a word containing the CAMAC transfer

;		count(in units of the memory format).

;	MEM- address of a byte containing memory format for the

;		CAMAC transaction(MEM=16; MEM=24). If not specified,

;		the current CIB memory format specifier is used.

;	IOSB- address of a quadword buffer to receive I/O completion

;		status from the CAMAC request.

;

;	Optional I/O Synchronization Parameters:

;	EFN- value specifying the number of a local event flag

;		to be set upon I/O completion. If not specified, the

;		default process event flag from the CAMAC image block

;		is used,

;	AST- address of an AST procedure to receive an I/O completion

;		AST,

;	ASTPRM- address of a longword containing a parameter to

;		be passed to the specified AST procedure.

;

;

;	Return Status:

;

;---

;+++

;

;	Procedure:

;	CAM$STOP(W)(%DESCR(MODULE_NAME),A,F,COUNT,DATA[,MEM][,IOSB]

;		[,EFN][,AST][,ASTPRM])

;

;		Or

;	CAM$STOP(W)(%DESCR(KEY),A,F,COUNT,DATA,[,MEM][,IOSB]

;		[,EFN][,AST][,ASTPRM])

;

;	Description:

;	This procedure performs a CAMAC block transfer utilizing

;	only the specified transfer count(COUNT) to control the

;	number of times the specified CAMAC module is referenced

;	using a fixed subaddress(A) and function code(F). The

;	transaction terminates when either the transfer count is

;	exhausted or an anomolous condition is detected.

;	If the I/O status block parameter is specified, the

;	status of the CAMAC I/O operation is returned to that

;	quadword buffer. If not, the status is returned to a

;	quadword buffer in the CAMAC image block.

;

;	If the proceed mode call(CAM$STOP) is used, control returns

;	 to the caller as soon as the CAMAC request is queued to

;	the system. If the wait mode call(CAM$STOPW) is used, control

;	is returned to the calling program only when the CAMAC I/O

;	is complete.

;

;	The I/O synchronization parameters(EFN, AST and ASTP) are used

;	in the same manner as their counterparts in the QIO and QIOW

;	system services. See the VAX System Services Reference Manual

;	for further discussion if needed.

;

;

;	Parameters:

;	MODULE_NAME- address of a CHARACTER variable containing an ASCII

;		character string identifying the CAMAC module to be referenced,

;	KEY- address of a longword descriptor for the key value

;		previously computed for the CAMAC module via

;		calling the procedure CAM$ASSIGN,

;	A- address of a byte containing CAMAC subaddress,

;	F- address of a byte containing CAMAC function code,

;	DATA- address of data buffer that contains CAMAC write data

;		or that will receive CAMAC read data.

;	COUNT- address of a word containing the CAMAC transfer

;		count(in units of the memory format).

;	MEM- address of a byte containing memory format for the

;		CAMAC transaction(MEM=16; MEM=24). If not specified,

;		the current CIB memory format specifier is used,

;	IOSB- address of a quadword buffer to receive I/O completion

;		status from the CAMAC request.

;

;	Optional I/O Synchronization Parameters:

;	EFN- value specifying the number of a local event flag

;		to be set upon I/O completion. If not specified, the

;		default process event flag from the CAMAC image block

;		is used,

;	AST- address of an AST procedure to receive an I/O completion

;		AST,

;	ASTPRM- address of a longword containing a parameter to

;		be passed to the specified AST procedure.

;

;

;	Return Status:

;

;---

;---