;+++

;

; FACILITY

;	OAK RIDGE NATIONAL LABORATORY

;	FUSION ENERGY DIVISION

;	TOKAMAK SECTION

;

; ABSTRACT

;

;	This library contains a collection of procedures

;	designed to provide LAM management support for the

;	VAX CAMAC system developed for the ORNL Fusion Energy

;	Division VAX-11/780 computer system(s).These procedures

;	are designed to conform to VAX linkage conventions and

;	as such should be callable from any VAX supported

;	high-level language. Specifically, this library contains

;	the following procedures:

;

;	CAM$LAMAST(W)

;	-Requests a LAM AST upon recognition of a LAM from a specific

;	 CAMAC module,

;	CAM$CANLAM(W)

;	-Cancels a LAM AST previously requested for a specific CAMAC

;	 module,

;	CAM$LAMWAIT

;	-Requests a LAM AST from a specific CAMAC module and forces

;	 the process to hibernate until either the LAM is recognized or

;	 the request times out.

;

;

;	A brief synopsis of each access procedure is included

;	in the preamble for that procedure.

;

; AUTHOR

;	E. T. Blair

;

; REVISION HISTORY

;	V01-000, E.T. Blair, 20-JUN-1982

;	Initial version of the LAM support access procedures.

;

;	V02-000, E.T. Blair,12-AUG-1982

;	Modified CAMAC access procedures performing QIO(W) system services

;	to provide analogous entry points,i.e. CAM$XXX and CAM$XXXW calls

;	with consistent argument lists for both calls. In V01 the determination

;	as to whether a call was wait/proceed mode was based on the argument

;	list parameters specified, however, this approach posed some problems

;	when attempting to perform CAMAC calls from PASCAL.

;

;	V03-000, E.T. Blair,31-AUG-1982

;	Made necessary modifications to support V02 of the CAMAC ACP. These

;	mods effect the QIO parameter list format used by CAM$LAMAST and

;	CAM$CANLAM.

;

;	V03-001, E.T. Blair, 8-SEP-1982

;	Incorporated optimization changes to utilize the CAM_INITCRP routine

;	vs. CAM_RELCRP.

;

;---

;+++

;

;	Procedure:

;	CAM$LAMAST(W)(%DESCR(MODULE_NAME),LAMAST,LAMASTP[,IOSB]

;		[,EFN][,AST][,ASTPRM])

;		Or

;	CAM$LAMAST(W)(%DESCR(KEY),LAMAST,LAMASTP[,IOSB]

;		[,EFN][,AST][,ASTPRM])

;

;

;	Description:

;	This procedure sets a LAM Attention AST for the specified

;	CAMAC module and enables CAMAC demand generation for that

;	module. Control is then returned to the caller with an

;	appropriate indication of the success of the set AST

;	request. The specified AST parameter is passed to

;	the specified LAM AST service procedure upon the

;	receipt of a CAMAC demand for the specified module.

;	The caller is responsible for synchronization of

;	the LAM AST procedure with normal program execution.

;

;

;	If the proceed mode call(CAM$LAMAST) is used, control returns

;	 to the caller as soon as the CAMAC request is queued to

;	the system. If the wait mode call(CAM$LAMASTW) is used, control

;	is returned to the calling program only when the CAMAC I/O

;	is complete.

;

;	The I/O synchronization parameters(EFN, AST and ASTP) are used

;	in the same manner as their counterparts in the QIO and QIOW

;	system services. See the VAX System Services Reference Manual

;	for further discussion if needed.

;

;

;	Parameters:

;	MODULE_NAME- address of an ASCII character string descriptor

;		identifying the CAMAC module to be referenced,

;	KEY- address of a longword descriptor for the key value

;		previously computed for the CAMAC module via

;		calling the procedure CAM$ASSIGN,

;	LAMAST- address of the entry mask for a VAX procedure

;		to be executed when the specified LAM occurrs,

;	LAMASTP- longword value to be passed to the LAM AST

;		service procedure when it is invoked.

;	IOSB- address of a quadword buffer to receive the I/O status

;		block contents resulting from the CAMAC I/O request,

;

;	Optional I/O Synchronization Parameters:

;	EFN- value specifying the number of a local event flag

;		to be set upon I/O completion. If not specified, the

;		default process event flag from the CAMAC image block

;		is used,

;	AST- address of an AST procedure to receive an I/O completion

;		AST,

;	ASTPRM- address of a longword containing a parameter to

;		be passed to the specified AST procedure.

;

;	Return Status:

;

;---

;+++

;

;	Procedure:

;	CAM$CANLAM(W)(%DESCR(MODULE_NAME)[,IOSB]

;		[,EFN][,AST][,ASTP])

;		Or

;	CAM$CANLAM(W)(%DESCR(KEY)[,IOSB]

;		[,EFN][,AST][,ASTP])

;

;

;	Description:

;	This procedure cancels any LAM attention AST's set for the

;	specified CAMAC module. Upon completion of this operation

;	control is returned to the caller with an appropriate

;	indication of the status of the operation. If the optional I/O

;	status block(IOSB) is specified, I/O completion status is returned

;	to that quadword buffer, otherwise, I/O completion status is

;	returned to the CAMAC image block IOSB.

;

;

;	If the proceed mode call(CAM$CANLAM) is used, control returns

;	 to the caller as soon as the CAMAC request is queued to

;	the system. If the wait mode call(CAM$CANLAMW) is used, control

;	is returned to the calling program only when the CAMAC I/O

;	is complete.

;

;	The I/O synchronization parameters(EFN, AST and ASTP) are used

;	in the same manner as their counterparts in the QIO and QIOW

;	system services. See the VAX System Services Reference Manual

;	for further discussion if needed.

;

;

;

;	Parameters:

;	MODULE_NAME- address of an ASCII character string descriptor

;		identifying the CAMAC module to be referenced,

;	KEY- address of a longword descriptor for the key value

;		previously computed for the CAMAC module via

;		calling the procedure CAM$ASSIGN,

;	IOSB- address of a quadword buffer to receive I/O completion status.

;		If not specified, the IOSB in the CAMAC image block is used.

;

;	Optional I/O Synchronization Parameters:

;	EFN- value specifying the number of a local event flag

;		to be set upon I/O completion. If not specified, the

;		default process event flag from the CAMAC image block

;		is used,

;	AST- address of an AST procedure to receive an I/O completion

;		AST,

;	ASTPRM- address of a longword containing a parameter to

;		be passed to the specified AST procedure.

;

;	Return Status:

;

;---

;+++

;

;	Procedure:

;	CAM$LAMWAIT(%DESCR(MODULE_NAME),TIME_OUT)

;		Or

;	CAM$LAMWAIT(%DESCR(KEY),TIME_OUT)

;

;

;	Description:

;	This procedure represents a high level LAM management service

;	call in the sense that it performs multiple functions on behalf

;	of the caller. Basically, the procedure enables a LAM AST from

;	the specified CAMAC module and then waits for the LAM AST to be

;	delivered. The time out value, specified in the call, determines how

;	long the process waits before cancelling the LAM AST request.

;	Specifically, the procedure performs the following actions:

;

;		Enables process AST delivery(via SYS$SETAST) and remembers

;			the previous state,

;		Cancels any pending wakeup requests for the process(via

;			SYS$CANWAK),

;		Enables a LAM AST from the specified CAMAC module(via

;			CAM$LAMAST),

;		Schedules a wakeup request(via SYS$SCHDWK) for the process

;			using the specified time out value(assumed to be a

;			delta time in units of seconds),

;		Hibernates(via SYS$HIBER) until awakened via either the

;			LAM AST being delivered or the wakeup request,

;		Determines the reason awakened and either cancels the

;			previous wakeup request or the LAM AST,

;		Restores the entry state of the process AST enable,

;		Returns SS$_WASSET status if the LAM AST is satisfied,

;			SS$_WASCLR status if the LAM is not received within

;			the specified time out. Other system codes may be

;			returned if an internal failure occurrs in one of the

;			procedures called in any of the above steps.

;

;	Notes:

;

;	1. The wake request queue for a VMS process is only ONE level

;	   deep. Hence, any wake requests pending upon entry into this procedure

;	   can cause unpredictable(and probably erroneous) results. In light of

;	   this, the procedure issues a cancel wakeup request for the process

;	   before setting the LAM AST and scheduling a wakeup request. Thus,

;	   any use of hibernate/wake system services in a process also using

;	   the CAM$LAMWAIT function must be carefully coordinated so as

;	   not to destroy a pending wakeup request.

;

;

;	Parameters:

;	MODULE_NAME- address of an ASCII character string descriptor

;		identifying the CAMAC module to be referenced,

;	KEY- address of a longword descriptor for the key value

;		previously computed for the CAMAC module via

;		calling the procedure CAM$ASSIGN,

;	TIME_OUT- address of a word specifying a time out value(in

;		seconds). This value defines the amount of time

;		to wait for a LAM from the specified module before

;		cancelling the LAM AST request. The maximum time out

;		value allowed is 32767 seconds.

;

;	Return Status:

;

;---

;---