;+++

;

; FACILITY

;	OAK RIDGE NATIONAL LABORATORY

;	FUSION ENERGY DIVISION

;	TOKAMAK SECTION

;

; ABSTRACT

;

;	This library contains a collection of procedures

;	designed to support list driven CAMAC operations on

;	the ORNL Fusion Energy Division VAX-11/780 computer

;	system(s). These procedures are designed to conform

;	to VAX linkage conventions and as such should

;	be callable from any VAX supported high-level language.

;	Specifically, this library contains the following

;	procedures:

;

;	CAM$DCLST

;	-Declare a CAMAC command list buffer and initialize it,

;	CAM$LSTATE

;	-Allows user to set/modify default CAMAC states for a given

;	 command list(defaults taken from current CIB values),

;	CAM$LPIO

;	-Generates a command list frame to perform a single-action

;	 CAMAC operation(equivalent of CAM$PIO),

;	CAM$LQSCAN

;	-Generates a command list frame to perform a CAMAC Q-Scan

;	 operation(equivalent of CAM$QSCAN),

;	CAM$LSTOP

;	-Generates a command list frame to perform a CAMAC Stop mode

;	 operation(equivalent of CAM$STOP),

;	CAM$EXECUTE(W)

;	-Executes a previously generated CAMAC command list,

;	CAM$RDATA

;	-Extracts CAMAC read data for a specific command frame from

;	 the reply list,

;	CAM$WDATA

;	-Updates CAMAC write data for a specific command frame in the

;	 CAMAC command list.

;

;

;	A brief synopsis of each access procedure is included

;	in the preamble for that procedure.

;

; AUTHOR

;	E. T. Blair

;

; REVISION HISTORY

;	V02-000, E.T. Blair,12-AUG-1982

;	Modified CAMAC access procedures performing QIO(W) system services

;	to provide analogous entry points,i.e. CAM$XXX and CAM$XXXW calls

;	with consistent argument lists for both calls. In V01 the determination

;	as to whether a call was wait/proceed mode was based on the argument

;	list parameters specified, however, this approach posed some problems

;	when attempting to perform CAMAC calls from PASCAL.

;

;	V02-001, E.T. Blair,13-AUG-1982

;	Modified CAM$LSTATE procedure call argument list to be consistent

;	with the CAM$CSTATE procedure call for setting default CIB

;	CAMAC state values. Also, made changes to documentation in several

;	procedure preambles to correct errors and clarify intent.

;

;	V02-002, E.T. Blair, 8-SEP-1982

;	Incorporated changes to optimize the generation of list command

;	frames via a single internal routine(CAM_LFRAME).

;

;---

;+++

;

;	Procedure:

;	CAM$DCLST(EXE_LIST,BLENGTH[,MEM][,EXE_FIBL])

;

;	Description:

;	The specified list buffer(EXE_LIST) is initialized as a CAMAC command

;	list. The first eight(8) words of the execute list buffer

;	are used by the list access procedures as a header block.

;	The format of this header follows:

;	FIELD		MEANING

;	WORD 0		Points to next available frame in execute

;			list,

;	WORD 1		Points to next available frame in the reply

;			list,

;	WORD 2		Execute list buffer length(in units of words),

;	WORD 3		Execute list CAMAC status/state word(fields defined

;				below),

;	WORDS 4-5	Address of the execute list Frame Information

;				Block List(FIBL),

;	WORD 6		Current execute list frame number,

;	WORD 7		VMS I/O channel used to submit list for execution,

;	WORD 8		Execute list internal status word,

;	WORD 9		CAMAC controller ID<00:07>; CAMAC crate<08:15>.

;

;	The Frame Information Block List(FIBL) is a buffer intended

;	to receive Frame Information Block(FIB) entries(1 per

;	execute list command frame) that may be used to perform

;	subsequent random access to the list by command frame

;	number. The format of a FIB is as follows:

;	FIELD		MEANING

;	WORD 0		Execute list frame pointer,

;	WORD 1		CAMAC function code<00:07>; command size<08:15>(words),

;	WORD 2		Data frame pointer(0-relative) defining index of first

;				word of data(index into execute list for CAMAC

;				writes;index into reply list for CAMAC reads),

;	WORD 2		Execute list frame length(in words),

;	WORD 3		Data frame length(in words).

;

;	The list status/state word contains the following fields

;	FIELD		MEANING

;	LIP		List execution is in progress(modify interlock bit),

;	QDC		Q-Don't Care state for determination of

;				NORMAL/ERROR status,

;	QNE		Q Not Expected state for determination of

;				NORMAL/ERROR status,

;	XI		X-Ignore state for determination of

;				NORMAL/ERROR status,

;	M16		Memory format control(M16=1, MEM=16; M16=0,

;				MEM=24),

;

;	The execute list header is initialized with all pointers

;	and counts reset. If the FIBL is specified, it's address

;	is saved in the execute list header. If not, the FIBL address

;	pointer is reset. In either case, the current frame count

;	is reset.

;

;	The list status/state word may be subsequently modified

;	by calling the CAM$LSTATE procedure.

;

;	Parameters:

;	EXE_LIST- address of buffer to be used as an execute

;		(command) list,

;	BLENGTH- address of a word containing the length of the

;		buffer in units of words,

;	MEM- address of a byte containing memory format for the

;		list being declared(MEM=16; MEM=24). If not specified,

;		the current CIB memory format state is used. Note

;		that 16 and 24 bit memory transfers may not be mixed

;		in a single execute list, i.e. ALL data transfers

;		must be 16 bit transfers or ALL data transfers must

;		be 24 bit transfers !!!!

;	EXE_FIBL- address of the Frame Information Block List

;		to receive command frame data that may be used

;		for subsequent random access/updates to the

;		execute list.

;

;	Return Status:

;

;---

;+++

;

;	Procedure:

;	CAM$LSTATE(EXE_LIST[,XSTATE][,QSTATE][,MEM])

;

;	Description:

;	This procedure allows the state/status word of the

;	specified CAMAC execute list buffer to be updated.

;	The call parameters correspond to list state/status

;	and are briefly described below.

;	Parameter: XSTATE

;

;	Function:

;	Allows the user to influence the logic equation used by the

;	CAMAC branch driver in the determination of 'ERROR'/'NORMAL'

;	termination of a list PIO operation. This value initially

;	defaults to CAMAC image block value.

;

;	Values:

;	CAM$XI-Ignores CAMAC X response in the determination of

;		ERROR status from the CAMAC branch driver.

;	CAM$_XE-Opposite of CAM$_XI.

;

;	Parameter: QSTATE

;

;	Function:

;	Allows the user to influence the logic equation used by the

;	CAMAC branch driver in the determination of 'ERROR'/

;	'NORMAL' termination of a list PIO operation. This value

;	initially defaults to CAMAC image block value.

;

;	Values:

;	CAM$_QDC-Ignores CAMAC Q response in the determination of

;		ERROR status from the CAMAC interface.

;	CAM$_QNE-CAMAC Q response not expected. If received, the CAMAC

;		interface terminates with ERROR status on list PIO

;		operations.

;	CAM$_QE-CAMAC Q response expected. If not received, the CAMAC

;		interface terminates with ERROR status on list PIO

;		operations.

;

;	Parameter: MEM

;

;	Function:

;	Allows the user to change the default memory format state for

;	a given image. The memory format used determines whether 16 or

;	24 bit CAMAC data transfer operations are performed. This value

;	initially defaults to the CAMAC image block value.

;

;	Values:

;	16-Sets default memory format to '16' bit format(1 CAMAC transfer =

;		1 word).

;	24-Sets default memory format to '24' bit format(1 CAMAC transfer =

;		2 words).

;

;	Parameters:

;	EXE_LIST- address of a CAMAC execute list buffer,

;	XSTATE- address of a word containing one of the symbolic values

;		CAM$_XI,CAM$_XE,

;	QSTATE- address of a word containing one of the symbolic values

;		CAM$_QDC, CAM$_QE, CAM$_QNE,

;	MEM- address of a byte containing one of the values 16 or 24.

;

;	Return Status:

;

;---

;+++

;

;	Procedure:

;	CAM$LPIO(EXE_LIST,%DESCR(MODULE_NAME),A,F[,DATA][,EXE_FRAME])

;		Or

;	CAM$LPIO(EXE_LIST,%DESCR(KEY),A,F[,DATA][,EXE_FRAME])

;

;	Description:

;	This procedure inserts a single action command frame into

;	the specified execute list. If the optional frame number

;	(EXE_FRAME) is specified, that frame in the execute list

;	is updated with the specified parameters provided the

;	command frame and data frame lengths in the FIB for

;	the specified frame are consistent with the LPIO command

;	and data frame lengths. If the frame number is not specified,

;	the next available list frame(as determined from the list header).

;	is used to receive the LPIO command frame. If the

;	specified execute list has a FIBL attached, a FIB is

;	generated for this frame.

;

;	If the action performs a CAMAC write operation, the

;	write data is extracted from the specified data buffer

;	(DATA). If the action performs a CAMAC read operation, the

;	data will be inserted at the next available location in the

;	reply list. The transfer count is determined in both

;	cases by the memory format associated with the specified

;	list buffer(i.e. '16' bit=1 word transferred; '24' bit=

;	2 words transferred).

;

;	Parameters:

;	EXE_LIST - address of an execute list command buffer previously

;		declared via a CAM$DCLST procedure call,

;	MODULE_NAME- address of a CHARACTER variable containing an ASCII

;		character string identifying the CAMAC module to be referenced,

;	KEY- address of a longword descriptor for the key value

;		previously computed for the CAMAC module via

;		calling the procedure CAM$ASSIGN,

;	A- address of a byte containing CAMAC subaddress,

;	F- address of a byte containing CAMAC function code,

;	DATA- address of CAMAC write data buffer. If omitted,

;		space is allocated in the execute list for

;		write data for CAMAC write function codes. CAMAC

;		read data is returned to the reply list,

;	EXE_FRAME- address of a word specifying the commmand frame

;		number to be updated by this LPIO request. If not

;		specified, the next available location in the execute

;		list will be used,

;

;	Return Status:

;

;---

;+++

;

;	Procedure:

;	CAM$LQSCAN(EXE_LIST,%DESCR(MODULE_NAME),A,F,COUNT[,DATA][,EXE_FRAME])

;		Or

;	CAM$LQSCAN(EXE_LIST,%DESCR(KEY),A,F,COUNT[,DATA][,EXE_FRAME])

;

;	Description:

;	This procedure builds a Q-scan command frame into the

;	specified execute list. If the optional frame number

;	(EXE_FRAME) is specified, that frame in the execute

;	list is updated with the specified parameters provided

;	the command and data frame lengths in the FIB for the

;	specified frame are consistent with the LQSCAN command

;	and data frame lengths. If the frame number is not specified,

;	the next available list frame(as determined from the list

;	header) is used to receive the LQSCAN command frame. If the

;	specified execute list has a FIBL attached, a FIB is generated

;	for this frame.

;

;	If the specified CAMAC function code indicates a

;	 write operation, data is cross-loaded from the

;	specified data buffer(DATA) into the execute list

;	as part of the command frame. For CAMAC read operations,

;	the data buffer need not be specified. Subsequent execution

;	of the specified list will result in a Q-scan block transfer

;	functionally equivalent to CAM$QSCAN. The memory format

;	associated with the specified execute list(MEM=16 or 24)

;	controls the interpretation of the transfer count(COUNT)

;	value(i.e. for MEM=16, 1 transfer = 1 word; for

;	MEM=24, 1 transfer = 2 words).

;

;	Parameters:

;	EXE_LIST- address of an execute list buffer previously

;		declared via a CAM$DCLST procedure call,

;	MODULE_NAME- address of a CHARACTER variable containing an ASCII

;		character string identifying the CAMAC module to be referenced,

;	KEY- address of a longword descriptor for the key value

;		previously computed for the CAMAC module via

;		calling the procedure CAM$ASSIGN,

;	A- address of a byte containing CAMAC subaddress,

;	F- address of a byte containing CAMAC function code,

;	COUNT- address of a word containing the CAMAC transfer

;		count(in units of the memory format).

;	DATA- address of data buffer that contains CAMAC write data

;		or that will receive CAMAC read data,

;	EXE_FRAME- address of a word specifying the command frame

;		number to be updated by this LQSCAN request. If not

;		specified, the next available location in the execute

;		list will be used.

;

;	Return Status:

;

;---

;+++

;

;	Procedure:

;	CAM$LSTOP(EXE_LIST,(%DESCR(MODULE_NAME),A,F,COUNT[,DATA][,EXE_FRAME])

;		Or

;	CAM$LSTOP(EXE_LIST,(%DESCR(KEY),A,F,COUNT[,DATA][,EXE_FRAME])

;

;	Description:

;	This procedure builds a Stop On Transfer Count command frame

;	into the specified execute list. If the optional frame number

;	(EXE_FRAME) is specified, that frame in the execute list

;	is updated with the specified parameters provided the

;	command frame and data frame lengths are consistent with

;	the LSTOP command and data frame lengths. If the frame number

;	is not specified, the next available list frame(as determined

;	from the list header) is used to receive the LSTOP command frame.

;	If the specified execute list has a FIBL attached, a FIB

;	is generated for this frame.

;

;	If the function code indicates a CAMAC write operation, data

;	is cross-loaded from the data buffer(DATA) into the

;	command frame. If the function code performs a CAMAC

;	read, the data buffer is not required. Subsequent execution

;	of the specified list will result in a stop mode

;	block transfer transaction functionally equivalent

;	to the execution of a CAM$STOP call. Data from

;	a CAMAC read operation is returned to the reply list

;	specified when the execute list is initiated.

;	The memory format associated with the specified execute list

;	(MEM=16 or 24) controls the interpretation of the transfer

;	count(COUNT) value(i.e. for MEM=16, 1 transfer = 1 word;

;	for MEM=24, 1 transfer = 2 words).

;

;	Parameters:

;	EXE_LIST- address of an execute list buffer previously

;		declared via a CAM$DCLST procedure call,

;	MODULE_NAME- address of a CHARACTER variable containing an ASCII

;		character string identifying the CAMAC module to be referenced,

;	KEY- address of a longword descriptor for the key value

;		previously computed for the CAMAC module via

;		calling the procedure CAM$ASSIGN,

;	A- address of a byte containing CAMAC subaddress,

;	F- address of a byte containing CAMAC function code,

;	COUNT- address of a word containing the CAMAC transfer

;		count(in units of the memory format).

;	DATA- address of data buffer that contains CAMAC write data

;		or that will receive CAMAC read data.

;	EXE_FRAME- address of a wordspecifying the command frame

;		number to be updated by this LSTOP request. If not

;		specified, the next available location in the execute

;		list will be used.

;

;	Return Status:

;

;---

;+++

;

;	Procedure:

;	CAM$EXECUTE(W)(EXE_LIST,[REPLY_LIST,BLENGTH][,IOSB]

;		[,EFN][,AST][,ASTPRM])

;

;	Description:

;	This procedure executes the specified list of CAMAC command

;	frames built by calls to the list mode access procedures.

;	For this call to be successful, the specified execute list must

;	 have a nonzero execute list frame pointer value.

;	The execute list frame pointer is contained in the list

;	header(an eight word reserved area at the beginning of the

;	execute list buffer) and is maintained by the list mode

;	access procedures. This pointer determines the length of

;	the execute list buffer for the execute list call.

;	If the specified list contains any CAMAC read command frames,

;	the optional reply list buffer(REPLY_LIST) and length

;	(BLENGTH) are required. The execute list header also contains

;	a reply list pointer that specifies the next available

;	word on the reply list(hence the length of the reply list

;	required to receive CAMAC read data from the execution of

;	the specified command list). If the buffer length specified

;	for the given reply list buffer is not sufficient to

;	contain the read data resulting from the execution of the

;	specified command list, request is not allowed.

;

;	If the I/O status block parameter is specified, the status

;	of the CAMAC I/O operation is returned in that quadword

;	buffer. If not, a quadword in the CAMAC image block is

;	used to receive the status.

;

;	If the proceed mode call(CAM$EXECUTE) is used, control returns

;	 to the caller as soon as the CAMAC request is queued to

;	the system. If the wait mode call(CAM$EXECUTEW) is used, control

;	is returned to the calling program only when the CAMAC I/O

;	is complete.

;

;	The I/O synchronization parameters(EFN, AST and ASTP) are used

;	in the same manner as their counterparts in the QIO and QIOW

;	system services. See the VAX System Services Reference Manual

;	for further discussion if needed.

;

;

;	Parameters:

;	EXE_LIST- address of an execute list buffer previously

;		declared via a CAM$DCLST procedure call and

;		containing CAMAC command frames built by calls

;		to the list mode access procedures CAM$LPIO,

;		CAM$LQSCAN, CAM$LSTOP,

;	REPLY_LIST- address of the buffer used to receive

;		CAMAC read data from the execution of the current

;		list,

;	BLENGTH- address of a word containing the length of the

;		buffer in units of words.

;	IOSB- address of a quadword buffer to receive I/O completion

;		status from the CAMAC request.

;

;	Optional I/O Synchronization Parameters:

;	EFN- value specifying the number of a local event flag

;		to be set upon I/O completion. If not specified, the

;		default process event flag from the CAMAC image block

;		is used,

;	AST- address of an AST procedure to receive an I/O completion

;		AST,

;	ASTPRM- address of a longword containing a parameter to

;		be passed to the specified AST procedure.

;

;

;	Return Status:

;

;---

;+++

;

;	Procedure:

;	CAM$RDATA(EXELST,FRAME,REPLST,READ_DATA)

;

;	Description:

;	This procedure provides a means of accessing CAMAC read data

;	resluting from the execution of a CAMAC command list(via the

;	CAM$EXECUTE call). The key for referencing the CAMAC read data is

;	the execute list frame number(FRAME). Successful use of this procedure

;	requires that a Frame Information Block List(FIBL) be associated with

;	the specified execute list(EXELST). The procedure verifies that the

;	specified frame in the execute list did indeed perform a CAMAC read

;	operation and if so, transfers the data read by the execution of that

;	frame from the specified reply list(REPLST) into the specified read

;	data buffer(READ_DATA). The number of words transfered is determined

;	from the FIB for the specified frame.

;

;	Parameters:

;	EXELST- address of an execute list buffer(assumed to have been

;		executed prior to this call),

;	FRAME- address of a word specifying the desired frame to be returned,

;	REPLST- address of a reply list buffer(assumed to contain CAMAC read

;		data as the result of executing the specified execute list),

;	READ_DATA- address of a buffer to receive the data resulting from the

;		execution of the specified frame from the execute list.

;

;---

;+++

;

;	Procedure:

;	CAM$WDATA(EXELST,FRAME,WRITE_DATA)

;

;	Description:

;	This procedure provides a means of updating CAMAC write data

;	in a CAMAC command list. The key for referencing the CAMAC read data is

;	the execute list frame number(FRAME). Successful use of this procedure

;	requires that a Frame Information Block List(FIBL) be associated with

;	the specified execute list(EXELST). The procedure verifies that the

;	specified frame in the execute list intends to perform a CAMAC write

;	operation and if so, transfers the data from the specified write data

;	buffer(WRITE_DATA) into the execute list to update the write data for

;	the specified frame. The number of words transfered is determined

;	from the FIB for the specified frame.

;

;	Parameters:

;	EXELST- address of an execute list buffer(assumed to have been

;		executed prior to this call),

;	FRAME- address of a word specifying the desired frame to be returned,

;	WRITE_DATA- address of a buffer to containing the data to be written

;		into the specified frame of the execute list.

;

;---

;---