;+++

;

; FACILITY

;	OAK RIDGE NATIONAL LABORATORY

;	FUSION ENERGY DIVISION

;	TOKAMAK SECTION

;

; ABSTRACT

;

;	This library contains a collection of procedures

;	designed to provide status checking support for the

;	VAX CAMAC system developed for the ORNL Fusion Energy

;	Division VAX-11/780 computer system(s). These procedures

;	are designed to conform to VAX linkage conventions and

;	as such should be callable from any VAX supported high-level

;	language. Specifically, this library contains the following

;	procedures:

;

;	CAM$ERROR

;	-Analyzes the I/O status block from a CAMAC request to determine

;	 if any errors occurred on the transaction,

;	CAM$X

;	-Analyzes the I/O status block from a CAMAC request to determine

;	 if the X response was generated,

;	CAM$Q

;	-Analyzes the I/O status block from a CAMAC request to determine

;	 if the Q response was generated,

;	CAM$XANDQ

;	-Performs the equivalent of (CAM$X .AND. CAM$Q),

;	CAM$GET_STAT

;	-Returns raw I/O status block from the CAMAC image block to the

;	 user.

;	CAM$SHOW_STAT

;	-Performs analysis and formatted display of CAMAC I/O status block.

;

;

;	A brief synopsis of each access procedure is included

;	in the preamble for that procedure.

;

; AUTHOR

;	E. T. Blair

;

; REVISION HISTORY

;	V01-001, E.T. Blair, 8-SEP-1982

;	Made changes necessary to support the parallel highway. These changes

;	are primarily assembly time checks made via the ASSUME macro to

;	verify consistency of parallel control register and error/status

;	register fields in the IOSB with their counterparts in the serial

;	control register and error/status register.

;

;	V01-002, E.T. Blair, 9-SEP-1982

;	Added CAM$SHOW_STAT procedure to the CAMAC status library. This

;	procedure analyzes the CAMAC I/O status block and produces a

;	formatted display of it's contents.

;

;---

;+++

;

;	Procedure:

;	CAM$ERROR(X,Q[,IOSB])

;

;	Description:

;	This procedure determines if a CAMAC transaction error

;	occurred and returns a logical value as the function output

;	(ERROR=.TRUE.; NOERROR=.FALSE.). Specifically, a .TRUE. return

;	value indicates one of the following conditions exists:

;		A system detected I/O error occurred(in which case

;			the first longword of the IOSB contains a

;			system status value),

;		A CAMAC transmission error occurred causing the CAMAC

;			transaction to terminate with 'ERROR' status,

;		The X/Q response from the CAMAC transaction is

;			inconsistent with the expected X/Q response.

;

;	If further clarification of the nature of the error is required

;	the CAM$X, CAM$Q, CAM$XANDQ or CAM$SHOW_STAT functions may be

;	used.

;

;	If no I/O status block is specified for the call,

;	the procedure uses the I/O status block from the CAMAC

;	image block. If the I/O status block is specified, it is

;	assumed that valid CAMAC I/O status is contained in the

;	block. This capability enables the CAM$ERROR function to

;	be used to interrogate I/O status from proceed mode CAMAC requests.

;

;	Parameters:

;	X- address of a LOGICAL variable(any length) specifying

;		the expected state of the CAMAC X response(may be

;		specified as null for DON'T CARE),

;	Q- address of a LOGICAL variable(any length) specifying

;		the expected state of the CAMAC Q response(may be

;		specified as null for DON'T CARE),

;	IOSB- address of a quadword CAMAC I/O status block. If

;		not specified, the IOSB from the last CAMAC request

;		is utilized.

;

;	Return Status:

;	.TRUE. if a CAMAC transmission error occurred or if the

;	resultant X and Q states are inconsistent with the expected

;	values of those states, otherwise, the value .FALSE. is

;	returned.

;

;---

;+++

;

;	Procedure:

;	CAM$X([IOSB])

;

;	Description:

;	This procedure determines the X response from a CAMAC I/O

;	and returns a logical value as the function output

;	(X=.TRUE.; NO X=.FALSE.). If no I/O status block

;	is specified for the call, the procedure uses the I/O

;	status block from the last CAMAC request. If the I/O

;	status block is specified, it is assumed that valid

;	CAMAC I/O status is contained in the block. This capability

;	enables the CAM$X function to be used to interrogate

;	I/O status from proceed mode CAMAC requests.

;

;	Parameters:

;	IOSB- address of a quadword CAMAC I/O status block. If

;		not specified, the IOSB from the last CAMAC request

;		is utilized.

;

;	Return Status:

;	.TRUE. if X=1 response received,

;		.FALSE. if X=0 response received.

;

;---

;+++

;

;	Procedure:

;	CAM$Q([IOSB])

;

;	Description:

;	This procedure determines the Q response from a CAMAC I/O

;	and returns a logical value as the function output

;	(Q=.TRUE.; NO Q=.FALSE.). If no I/O status block

;	is specified for the call, the procedure uses the I/O

;	status block from the last CAMAC request. If the I/O

;	status block is specified, it is assumed that valid

;	CAMAC I/O status is contained in the block. This capability

;	enables the CAM$Q function to be used to interrogate

;	I/O status from proceed mode CAMAC requests.

;

;	Parameters:

;	IOSB- address of a quadword CAMAC I/O status block. If

;		not specified, the IOSB from the last CAMAC request

;		is utilized.

;

;	Return Status:

;	.TRUE. if Q=1 response received,

;		.FALSE. if Q=0 response received.

;

;---

;+++

;

;	Procedure:

;	CAM$XANDQ([IOSB])

;

;	Description:

;	This procedure determines the logical AND of the X and Q

;	responses from a CAMAC I/O and returns a logical value

;	as the function output( X=1 .AND. Q=1 returns .TRUE.;

;	X=0 .OR. Q=0 returns .FALSE.). If no I/O status block

;	is specified for the call, the procedure uses the I/O

;	status block from the last CAMAC request. If the I/O

;	status block is specified, it is assumed that valid

;	CAMAC I/O status is contained in the block. This capability

;	enables the CAM$Q function to be used to interrogate

;	I/O status from proceed mode CAMAC requests.

;

;	Parameters:

;	IOSB- address of a quadword CAMAC I/O status block. If

;		not specified, the IOSB from the last CAMAC request

;		is utilized.

;

;	Return Status:

;	.TRUE. if (X=1 .AND. Q=1),

;		.FALSE. if (X=0 .OR. Q=0).

;

;---

;+++

;

;	Procedure:

;	CAM$GET_STAT(IOSB)

;

;	Description:

;	This procedure provides a means for a CAMAC user to sample the

;	CAMAC image block I/O status block(which is used as the default

;	IOSB for all CAMAC I/O requests). The contents of the

;	CIB IOSB is returned in the quadword buffer specified by

;	the caller(IOSB). Function status is always returned as

;	SS$_NORMAL.

;

;	Parameters:

;	IOSB- address of a quadword buffer to receive the CAMAC image

;		block IOSB contents.

;

;---

;+++

;

;	Procedure:

;	CAM$SHOW_STAT([IOSB])

;

;	Description:

;	This procedure performs an analysis of the CAMAC I/O status

;	block resulting from a CAMAC I/O request. The quadword IOSB

;	is interpreted as follows:

;

;		If (.not.IOSB(1)) then

;			IOSB(1) contains a system status code(SS$_xx),

;			IOSB(2) contents are unpredictable.

;		Else

;			IOSB_W(1) contains final CTR state(SCT/PCT),

;			IOSB_W(2) contains net transfer count(bytes),

;			IOSB_W(3) contains final ESR state(SES/PES),

;			IOSB_W(4) contains net reply list count(bytes).

;		End If

;

;	The resulting display of the CAMAC IOSB consists of either an

;	appropriate system status message or a collection of CAMAC status

;	messages.

;

;	If the optional I/O status block parameter is specified, it is assumed

;	that it represents a valid CAMAC I/O status block. If not specified,

;	the call defaults to the I/O status block in the CAMAC image block.

;

;	Parameters:

;	IOSB- address of a quadword buffer containing valid CAMAC I/O status.

;		If not specified, defaults to CIB IOSB.

;

;---

;---