C+++

C

C FACILITY

C	OAK RIDGE NATIONAL LABORATORY

C	FUSION ENERGY DIVISION

C	TOKAMAK SECTION

C

C ABSTRACT

C	This FORTRAN procedure library provides several support procedures

C	for the FED VAX CAMAC system. The procedures included presently fall

C	into one of two categories, namely (1) access to privileged CAMAC

C	ACP functions and (2) CAMAC status display. Specifically, the library

C	contains the following procedures:

C

C	CAM$CONL

C	-Request ACP set CAMAC crate unit 'ON-LINE',

C	CAM$ENLAMS

C	-Request ACP to enable LAM support for a specified CAMAC

C	 crate unit,

C	CAM$COFL

C	-Request ACP to set a specified CAMAC crate unit 'OFF-LINE',

C	CAM$DABLAMS

C	-Request ACP to disable LAM support for a specified CAMAC

C	 crate unit,

C

C AUTHOR

C	E.T. Blair

C

C REVISION HISTORY

C	V01-001,E.T. Blair,25-AUG-1982

C	Added documentation for internal procedure CAM_SUPINIT.

C

C	Modified CAM$CONL, CAM$COFL, CAM$ENLAMS, CAM$DABLAMS procedures

C	to return the CAMAC IOSB resulting from QIO's to the CAMAC ACP

C	to the calling program. These procedures no longer try to interpret

C	the CAMAC IOSB for success/error indication. This now becomes the

C	responsibility of the caller, just as for the standard CAMAC access

C	calls.

C

C---

C+++

C

C	Procedure:

C	CAM$CONL(UNIT_NAME,IOSB)

C

C	Description:

C	This procedure makes a request to the CAMAC ACP to set the

C	specified CAMAC crate unit 'ON-LINE'. The ACP performs the

C	necessary CAMAC commands to bring the CAMAC crate controller

C	to an operational state and performs the necessary internal

C	data structure manipulation to set the crate UCB 'ON-LINE'.

C

C	Parameters:

C	UNIT_NAME- address of a CHARACTER string descriptor specifying

C		an ASCII string identifying the crate unit to be set

C		on-line.

C	IOSB- address of a quadword buffer to be used as the CAMAC I/O

C		status block for the I/O request.

C

C	Operational Notes:

C	1. Successful execution of this procedure requires that the

C	   process executing the image using CAM$CONL have PHYS_IO

C	   privilege.

C

C

C	Return Status:

C	SS$_NORMAL- CAMAC request successful,

C	Any other system status indicates faliure(see system

C		status message definitions for explicit error

C		message).

C

C---

C+++

C

C	Procedure:

C	CAM$ENLAMS(UNIT_NAME,LAMG_N,IOSB)

C

C	Description:

C	This procedure makes a request to the CAMAC ACP to enable LAM

C	support for the specified CAMAC crate unit. The ACP performs the

C	necessary CAMAC commands to bring the CAMAC crate controller

C	and LAM grader/encoder to a state capable of generating CAMAC

C	demands to the associated VAX highway driver. The ACP also allocates

C	and initializes the necessary internal data structures to allow

C	LAM AST requests to be queued to specific stations within the

C	crate. Each crate is considered to have a total of 24 LAM stations

C	numbered 0-23, where station 0 corresponds to the internal LAM

C	associated with serial CAMAC L2 crate controllers. Attempts to

C	perform LAM service requests to a crate without LAM support enabled

C	will result in failure status from the CAMAC system.

C

C	Parameters:

C	UNIT_NAME- address of a CHARACTER string descriptor specifying

C		an ASCII string identifying the crate unit to be set

C		on-line.

C	LAMG_N- value of the CAMAC station to be used to address the

C		LAM grader/encoder module.

C	IOSB- address of a quadword buffer to be used as the CAMAC I/O

C		status block for the I/O request.

C

C	Operational Notes:

C	1. Successful execution of this procedure requires that the

C	   process executing the image using CAM$ENLAMS have PHYS_IO

C	   privilege.

C	2. The VAX CAMAC LAM management philosophy assumes the use of

C	   LAM grader/encoder modules to provide application independent

C	   control over the generation of CAMAC demands. In this way, system

C	   integrity is not compromised by a failure to clear an existing LAM.

C

C

C	Return Status:

C	SS$_NORMAL- CAMAC request successful,

C	Any other system status indicates faliure(see system

C		status message definitions for explicit error

C		message).

C

C---

C+++

C

C	Procedure:

C	CAM$COFL(UNIT_NAME,IOSB)

C

C	Description:

C	This procedure makes a request to the CAMAC ACP to set the

C	specified CAMAC crate unit 'OFF-LINE'. The ACP performs the

C	necessary CAMAC commands to disable further CAMAC crate controller

C	activity and performs the necessary internal data structure

C	manipulation to set the crate UCB 'OFF-LINE'.

C

C	Parameters:

C	UNIT_NAME- address of a CHARACTER string descriptor specifying

C		an ASCII string identifying the crate unit to be set

C		on-line.

C	IOSB- address of a quadword buffer to be used as the CAMAC I/O

C		status block for the I/O request.

C

C	Operational Notes:

C	1. Successful execution of this procedure requires that the

C	   process executing the image using CAM$COFL have PHYS_IO

C	   privilege.

C

C

C	Return Status:

C	SS$_NORMAL- CAMAC request successful,

C	Any other system status indicates faliure(see system

C		status message definitions for explicit error

C		message).

C

C---

C+++

C

C	Procedure:

C	CAM$DABLAMS(UNIT_NAME,IOSB)

C

C	Description:

C	This procedure makes a request to the CAMAC ACP to disable LAM

C	support for the specified CAMAC crate unit. The ACP performs the

C	necessary CAMAC commands to take the CAMAC crate controller

C	and LAM grader/encoder to a state incapable of generating CAMAC

C	demands to the associated VAX highway driver. The ACP then flushes

C	all pending AST requests for the specified CAMAC crate, accounting

C	for the AST charged aganist the process in the process AST quota.

C	Then the associated internal data structure used by the ACP for

C	LAM management support is deallocated back to non-paged pool.

C

C	Parameters:

C	UNIT_NAME- address of a CHARACTER string descriptor specifying

C		an ASCII string identifying the crate unit to be set

C		on-line.

C	IOSB- address of a quadword buffer to be used as the CAMAC I/O

C		status block for the I/O request.

C

C	Operational Notes:

C	1. Successful execution of this procedure requires that the

C	   process executing the image using CAM$DABLAMS have PHYS_IO

C	   privilege.

C	2. The VAX CAMAC LAM management philosophy assumes the use of

C	   LAM grader/encoder modules to provide application independent

C	   control over the generation of CAMAC demands. In this way, system

C	   integrity is not compromised by a failure to clear an existing LAM.

C

C

C	Return Status:

C	SS$_NORMAL- CAMAC request successful,

C	Any other system status indicates faliure(see system

C		status message definitions for explicit error

C		message).

C

C---

C+++

C

C	Procedure:

C	CAM$CSU(UNIT_NAME,HSU_NAME,CRATE_NUMBER)

C

C	Description:

C	This procedure parses the CAMAC Crate Support Unit name input

C	in the CHARACTER variable UNIT_NAME, returning the name of the

C	CAMAC Highway Support Unit for the CSU and the crate number

C	from the CSU.

C

C	Parameters:

C	UNIT_NAME- address of a descriptor for a CHARACTER variable containing

C	     a valid CAMAC Crate Support Unit name,

C	HSU_NAME- address of a descriptor for a CHARACTER variable to receive

C	     the Highway Support Unit name for the input CSU,

C	CRATE_NUMBER- address of a longword to receive the CAMAC crate number

C	     taken from the CSU name.

C

C---

C---