;+++

;

; FACILITY

;	OAK RIDGE NATIONAL LABORATORY

;	FUSION ENERGY DIVISION

;	TOKAMAK SECTION

;

; ABSTRACT

;	This module contains procedures to support the CAMAC

;	Topology Supervisor. Specifically, the following

;	procedures are included:

;

;	CAM$PHYMOD

;	-Verifies CAMAC physical module name syntax.

;

; AUTHOR

;	E.T. Blair, 16-August-1982.

;

; REVISION HISTORY

;	V01-001,E.T. Blair,25-AUG-1982

;	Modify CAM$PHYMOD error status returned if physical name length

;	exceeds max string length allowed for module names from SS$_IVDEVNAM

;	to SS$_RESULTOVF. Add instruction to clear CRP status word when CRP

;	is allocated on stack.

;

;---

;+++

;

;	Procedure:

;	CAM$PHYMOD(PHY_MOD_NAME,CSU_NAME,MOD_DATA)

;

;	Description:

;	This procedure verifies the syntax of the specified CAMAC

;	physical module name and returns the fields of the CAMAC physical

;	module name. The CAMAC Crate Support Unit(CSU) is returned

;	as an ASCII string in the CHARACTER variable CSU_NAME. The

;	module UIC group and member, CAMAC crate and station numbers

;	are returned to the longword array MOD_DAT in exactly that

;	order. The call returns a success status code(SS$_NORMAL) if

;	the physical module name is completely parsed, otherwise, a

;	pertinent failure status code is returned.

;

;	Parameters:

;	PHY_MOD_NAME- address of a CHARACTER string descriptor pointing to

;		an ASCII string containing the physical CAMAC module name

;		string. The string must not contain leading blanks and the

;		descriptor passed must contain the exact length of the string,

;	CSU_NAME- address of a CHARACTER string descriptor pointing to

;		a buffer that will receive the CAMAC Crate Support Unit

;		string,

;	MOD_DATA- address of a longword array(4 elements in size) to receive

;		CAMAC module data as follows:

;		Longword	Value Received

;		0		Module UIC Group(MSB=1 => Wild Group),

;		1		Module UIC Member(MSB=1 => Wild Member),

;		2		Module Crate number,

;		3		Module Station number.

;

;	Return Status:

;	SS$_NORMAL- successful call,

;	SS$_INSFARG- insufficient call arguments(programming error),

;	SS$_RESULTOVF- Physical module name too large or CSU string

;		to small(programming error),

;	SS$_IVDEVNAM- invalid device name(parse failure).

;

;---

;---