CHAPTER 2

USING A HOST-BASED PAD

2.1  INTRODUCTION

As mentioned in Chapter 1, X.29 terminals communicate with remote DTEs

through  a  PAD  using  the  X.3, X.28, and X.29 protocols.  The PPSNs

provide PADs which you can access over  leased  lines  or  by  dial-up

lines.  If your terminal is connected to a VAX/VMS system running PSI,

you can use the host-based PAD facility to connect your terminal to  a

remote  DTE  (converting the local terminal to an X.29 terminal for as

long as you are using the PAD program).

The host-based PAD has a number of advantages over  a  PPSN  PAD.   It

costs  less;   you  do  not have to pay for a dial-up call and can use

existing leased lines that are connected to the VAX.  It  is  quicker;

the  link between the host-based PAD and your terminal usually runs at

9600 baud, a dial-up line would run at 300 or 1200 baud.  It  is  more

reliable;   dial-up  lines  in  particular  are prone to errors due to

interference.

This chapter explains how to log on to and exit from a host-based PAD.

If  you  are  using a PPSN PAD, refer to the documentation supplied by

your PPSN for accessing their PADs.  The chapter  also  describes  the

command  signals  that  control  the  host-based  PAD  and the service

signals that the PAD returns to the X.29 terminal.  Refer to  Appendix

B for the CCITT command and service signals.

2.2  LOGGING ON TO A HOST-BASED PAD

To invoke the PAD program use the following command:

$ RUN SYS$SYSTEM:PSIPAD

When the PAD program starts, it displays:

PAD>

To make the PAD program easier to access,  define  a  symbol  in  your

LOGIN.COM file, as follows:

PAD :=="$PSIPAD"

If you try to run the PAD program on a system which does not have  the

PSI software running, the PAD program displays:

%PAD-E-PSINOTLD, PSI not loaded

USING A HOST-BASED PAD                                        Page 2-2

2.3  PAD COMMAND SIGNALS

The following sections describe the commands that control the PSI-PAD.

Using the PAD commands, you can:

.  Establish a call to a remote DTE with the A command

.  Display and set the PAD parameters using the  PAR?   and  SET

commands

.  Display the PAD status with the STAT command

.  Send break, interrupt, and reset signals to  the  remote  DTE

with the BREAK, INTERRUPT, and RESET commands.

.  Include up to one line of data in  the  call  request  packet

with the U command

The  commands  are  described  in  alphabetical  order.   Appendix   A

describes the PAD parameters.

If you enter an incorrect command, the PAD program displays:

%PAD-E-ERR, ERR??, Error in command

and reprompts:

PAD>

Other PAD service signals are described in Section 2.4.

2.3.1  Call Request (A Dte-address)

Use the A command to connect your terminal with  a  remote  DTE.   The

format for the command is:

PAD>A dte-address

where dte-address is  the  12-character  DTE  address  (including  the

4-digit DNIC) for the remote DTE.

You need not reenter the DTE address if:

1.  You fail to establish the call at the first attempt.

2.  The link with the DTE is broken, returning  you  to  the  PAD

command mode.

When you enter A, the PAD tries to establish the call  using  the  DTE

address specified on the failed call.

When the circuit is established, the PAD program displays the  service

signal:

%PAD-I-COM, COM, Call connected

The start of the login procedure for the remote DTE is also displayed.

The  PAD is now in data transfer mode.  Return to the PAD command mode

by typing <CTRL/P>.  You can then enter one PAD command,  after  which

USING A HOST-BASED PAD                                        Page 2-3

(unless  you  have cleared the call using CLR or EXIT) the PAD returns

you to the remote DTE.

If PAD parameter 1 is set  to  0,  the  PAD  does  not  recognize  any

character  as  an  escape  to  command  mode character (<CTRL/P>).  To

return to  the  PAD  command  mode,  type  nine  consecutive  <CTRL/Y>

characters.  If you enter a tenth <CTRL/Y>, the PAD program clears the

call and exits.

PSI sets a default value of 1 for parameter 1 on the host-based PAD.

You can DEFINE logical names for DTE addresses  and  use  the  logical

name  in  place of the full DTE address when trying to connect to that

DTE.

2.3.2  Break (BREAK)

Use the BREAK command to send a break signal to  the  host-based  PAD.

The format for the command is:

PAD> BREAK

The following values  for  PAD  parameter  7  determine  how  the  PAD

responds when it receives the break signal:

0   No action

1   PAD sends an interrupt to the remote DTE

2   PAD sends a reset to the remote DTE

4   PAD sends an Indication Break PAD message to the remote DTE

8   PAD leaves the Data Transfer state and enters the waiting for

command state

16   PAD discards output to the terminal by setting parameter 8 to

1

21   Combination of 1+4+16

PSI sets a default value of 0 for parameter 7 on the host-based PAD.

The BREAK command is needed because the PAD program ignores any  break

characters  typed  at  the  terminal  (because the VMS terminal driver

discards break characters).

2.3.3  Call Status Request (STAT)

Use the STAT command to find out if you have established a  call  from

the terminal that you are using.  The format for the command is:

PAD> STAT

If there is a call in progress from that terminal, the PAD displays:

%PAD-I-ENGAGED, PAD engaged

USING A HOST-BASED PAD                                        Page 2-4

If there is no call in progress, the PAD displays:

%PAD-I-FREE, PAD free

2.3.4  Interrupt (INT)

Use the INT command to transmit an interrupt packet to the remote DTE.

The format for the command is:

PAD> INT

When the host-based PAD sends the interrupt, you may lose any data  in

transfer between the host-based PAD and the remote DTE,

The effect of the interrupt on the remote DTE depends on how  the  DTE

is  set  up.  For example, if the DTE was running VAX-11 PSI, it would

ignore an interrupt.

If you try to send a second interrupt before the remote  DTE  and  the

PAD  have  completed  handling the previous interrupt, the PAD program

displays:

%PAD-W-WARNINT, Previous interrupt incomplete

2.3.5  Read PAD Parameter Values (PAR?)

The PAR?  command lists the current values of all the PAD  parameters.

The format for the command is:

PAD> PAR?

The PAD program displays:

PAR 1:1,2:1,3:126,4:10,5:1,6:1,7:0,8:0,9:0,10:80,11:14,12:1,13:0,14:0,15:1,16:12

7,17:24,18:18

The values for parameters 16, 17, and 18 are displayed  only

if you set the WRAPAROUND terminal SET-UP feature to on (1).

You can display the values  for  a  specific  parameter  (or

parameters).  For example, if you enter:

PAR?  3,4,9

the  PAD  program  displays  the  current  values  for   PAD

parameters 3, 4, and 9:

PAR 3:126,4:10,9:0

See Appendix A for a description of the PAD  parameters  and

values.

USING A HOST-BASED PAD                             Page 2-5

2.3.6  Reset the Circuit (RESET)

The RESET command transmits  a  RESET  to  the  remote  DTE,

resetting the virtual call.  The format for the command is:

PAD> RESET

Use this command if, for example, the  line  is  slow.   Any

data  in  transfer between the remote DTE and the host-based

PAD may be lost.

If you try to send a second reset before the remote DTE  and

the  PAD have completed handling the previous reset, the PAD

displays:

%PAD-W-WARNRES, Previous reset incomplete

2.3.7  Set Parameter Values (SET)

Use the SET command to set the values  for  PAD  parameters.

The format for the command is:

PAD> SET i:x,j:y,k:z ...

The letters i, j,  and  k  represent  the  parameters  whose

values  you want to change to x, y, and z respectively.  For

example, to set the values of PAD parameters 3, 4, 5, and 10

to 48, 160, 0, and 132 respectively, enter:

PAD> SET 3:48,4:160,5:0,10:132

If you do not specify any parameters, the PAD  program  sets

all  supported  parameters  to their initial values (section

2.5 lists the default values for the PAD  parameters).   See

Appendix  A  for  a  description  of  the PAD parameters and

values.

NOTE

Use  the  SET  command  before

establishing a call.  The X.29

terminal   driver   will   not

recognize     any     terminal

characteristics changed  by  a

SET  command  after a call has

been established.   Therefore,

you  have to use the PSISETPAD

utility    to    change    PAD

parameters   during   a  call.

Section 3.3  explains  how  to

use PSISETPAD.

If you want to change parameter values and display  the  new

values, enter:

PAD> SET?  i:x,j:y,k:z

USING A HOST-BASED PAD                   		Page 2-6

The SET?  command sets the specified  parameter  values  and

confirms  the  settings  by  displaying  the  new  parameter

values.  For example, if you enter:

PAD> SET?  3:126,4:20

the PAD program displays:

PAR 3:126,4:20

2.3.8  User Data - (U)

The U command lets you specify the contents of the user data

field  in  the  call  request  packet.   Therefore, use this

command before using the A command to establish the call.

Use this command in the following way:

U ccccc

where ccccc is user data.   Enter  user  data  in  the  call

request packet only if the remote DTE subscribes to the Fast

Select facility provided by your PPSN.

Note that the data that you enter must contain only printing

characters  (the PAD discards non-printing characters).  The

PAD program converts all lowercase characters to uppercase.

2.3.9  Clear a Virtual Call (CLR)

The CLR command clears any  virtual  call  in  progress  and

returns you to the PAD program.

2.3.10  Exiting from the Host-Based PAD (EXIT)

The EXIT command clears any virtual  call  in  progress  and

exits the PAD program.

2.4  PAD SERVICE SIGNALS

The host-based PAD returns service signals  in  response  to

command  signals  or  data  from  the  X.29  terminal.  This

section describes the format of  the  PAD  service  signals,

lists  their  effects, and indicates when they are likely to

occur.  If they indicate an  error,  the  required  recovery

action is also described.

%PAD-W-CLR, Call cleared

A remote DTE has received a CLR command signal from the

host-based  PAD,  and is in the process of clearing the

call.  The PAD program may also display:

USING A HOST-BASED PAD           			Page 2-7

%PAD-I-msg, CLR msg, txt

%PAD-I-CAUSEDIAG, Cause code xx, diagnostic code yy

The first message  is  an  information  message,  which

explains why the call has been cleared.  These messages

are  listed  below,  with   the   appropriate   acronym

replacing msg, and explanatory text replacing txt.  The

second message gives the cause code and the  diagnostic

code  (in  hexadecimal), here represented by xx and yy.

Refer to your  PPSN's  technical  documentation  for  a

description  of  the cause and diagnostic codes used on

your network.

The information messages are as follows:

%PAD-I-DER, CLR DER, Remote DTE is out of order

The remote DTE is not running.

%PAD-I-DTE, CLR DTE, Remote DTE cleared the call

The remote DTE cleared the call (cause code = 0).

%PAD-I-ER, CLR ERR, Local procedure error

The local machine detected a protocol error.   This

may  indicate  that  the  local  machine is wrongly

configured.

%PAD-I-INV, CLR INV, Invalid facility request

The network detected an error in the  call  request

packet.   This  is usually because you specified an

invalid DTE address.

%PAD-I-NA, CLR NA, Access barred

The remote DTE is in a closed user group  to  which

you have no access.

%PAD-I-NC, CLR NC, Network congestion

The network cannot complete the call.

%PAD-I-NP, CLR NP,Number for DTE is not obtainable

You selected a non-existent remote DTE.

%PAD-I-NRC, CLR NRC, No reverse charging

You specified a reverse charge call, but the remote

DTE does not accept reverse charging.

%PAD-I-OCC, CLR OCC, Number busy

The remote DTE cannot accept the call at this time.

USING A HOST-BASED PAD                         	Page 2-8

%PAD-I-RPE,CLR RPE, Remote procedure error

The remote DTE detected a protocol error.

%PAD-I-LINDWN, Line down

The connection between the local host and the  PPSN

went down.  Any calls in progress have been lost.

%PAD-I-LINREST, Line restart

The connection between the local host and the  PPSN

has  been  reestablished.   The  line  is  probably

working again, but all calls in progress  when  the

line failed have been lost.

%PAD-I-NETPROT, Network protocol error

The PSI  software  at  the  local  DTE  detected  a

protocol error.

%PAD-I-OPACT, Operator action

Operator action has caused the call to be cleared.

%PAD-I-TIMOUT, Timeout no response

The timer operated by the PSI software at the local

host  ran  out  before  the  remote DTE accepted or

rejected the call.  Usually, the timer operated  by

the network will run out first.

%PAD-I-CLRCONF, CLR CONF, Clear confirmation received

The remote DTE has cleared the call as a result of  the

CLR command signal it received from the host-based PAD.

%PAD-E-ERR, ERR ??, Error in command

The PAD program  cannot  set  up  the  call.   The  PAD

program may also display:

%PAD-E-NOLINES, No lines available for call

%PAD-I-IOSB, IOSB, iosb[0] xx, iosb[1] yy

This indicates either that the lines are not working or

that  they  already have the maximum number of circuits

in use.

iosb[0]xx, iosb[1]yy displays the contents of  the  I/O

Status  Block, where xx is the first longword and yy is

the second (in hexadecimal).  See the VAX-11  PSI  X.25

Programmer's Guide for a full description of the status

information held in the IOSB.

USING A HOST-BASED PAD                   		Page 2-9

%PAD-I-INTERRUPT, INTERRUPT received

The remote DTE has sent an interrupt  and  the  PAD  is

confirming receipt of the interrupt.

%PAD-I-RESET, RESET received

The PAD program also displays:

%PAD-I-CAUSEDIAG, Cause code OD, diagnostic code xx, yy

The  letters  xx  represent  the  cause  code,  yy  the

diagnostic  code  (in hexadecimal).  The remote DTE has

sent a reset to the  host-based  PAD.   Refer  to  your

PPSN's technical documentation for a description of the

cause and diagnostic codes used on your network.

XXX

Sent by the host-based PAD to  the  terminal  when  you

delete  a  line from the PAD's editing buffer.  See PAD

Parameters 15,16, 17, and 18 (1980 only) in Appendix  A

for  details  of  editing  PAD command signals and user

data held in the editing  buffer.   The  value  of  PAD

parameter 17 determines the delete-line character.

2.5  PARAMETER VALUES FOR THE HOST-BASED PAD

The host-based PAD supports the 18 parameters as defined  by

the CCITT X.3 recommendation (1980).

Table 2-1 lists initial values of these parameters (the  PAD

profile) for the host-based PAD.  Some values are determined

by the terminal characteristics (see Chapter 3, Section  3.2

for details on setting terminal characteristics) and are not

set until after call set-up and you have logged  in  to  the

remote DTE.

USING A HOST-BASED PAD                      	Page 2-10

Table 2-1

Host-Based PAD Profile

Parameter                   Value

1               1

2               Set by ECHO characteristic

3               126

4               10

5               Set by HOSTSYNC characteristic

6               1

7               0

8               0

9               Set by CRFILL characteristic

10              Set by WRAP characteristic and terminal width

11              Set by terminal speed

12              Set by TTSYNC characteristic

13              0

14              Set by LFFILL characteristic

15              1

16              127 (Delete character)

17              18 (control R)

18              24 (control X)

Read Chapter 3 and Appendix A before  changing  any  of

the above values.

NOTES

Parameter 1

Values 2 to 31 allow escape to command  mode  with  the

corresponding   control   character.   CCITT  does  not

specify action on these values.

Parameter 3

Setting parameter 3 to 128  sets  the  data  forwarding

characters  to  all  control  characters not defined by

CCITT values.  CCITT does not specifiy what these other

characters are.

USING A HOST-BASED PAD                                                   Page 2-11

This parameter is generally set by the remote DTE,  and

you should not need to change the setting.

Restrictions

1.   The PAD program sets the terminal to  PASSALL  mode  so

that the program receives all characters.  As a result,

the  terminal  does  not  display  broadcast  messages.

Chapter   3  describes  the  terminal  characteristics,

including PASSALL.

If you use the PAD program from another machine  (using

SET  HOST) and the DECnet logical link goes down, or if

the PAD program does not exit normally  for  any  other

reason,  the terminal characteristics do not get reset.

In particular  the  characteristics  are  left  set  to

PASSALL and NOECHO.

2.   The PAD program will be slow if parameter 5 is set to 1

and  you have SET HOST from another machine in order to

use the program.

3.   The PAD program  echoes  input  of  characters  itself.

This  may  cause  a  delay  in  echoing,  and  may also

increase the CPU time that it uses.

4.   Do not enter CTRL/P from the  console  terminal  unless

you  have  set  the  terminal to LOCAL DISABLE with the

keyswitch.