NOTE FOR RECIPIENTS OF ANSI LABELED TAPES:

ANSI labeled tapes are provided to KERMIT requestors who:

(a) asked for them;

(b) didn't mention tape format specifically, but have VAX/VMS;

(c) asked for a tape in a format we cannot provide; or

(d) did not specify a system or tape format.

All systems are supposed to be able to read ANSI labeled tapes, which,

according to ANSI standard X3.27-1978: Magnetic Tape Labels and File Structure

for Information Interchange, should be the universal medium for exchange

between mainframe computers.  If your system can't read this kind of tape, it

might be a good idea to get hold of the standard and implement it.

The tape is written as follows:

Tracks:  9

Density: 1600 bits per inch, phase encoded.

Parity:  Odd

ANSI labels:

Volume label: KERMIT

File header labels give name of each file.

ANSI format: "D" - variable length records with length fields at the

beginning of each record, line terminators stripped, and no record crossing a

block boundary.  "F" (fixed-length records), although simpler to read, was

not used because the KERMIT distribution would overflow smaller tape reels

if written in this way because of the space wasted by padding.

Character encoding: ASCII (ANSI X3.4-1968).

Record length: variable, maximum 200 (no lines of text or program source in

the Kermit distribution are more than 200 characters long, very few are longer

than 100).

Block length: 2000

Only printable files (text, program source, text formatter source, or hex)

are included on this tape.

[End of <KERMIT>ANSI.TXT, 5 May 83]