FOR RECIPIENTS OF ANSI LABELED TAPES:

ANSI labeled tapes are provided to KERMIT requestors who:

(a) asked for them;

(b) didn't mention tape format specifically, but have VAX/VMS;

(c) asked for a tape in a format we cannot provide; or

(d) did not specify a system or tape format.

All systems are supposed to be able to read ANSI labeled tapes, which,

according to ANSI standard X3.27-1978: "Magnetic Tape Labels and File

Structure for Information Exchange", should be the universal medium

for exchange between mainframe computers.  If your system can't read

this kind of tape, it might be a good idea to get hold of the standard

and implement it.

The tape is written as follows:

Tracks:   9

Density:  1600 bits per inch

Parity:   Odd

ANSI labels:

Volume label: KERMIT

File header labels give name of each file.

ANSI Format: "D" -- variable length records with length fields at the beginning

of each record, line terminators stripped, and no record crossing a block

boundary.  "F" (fixed length record) format, although simpler to read, was

not used because the KERMIT distribution would not fit on a standard 2400'

reel of tape if written in this way because of the space wasted by padding.

Character Encoding:  ASCII (ANSI X3.4-1968)

Record Length:  Variable, maximum 200 (no lines of text or program source in

the Kermit distribution are more than 200 characters long; very few are

longer than 100).

Block Length:  2000

Only printable files (text, program source, text formatter source, or hex) are

included on this tape.

[End of <KERMIT>ANSI-TAPE.DOC]