1.0  DESCRIPTION

VTERM is a 'virtual terminal' handler for VAX/VMS.  Once in VTERM,  you

are  logically  connected to the modem as if you were hard-wired to it.

VTERM has a small number of commands used to  control  it's  execution.

To issue a VTERM command, enter control-A, which will display the VTERM

command prompt <VTERM:>.  The following terminology is used  throughout

VTERM documentation:

o  Local computer - The VAX on which VTERM is run.

o  Remote computer  -  The  computer  to  which  the  remote  port  is

connected.

o  Local terminal - The terminal (on the local VAX) from  which  VTERM

is run.

o  Remote terminal - The 'outgoing' port on the local  VAX,  which  is

connected to the remote computer, typically by way of a modem.

2.0  INITIALIZATION

When VTERM is entered, it takes the following steps:

o  Assigns channels  to  the  terminal  devices  using  the  following

logical names:

Name            Device

----            ------

VTERM$REMOTE    Port to remote computer

SYS$COMMAND     Port on local computer

o  Sets the remote terminal  speed  to  the  value  specified  in  the

logical name VTERM$SPEED (or 1200 baud if the logical is invalid)

o  Opens the file VTERM$INIT and executes any vterm commands  that  it

contains.

o  Displays the message "VTERM - Ready" on the local terminal.

o  Waits for input from either the local or remote terinal.

3.0  VTERM COMMANDS

A brief description of the allowable VTERM commands follows:

o  EXIT - Exits from VTERM.

o  HELP - Displays information about VTERM commands.

Page 2

o  SET - Allows you to set various values

-  BUFFER

Format:  SET BUFFER /REMOTE=nn

nn  must  be  an  integer  between  1  and  128.   When   VTERM

initializes,  this  value  is  set  to 4.  In general, a larger

value decreases CPU utilization by VTERM, while smaller  values

give  a  more  comfortable  'feel'  to  the  user.  With values

greater than one, up to a second  may  elapse  between  when  a

character  is  received from the remote computer and it appears

on the local screen.

-  LOCALECHO

Format:  SET LOCALECHO

This sets VTERM in 'half-duplex' mode, where the input from the

local  terminal  is echoed by VTERM as it is sent to the remote

computer.

-  SPEED

Format:  SET SPEED = speed

Speed may be 110, 300, 1200, 2400, 4800, or 9600.   When  VTERM

is  initialized,  speed  is  set  to the value specified by the

logical name VTERM$SPEED.  If the logical does  not  exist,  or

translates to an invalid value, 1200 baud is used.

-  LOG

Format:  SET LOG filename

The VTERM log is  a  generally  faithful  reproduction  of  the

output   to  the  local  terminal,  but  may  have  some  small

differences.   In  particular,  VTERM  edits  any  record  that

contains  a backspace character to remove the backspace and the

character preceeding it from the buffer.

Qualifiers

/APPEND

Specifies that the  new  log  should  be  appended  to  an

existing  one.   If no log exists with the name specified,

creates a new one.

/CLOSE

Specifies that the current log should  be  closed.   If  a

file  name  is  specified  with  /CLOSE,  the file name is

ignored.  /CLOSE should  only  be  used  to  speicify  the

/PRINT qualifier.  No new log file is opened.

/PRINT

Specifies that the log  file  should  be  spooled  to  the

Page 3

default print queue when it is closed.

-  NOLOG

Format:  SET NOLOG

Stops logging and closes the log file.

o  STOP Exits from VTERM.  Control-Z is equivalent to STOP.  VTERM.COM

will  prompt  you for whether or not it should deallocate the port.

If the auto-dial modem was used to make the call, deallocating  the

port will terminate the phone connection.

4.0  USING THE VADIC VA3451 AUTO-DIAL MODEM WITH VTERM

Enter control-E <CR>

This will invoke a message and "*" prompt.

Enter D<CR>

Modem will prompt "NUMBER?"

Enter the desired number (dashes and parens are allowed, but

ignored). Use the backspace key to make corrections.

Notes:

- The number will not be echoed as you enter it.

Terminate the number with <CR>.   The modem will then

echo the number.

- The letter "K" will cause a 5 second pause in the dialing

sequence.  It is most useful when specified at the end of the

phone number to extend the number of seconds required for a

connection to be established.

Hit <CR> to start dialing.

After a period of time, the modem will respond with either

"ON LINE" or "FAILED CALL".  ON LINE indicates that the call

completed, and you are now connected to the target machine.

FAILED CALL indicates either that the line was busy, did not

answer, or the call did not complete in a specified time

period.  You can not tell why the call did not complete.