TAPECHECK is a program designed to quickly and inexpensively test the

data reliability of a magnetic tape.  It does this by writing blocks

of data on the entire tape, rewinding the tape, and reading the entire

tape back in.  Data blocks which do not match the original data pattern

or block length are announced along with the approximate position on the tape.

THIS PROGRAM OVERWRITES ALL THE DATA ON THE TAPE.  Please do not attempt

to check a tape that has valuable information on it.

The check performed by this program is not as good as is available from

any hardware tape evaluator.  The errors that this program reports might

conceivably be caused by the tape drive or some other factor.  However,

this program should run on virtually any VMS configuration with a mag tape.

It has been tested on various combinations of (750/780),(TS11/TU77/TU78),(V3.6).

The author would be pleased to hear of any bugs found or improvements made.

Restrictions:

If you have a tape drive that does not run at 800, 1600 or 6250 BPI,

the program will complain and quit.  If the density you run at does not

match the density the system thinks you are at, the program may run but

will quote lengths incorrectly.

To use the program:

1) Mount the tape /FOREIGN at desired density* with logical name SCRATCH.

$ MOUNT name/FOR  SCRATCH

*	If the tape has been previously used at

another density, the tape drive may set

the density for you.  If this is a problem,

INIT the tape first at the required density.

2) Run the program.

Sample Output:

Checking tape at 1600 BPI

The tape is about 2332 feet long

A Tape error occurred at about 2301 feet

A tape error occurred at about 2302 feet

2 tape errors occurred in 2332 feet

3) Dismount the tape.

There is reasonable documentation within the source file for finding out

how it works if anyone is interested.  The original program has been modified

to output error messages via a Fortran subroutine so that it can be run from

within a Batch procedure.  The tape check has been made very stringent in that

retries are inhibited both for writing and for reading.

This program writes on about 95% of the tape surface, the rest goes to

interrecord gaps.  Obviously the gaps are not checked for data reliability.

It is difficult to do much better than that on a standard tape drive.

Original Version:

[DECUS.VAX82B.SEARLE]

Jim Lellman

G. D. Searle & Co.

Research Computing Services

4901 Searle Parkway

Skokie Il, 60077

October 14, 1982

Modified:	James G. Downward

KMS Fusion, Inc.

P.O. Box 1567

Ann Arbor, Mich 48106

07-Oct-1984