SCANLIB

DOCUMENTATION FOR SCANNER-TO-VAX SUBROUTINES

VERSION 1.0

M. Sheehan

UNC-Wilmington

Systems Programming

SCANLIB is a collection of subroutines to load data from an NCS 7004

Optical Scanner to a VAX.  The subroutines may be called in any

language. The user's program must be linked to the SCANLIB library.

Once the user program is compiled and linked to the subroutine

library, re-linking will not be necessary when additions or

enhancements are made to the library routines. This document

describes user program requirements and the calling standard for the

subroutines.

USER PROGRAM REQUIREMENTS

Three variables must be declared in the user program. The following

names of these variables are recommended:

VARIABLE NAME				DECLARATION TYPE

=============				================

INPUT_BUFFER				CHARACTER	(512 bytes)

Receives each record from the scanner.

INPUT_SIZE				INTEGER		(4 bytes)

Indicates the size of a record from the scanner.

EOF					LOGICAL		(1 byte)

Initial value = FALSE

Flags an End of File condition from the scanner.

These variables are used as arguments to the subroutines and are

available to the user program for examination. The initial value of

the logical variable EOF should be declared as FALSE, or set equal to

zero through an appropriate assignment statement.  In the following

section, CALLABLE SUBROUTINES, the parameter arguments used match the

variable names apecified above.

CALLABLE SUBROUTINES

The following describes the calling standard and function of each

subroutine in SCANLIB. The ARGUMENTS (parameters) documented must be

included in calls where specified in the calling standard, in the

order specified.

SCN_INIT

========

EXAMPLE CALL			ARGUMENTS (parameters)

------------			---------

CALL SCN_INIT			none

The subroutine SCN_INIT assigns and allocates a port on VXA,

designated for connection to the NCS Scanner, and prompts the user

for the name of a file in which the scanner records are to be placed.

The file is opened as a VARIABLE LENGTH SEQUENTIAL file with a

maximum record length if 512 bytes.  This routine should be called

ONCE at the beginning of the user program.

SCN_STATUS

==========

EXAMPLE CALL			ARGUMENTS (parameters)

------------			---------

CALL SCN_STATUS			none

The subroutine SCN_STATUS continuously sends "requests for status" to

the NCS Scanner until the scanner responds with a code that

corresponds to "Scanner Ready".  Additionally, the subroutine is

automatically called whenever a jam or timeout is perceived. In the

event of a scanner JAM, a table lookup is made, and the probable

cause of the jam is displayed at the user terminal if the cause is

known. After the jam is corrected, control will again pass to the

user program. This routine is called ONCE in the user program AFTER

the subroutine SCN_INIT is called.

SCN_GETREC

==========

EXAMPLE CALL			ARGUMENTS (parameters)

------------			---------

CALL SCN_GETREC			INPUT_BUFFER,INPUT_SIZE,EOF

(Required)

SCN_GETREC requests a record from the scanner and returns three

values:

INPUT_BUFFER	(Data record from scanner, max=512 bytes)

INPUT_SIZE	(Length of data in INPUT_BUFFER)

EOF		(End of file condition: .true. if end of

file, .false. if not end of file)

As the data transferred into the buffer INPUT_BUFFER, it is displayed

on the user's screen.  The data in INPUT_BUFFER may then be checked,

edited, or transformed as the user sees fit.  The length of the data

is stored in the variable INPUT_SIZE. If an end of file record is

transmitted from the scanner via an EOJ record, the logical variable

EOF will be TRUE, or will contain the integer value 255 (depending

upon the language and declaration), and INPUT_BUFFER will be empty or

will contain INVALID DATA. EOF should be checked after each call to

SCN_GETREC to avoid writing the EOF record to the file. During a

SCN_GETREC call, if there is either a timeout or a jam on the scanner

end, control will pass to status/jam routines until corrected.

Each time a record is needed from the scanner, SCN_GETREC must be

called.

SCN_REJECT

==========

EXAMPLE CALL			ARGUMENTS (parameters)

------------			---------

CALL SCN_REJECT			none

After a record is loaded into INPUT_BUFFER via a call to

SCN_GETREC, the user may, for failing user edit or other reason,

reject the record by calling this routine.  The contents of the input

buffer are erased. Code is then generated to the scanner to feed the

sheet to the secondary hopper, wait 5 seconds and return to the user's

program.

SCN_WRITE

=========

EXAMPLE CALL			ARGUMENTS (parameters)

------------			---------

CALL SCN_WRITE			INPUT_BUFFER,INPUT_SIZE

(required)

Users may call this routine to write the contents of the input buffer

to the file designated in the SCN_INIT routine.  The contents of

INPUT_BUFFER from position 1 to INPUT_SIZE are written to the file.

SCN_DEINIT

==========

EXAMPLE CALL			ARGUMENTS (parameters)

------------			---------

CALL SCN_DEINIT			none

This subroutine is called to CLOSE the file opened by the user in

SCN_INIT, deallocate, and deassign the port from the scanner to the

VAX.  Normally this routine would be called just prior to end of the

user's program.

LINKING USER PROGRAMS TO SCANLIB

The user's program, after it is compiled without errors, must be

linked to the shareable image library SCANLIB.  This is done by

pointing the linker to the library via the LINK command:

$ LINK <User Program Name>, SYS$INPUT/OPTIONS

SYS$LIBRARY:SCANLIB/SHARE

^Z (Control Z)

PROGRAM EXECUTION

For VAX access, the NCS 7004 scanner must be set in SCAN-TO-COMM

mode. The scanner program and the user program may be started

independantly in either order.  If the user program starts first,

when SCN_STATUS is called the message "Scanner not ready, retry" will

be displayed every 5 seconds until the scanner program starts and the

first record is loaded.  If the scanner program starts first, the

first record will be read and the scanner will wait until it is

polled by the user program. When an END OF FILE is received from the

scanner, the message "End of File received from Scanner" will be

displayed; the file will be closed; and the scanner will be released

from the VAX.

EXAMPLE USER PROGRAM -- FORTRAN

The following is an example user program using FORTRAN to call the

SCANLIB routines in order to access records from the scanner and write

them to a file in the user's default directory.

PROGRAM EXAMPLE_SCANNER_PROGRAM

c

c

IMPLICIT       	INTEGER*4	(A-Z)

c

c		Set up buffer to receive data.

c

CHARACTER*512	INPUT_BUFFER

LOGICAL*1	EOF /.FALSE./

INTEGER*4	INPUT_SIZE

c

c		Initialize the scanner and open a

c		file to receive the scanner data.

c

CALL SCN_INIT

c

c		Send requests for status to the scanner

c		until the scanner is ready on SCAN-TO-COM.

c

CALL SCN_STATUS

c

c		Set loop to get records till end of file (EOF)

c		is received from the scanner.

c

I = 0

DO 20 WHILE (.NOT. EOF)

I = I + 1

WRITE (6,*) ' Record number: ', I

c

c			Get a record.

c

CALL SCN_GETREC ( INPUT_BUFFER, INPUT_SIZE, EOF)

c

c		<<<  User program edit checks HERE, if any. >>>

c

c

c		Write the record to the file if EOF is not reached.

c

IF (.NOT. EOF) THEN

CALL SCN_WRITE (INPUT_BUFFER, INPUT_SIZE)

END IF

20	CONTINUE

c

c		Close the file and deinitialize the scanner.

c

200 	CALL SCN_DEINIT

END

EXAMPLE USER PROGRAM -- BASIC

The following is an example user program using BASIC to call the

SCANLIB routines in order to access records from the scanner and write

them to a file in the user's default directory.

10	REM	EXAMPLE_SCANNER_PROGRAM

!

!      	Set up buffer to receive data.

!

20	DECLARE STRING INPUT_BUFFER, BYTE EOF, WORD INPUT_SIZE

25	INPUT_BUFFER = SPACE$(512)

!

!		Initialize the scanner and open a

!		file to receive the scanner data.

!

30	CALL SCN_INIT

!

!		Send requests for status to the scanner

!		until the scanner is ready on SCAN-TO-COM.

!

40 	CALL SCN_STATUS

!

!		Set loop to get records till end of file (EOF)

!		is received from the scanner.

!

50	I% = 0

60	WHILE NOT EOF

I% = I% + 1

PRINT " Record number: ", I%

!

!			Get a record.

!

70	    	CALL SCN_GETREC ( INPUT_BUFFER, INPUT_SIZE, EOF)

!

!		<<<  User program edit checks HERE, if any >>>

!

!

!		Write the record to the file if EOF is not reached.

!

80		IF (NOT EOF) THEN

CALL SCN_WRITE (INPUT_BUFFER, INPUT_SIZE)

END IF

90 	NEXT

!

!		Close the file and deinitialize the scanner.

!

100 	CALL SCN_DEINIT

110	END

EXAMPLE USER PROGRAM -- COBOL

The following is an example user program using COBOL to call the

SCANLIB routines in order to access records from the scanner and write

them to a file in the user's default directory.

IDENTIFICATION DIVISION.

PROGRAM-ID. EXAMPLE_SCANNER_PROGRAM.

ENVIRONMENT DIVISION.

DATA DIVISION.

WORKING-STORAGE SECTION.

*

*	Set up buffer to receive data.

*

77  RECORD-COUNT   PIC 9(5)     VALUE ZERO.

01  EOF-SWITCH     PIC 9(3)	COMP.

88  EOF                     VALUE 255.

01  INPUT-SIZE     PIC X(4)     VALUE ZERO.

01  INPUT-BUFFER   PIC X(512).

PROCEDURE DIVISION.

000-HOUSE-KEEPING.

*

*	Initialize the scanner and open the output file.

*

CALL "SCN_INIT".

001-MAIN.

*

*	Send request for status to the scanner until scanner is ready.

*	on SCAN-TO-COM

*

CALL "SCN_STATUS".

002-PROCESS.

*

*	Set loop to get records until end-of-file (EOF) is

*	received from the scanner.

*

PERFORM 003-CALL-SCANNER UNTIL EOF.

GO TO 009-END-OF-JOB

003-CALL-SCANNER.

ADD 1 TO RECORD-COUNT.

DISPLAY "Record number: ", RECORD-COUNT.

*

*	Call scanner routine, placing data in INPUT-BUFFER, and setting

*	EOF-SWITCH to 255 at END-OF-DATA.

*

CALL "SCN_GETREC" USING BY DESCRIPTOR INPUT-BUFFER

BY REFERENCE INPUT-SIZE, EOF-SWITCH.

*

*	<<< User program edit checks HERE, if any >>>

*

*	Write the record to the file if EOF is not reached.

*

IF NOT EOF

CALL "SCN_WRITE" USING BY DESCRIPTOR INPUT-BUFFER

BY REFERENCE INPUT-SIZE.

009-END-OF-JOB.

*

*	De-initialize the scanner and close the output file.

*

CALL "SCN_DEINIT".

STOP RUN.