1. PTY Driver

This  chapter  describes  the  use  of  the VAX/VMS Pseudo Terminal Driver as

implemented Carnegie-Mellon University Computer Science Department.  Additional

information about VMS device drivers and the VMS operating system interface can

be found in VAX/VMS I/O User's Guide, and in VAX/VMS System  Service  Reference

Manual.

1.1 PTY Driver Features and Capabilities

Pseudo  terminals  (or  PTYs)  are  virtual  terminals in the system.  Unlike

normal terminals where the I/O is actually done with a physical  device,  PTY's

I/O  has  no  interface to any physical device.  In this way they are much like

mailboxes.

1.1.1 Concept of Control and Terminal Device

A single PTY consists of two devices, a control and a terminal device.    The

terminal device is the device that acts like a terminal.

The  control  device  on  VMS is named ``PYAx:''.  For example, a PTY control

device could be named ``PYA1:''.  The terminal device portion  of  the  PTY  is

name  ``TWAx:''.    The  terminal  device  ``TWA1:'' is the mate to the control

device ``PYA1:''.  Similarly, the control device ``PYA5:'' is the mate  to  the

terminal device ``TWA5:''.

1.1.1.1 Similarities Between PTYs and Terminals

The  terminal  portion  of  a  PTY  behaves  very much like a regular VAX/VMS

terminal.  The terminal portion of a PTY has

- Type-ahead

- Specifiable or default line terminators

- Special operating modes, such as NOECHO and PASSALL

- American National Standard escape sequence detection

- Terminal/mailbox interaction

- Terminal control characters  such  as  Sontrol-S  and  Control-Q  for

starting  and  stopping  output, Control-O for discarding output, and

all other special characters that are handled by the VAX/VMS terminal

driver.

- Limited  full-duplex  operation(simultaneously  active read and write

requests)

1.1.1.2 Differences Between PTYs and Terminals

The difference between a VAX/VMS terminal and the terminal portion of  a  PTY

is  where  the  input  comes  from  and where the output goes to.  On a VAX/VMS

terminal, the input comes from an actual terminal and the  output  goes  to  an

actual  terminal.    On  a  PTY the input comes from the control device and the

output goes to the control device.  In order to simulate someone typing at  the

terminal  device  ``TWA5:''  we must write to the control device ``PYA5:''.  In

order to read what has been typed out to the terminal device ``TWA5:''  we must

read  from  the  control  device  ``PYA5:''.  PTYs are like terminals that only

software can access.

There is no buffering of the input on the control device.  It is possible  to

fill up the terminal device's typeahead buffer by writing large amounts of data

to the control device.

PTYs are virtual devices.  When allocating  a  PTY,  they  behave  much  like

mailboxes or network devices.  To allocate a PTY, simply allocate the first one

('PYA0:').  The allocating routine will be create a new PTY and  assign  it  to

you.  The PTY will be deallocated when no process is referencing the device.

1.1.1.3 Uses of PTYs

There  is  no  device  from  DEC  which suffices for remote logins when using

non-DECnet protocols.  The remote terminal driver expects the remote node to do

the local line editing.  This includes delete, control-o, control-s, control-q,

and much other stuff.  PTYs handle the local line editing locally.    When  the

line  editing  is  done locally, the remote machine has to know much less about

VMS and its terminal handling conventions.

1.2 Installing PTYs

In VAX/VMS V3.0, several pieces of software support the  terminal  interface.

The  hardware  independent portion is SYS$SYSTEM:TTDRIVER.EXE.  This version of

PTYs does not require any changes or patches to this code.    There  are  other

pieces   of   software  for  particular  pieces  of  hardware.    The  code  in

SYS$SYSTEM:DZDRIVER.EXE is the device driver required for  DZ11's  and  DZ32's.

The  code in SYS$SYSTEM:YCDRIVER.EXE is the device driver required for DMF32's.

PTY drivers consist of  two  device  drivers,  TWDRIVER.EXE  for  the  terminal

portion of the PTY's, and PYDRIVER.EXE for the control portion.

1.2.1 Compiling Sources

There   are  several  pieces  of  source  code  that  make  up  PTY  drivers.

PYDRIVER.MAR is the source for the control portion.  TWDRIVER.MAR is the source

for the terminal portion.

Before compiling, be sure to edit the MACRO sources, examine the 7th

line of the source files and make sure you will be building the appropriate

drivers for your version of VMS.  The drivers, as distributed, will be

assembled for a VMS V5 system.  If you wish to build drivers for VMS V4,

you will need to uncomment the VMS_V4 = 1 symbol definition.

The commands for compiling and linking the devices are:

$ ! Compile the drivers

$ MACRO TWDRIVER.MAR

$ MACRO PYDRIVER.MAR

$ ! Link the drivers

$ LINK TWDRIVER,DRIVER/OPTION

$ LINK PYDRIVER,DRIVER/OPTION

Ignore the LINK warning message

%LINK-W-USRTRF, image has no user transfer address

as it is normal for device drivers.

1.2.2 Installing the Devices

To  load  a  PTY,  ensure you have CMKRNL privilege and:

$ RUN SYS$SYSTEM:SYSGEN

SYSGEN> LOAD device:[directory]TWDRIVER

SYSGEN> LOAD device:[directory]PYDRIVER

SYSGEN> CONNECT TWA0/NOADAPTER/DRIVER=TWDRIVER

SYSGEN> CONNECT PYA0/NOADAPTER/DRIVER=PYDRIVER

You may want to build a command file which installs the devices at boot  time

by modifying SYS$MANAGER:SYSTARTUP.COM.

Don't try to use TWB0 or PYB0.  The code is not built to handle these.  It is

only built to handle the PYA and TWA control and terminal devices.

1.3 Device Information

User processes can obtain terminal and control device  information  by  using

the  $GETDVI,  $GETCHN and $GETDEV system services (see VAX/VMS System Services

Reference Manual).

It is recommended that new programs make use  of  the  $GETDVI  and  $GETDVIW

system services.

1.3.1 PTY Terminal Device Dependent Information

The  information  returned  about  a terminal device is in the same format as

information returned about a regular VAX/VMS terminal.  By only looking at  the

information returned from $GETCHN and $GETDEV system services, it is impossible

to tell the difference between a PTY terminal device  and  an  actual  terminal

device.

1.3.2 PTY Control Device Dependent Information

When  applied  to  a  PTY  control  device, $GETCHN return information in the

format as show in Figure 1-1.

31        24 23        16 15         8 7          0

-----------------------------------------------------

|              Device Characteristics               |

-----------------------------------------------------

|       Buffer Size       |    Type    |    Class   |

-----------------------------------------------------

|           Unused        |     TW Unit Number      |

-----------------------------------------------------

Figure 1-1:   PTY Control Device Dependent Information

The first longword contains device-independent data.  The  second  and  third

longwords contain device-dependent data.

1.3.2.1 Device Characteristics

The  characteristics  of  the  the  PTY  control device (PYA) can be found by

either using the first longword returned by  the  $GETDEV  and  $GETCHN  system

services, or by using the item code DVI$ DEVCHAR to the $GETDVI system service.

A PTY control device has the following characteristics.

- DEV$M AVL - On line and available

- DEV$M IDV - Capable of input

- DEV$M ODV - Capable of output

- DEV$M REC - Record oriented

A PTY control device does not have the following characteristics.

- DEV$M CCL - Carriage Control

- DEV$M TRM - Terminal Device

1.3.2.2 Buffer Size

Also returned from $GETDVI by using item code DVI$ DEVBUFSIZ.

1.3.2.3 Device Type

The  device  type  is  DT$ PY.  Since a PTY is a nonstandard device, you will

probably not find the correct macros, literals or  constants  for  this  device

type.  This field should have a value of 0.

Also returned from $GETDVI by using item code DVI$ DEVTYPE.

1.3.2.4 Device Class

The  device  class  is DC$ PY.  Since a PTY seems to be such an odd creature,

this field should be FF in hex, or 177 in octal.

Also returned from $GETDVI by using item code DVI$ DEVCLASS.

1.3.2.5 PT Unit Number

The unit number is unit number of the associated PTY terminal device.

The unit number is also returned as the low sixteen bits in DVI$ DEVDEPEND by

$GETDVI.

1.4 PTY Function Codes

The  function  codes for the terminal device portion of a PTY are exactly the

same as those for regular VAX/VMS terminals.  For more information  on  VAX/VMS

terminal I/O see VAX/VMS I/O User's Guide.

The  basic  function  of the control portion of a PTY are read, write and set

mode or characteristics.

A user does not need to have assigned the PTY terminal device in order to  do

I/O  operations  on  the  PTY  control device.  If the terminal device has type

ahead enabled, sending the right characters at the control device will  send  a

message  to  OPCOM  to  start  running  SYS$SYSTEM:LOGINOUT.EXE on the terminal

device.

1.4.1 Read

The basic purpose of a PTY control device read is to transfer data  from  the

output buffer of the PTY terminal device to a user specified buffer.  There are

three read functions which a user can apply to a PTY control device.

- IO$ READVBLK - Read virtual block

- IO$ READLBLK - Read logical block

- IO$ READPBLK - Read physical block

A read is complete if either of the below conditions occur:

- The user specified buffer is full

- At least one character is available from the PTY terminal device

The read function codes can  take  the  following  device/function  dependent

arguments:

- P1  =  The  starting virtual address of the buffer that is to receive

the data read

- P2 = The size of the buffer that is to receive the data read in bytes

- P3, P4, P5, P6 = ignored

1.4.2 Write

The basic purpose of a PTY control device write is to transfer data from  the

user  specified  buffer  to  the  typeahead  buffer of the PTY terminal device.

There are three write functions which a user can apply to a PTY control device.

- IO$ WRITEVBLK - write virtual block

- IO$ WRITELBLK - write logical block

- IO$ WRITEPBLK - write physical block

The write function codes can take  the  following  device/function  dependent

arguments:

- P1 = The starting virtual address of the buffer that is to be written

to the PTY terminal device

- P2 = The number of bytes that are to be sent

- P3, P4, P5, P6 = Ignored

1.4.3 Set Mode and Set Characteristics

The Set Mode function affects the mode and temporary characteristics  of  the

associated PTY control device.  Set Mode is a logical I/O function and requires

no privilege.  A single function code is provided:

- IO$ SETMODE

The Set Charateristics function affects the permanent characteristics of  the

associated  PTY control device.  Set Characteristics is a physical I/O function

and requires the privilege  necessary  to  perform  physical  I/O.    A  single

function code is provided:

- IO$ SETCHAR

These  functions take the following device/function dependent arguments if no

function modifiers are specified:

- P1 = address of characteristics buffer

- P2, P3, P4, P5, P6 = ignored

The P1 argument points to a quadword block, as shown in Figure 1-2.

31        24 23        16 15         8 7          0

-----------------------------------------------------

|       Buffer Size       |    Type    |    Class   |

-----------------------------------------------------

|                      Unused                       |

-----------------------------------------------------

Figure 1-2:   Set Mode Characteristics Buffer

1.4.3.1 Function Modifiers

Function Modifiers to the control device currently do not affect the state or

accessibility of terminal device.

You  cannot  change  the  mode  or  characteristics of the terminal device by

adding function modifiers to the I/O routines  for  the  control  device.    An

earlier version of PTYdrivers supported this feature.  This feature has not yet

been added to this version.

1.4.4  Sense Mode and Sense Characterisitics

The two function codes to sense the mode of the PTY control device are:

- IO$ SENSEMODE

- IO$ SENSECHAR

The IO$ SENSEMODE function returns the process-associated, that is temporary,

characteristics  of the PTY control device.  The IO$ SENSECHAR function returns

the permanent characteristics of the PTY control  device.    The  IO$ SENSEMODE

function  is  a  logical function and requires no privilege.  The IO$ SENSECHAR

function is a physical function and requires the privilege necessary to perform

physical I/O.

These function codes take the following device/function dependent.

- P1 = address of the quadword characteristics buffer

- P2, P3, P4, P5, P6 = ignored

The  P1  argument  points to a quadword block which is the same format as the

Set Mode Characteristics Buffer, shown in Figure 1-2

1.4.5 I/O Status Block

The I/O status block formats for read, write, set mode  and  sense  mode  I/O

functions are shown in Figures 1-3 and 1-4.

31        24 23        16 15         8 7          0

-----------------------------------------------------

|       Byte Count        |         Status          |

-----------------------------------------------------

|                      Unused                       |

-----------------------------------------------------

Figure 1-3:   IOSB Contents - Read and Write Function

31        24 23        16 15         8 7          0

-----------------------------------------------------

|         Unused          |         Status          |

-----------------------------------------------------

|                      Unused                       |

-----------------------------------------------------

Figure 1-4:   IOSB Contents - Set and Sense Mode and Characteristics Functions

The status indicates the succes or failure of the specified operation.  Below

are possible values for the status field:

- SS$ ABORT - The operation was canceled by the Cancel I/O  on  Channel

($CANCEL)  system service.  Applicable only if the drive was actively

involved in an operation.

- SS$ NORMAL - Successful Completion

- SS$ ACCVIO - The specified buffer is not accessible to the  specified

process

1.5 Possible Improvements and Bugs

Watch  the  error counts on the devices.  Occasionally, a PTY terminal device

might get a timeout.  Perhaps,  it  is  not  executing  at  a  sufficient  IPL.

Somehow, we aren't entering the startio routine soon after the UCB$V_TIM bit is

set in UCB$W_STS.

It may be useful for the associated mailbox of the PY  device  to  receive  a

message  every  time  the  stop,  start, abort, resume, xoff, or xon routine is

called.  This would permit programs that control the  PY  device  to  determine

when  to slow down the data rate, or abort the output in the buffers which have

already been read.

Table of Contents

1. PTY Driver                                                                 2

1.1 PTY Driver Features and Capabilities                                   2

1.1.1 Concept of Control and Terminal Device                           2

1.1.1.1 Similarities Between PTYs and Terminals                    2

1.1.1.2 Differences Between PTYs and Terminals                     2

1.1.1.3 Uses of PTYs                                               2

1.2 Installing PTYs                                                        2

1.2.1 Compiling Sources                                                2

1.2.2 Installing the Devices                                           2

1.3 Device Information                                                     2

1.3.1 PTY Terminal Device Dependent Information                        2

1.3.2 PTY Control Device Dependent Information                         2

1.3.2.1 Device Characteristics                                     2

1.3.2.2 Buffer Size                                                2

1.3.2.3 Device Type                                                2

1.3.2.4 Device Class                                               3

1.3.2.5 TW Unit Number                                             3

1.4 PTY Function Codes                                                     3

1.4.1 Read                                                             3

1.4.2 Write                                                            3

1.4.3 Set Mode and Set Characteristics                                 3

1.4.3.1 Function Modifiers                                         3

1.4.4  Sense Mode and Sense Characterisitics                           3

1.4.5 I/O Status Block                                                 3

1.5 Possible Improvements and Bugs                                         3

List of Figures

Figure 1-1:   PTY Control Device Dependent Information                     2

Figure 1-2:   Set Mode Characteristics Buffer                              3

Figure 1-3:   IOSB Contents - Read and Write Function                      3

Figure 1-4:   IOSB Contents - Set and Sense  Mode  and  Characteristics    3

Functions