VAX/VMS DGI-086/7 SOFTWARE DOCUMENTATION

The  DGI-086/7   asynchronous  reader-punch  controller   can  be

interfaced with  a VAX computer  via a communications  controller

such  as the  DMZ-32 or  through a  properly mapped ethernet port

using the LATSYM driver.

The  RP   software  requires  the   logical  definition  of   the

RP_CARDREADER_PORT. For non-ethernet  controllers the VMS command

for this definition for a  DGI-086/7 connected to port TXA0 would

look like this:

$ DEFINE RP_READER_PUNCH_PORT TXA0

For ethernet  networked systems, the system  manager must use the

LATCP utility to  map the actual port to  a logical terminal. The

VMS  commands  to  accomplish  this  for  a  logical port LTA1001

connected to  port 1 on a  DECserver 100 or 200  named S01 are as

follows:

$ MCR LATCP CREATE PORT LTA1001

$ MCR LATCP SET PORT LTA1001 /APPLICATION /NODE=S01 -

/PORT=PORT_1

The server port must be configured as a REMOTE port with XON/XOFF

flow control  enabled and with  the proper baud  rate. The server

name and the port name must be set at the server. LATSYM will not

access the port correctly based on the server name in the Network

Control  Program  (NCP)  database.  This  can  be an annoying and

insidious source of problems in mapping any remote port. The node

and port  names used here must  match the node (server)  and port

names at the server exactly.

Once the port has been  defined, the system manager should decide

if  device protection  levels should  allow access  by non-system

users. To  allow use by  all users use  the set protection/device

command:

$ SET PROTECTION = W:RWLP /DEVICE

USING THE MENU

To start the RP software enter the following VMS command:

$ @RP_MENU

This executes the DCL command procedure RP_MENU.COM which in turn

executes the RP_MENU.EXE image. The  RP_MENU system is a flexible

and friendly menu  driver for the RP software.  From the menu the

user can choose to change the  input or output file name, to read

or punch cards, or to edit the file on disk.

SETTING DEFAULT VALUES

The  first  menu  option  allows   the  user  to  change  logical

definitions "on the fly." This option can also be used to set the

initial values  for the input file,  reader/punch port and reader

timeout  if these  values are  not set  system-wide (setting them

here  will   override  system  table   values  for  the   logical

definitions).

SETTING THE INPUT (DISK) FILE NAME

This allows the simple redefinition  of only the input file name,

without  having  to  reset  the  other  default values. Enter the

entire VMS file specification at the prompt.

READING CARDS

RP_CARDREADER.COM  is a  procedure for  reading ASCII  input from

serial interfaced cardreaders connected to  a VAX. It is designed

to  work  in  network  environments  using  terminal servers, but

should work as well with other communications controllers such as

the DMZ-32.

Certain VMS privileges are  required to execute RP_CARDREADER.COM

as  it  is  currently  written.  LOG_IO  is  required  to set the

terminal characteristics of the input port. SYSPRV is required to

open  the input  device as  a "file."  Alternatively, the  system

manager could create the device and allow access to it by setting

the  appropriate protection  level. The  terminal characteristics

could  be  preset  and  the  SET  TERMINAL  command  removed from

RP_CARDREADER.COM,  thus  allowing  RP_CARDREADER.COM  to execute

without any special privileges.

Logical definitions are used to simplify using RP_CARDREADER.COM.

If RP_MENU is not used, the definition commands could be added to

RP_CARDREADER.COM  or placed  in a  DCL command  file intended to

execute both the definitions  and RP_CARDREADER.COM. The commands

are as follows:

$ DEFINE RP_READER_PUNCH_PORT portname:

$ DEFINE RP_READER_PUNCH_FILE file_specification

$ DEFINE RP_READER_PUNCH_TIMEOUT seconds

Portname is  the device specification  for the port  to which the

cardreader  is  connected,  e.g.,  LTA1111:  or  TXA0:. The colon

should be included in the  definition. There is no default value;

this  logical must  be defined  or an  invalid device  error will

result.

File_specification  is the  full name  of the  output disk  file,

e.g., CARDS.DAT. The default name is RP_READER_PUNCH_FILE.DAT.

Seconds is the number of seconds  each attempt to read input will

wait  before giving  up. This   should be  set long  enough allow

starting the  cardreader, correcting pick and  stack checks, etc.

The default value is 10.

Complete examples are as follows:

$ DEFINE RP_READER_PUNCH_PORT LTA1111:

$ DEFINE RP_READER_PUNCH_FILE D18:[DATA]CARDS.DAT

$ DEFINE RP_READER_PUNCH_TIMEOUT 15

RP_CARDREADER.COM will  read cards (or any  other input, for that

matter) as  complete records of  ANY length. For  this reason the

end of each  "record" must be indicated with  a carriage return -

line  feed sequence.  The records  will be  stored on  the VAX as

variable length RMS records.

RP_CARDREADER.COM  will continue  to  read  cards until  either a

timeout period is exceeded or an end of file indicator is read. A

control-Z  will  indicate  the  end  of  file.  In  either  case,

RP_CARDREADER.COM will close the output file automatically.

PUNCHING CARDS

The option  to punch cards  in RP_MENU executes  RP_CARDPUNCH.COM

which  reads   input  records  from  the   file  defined  by  the

RP_READER_PUNCH_FILE  logical.  RP_CARDPUNCH   then  outputs  the

record to the reader/punch port (defined by RP_READER_PUNCH_PORT)

with a  trailing carriage return and  line feed. RP_CARDPUNCH.COM

can be  easily modified to  alter the record  or provide a  delay

between punching cards if necessary.

If the port device and input file protection levels allow access,

no special privileges will be required to execute this option. If

RP_MENU is not used, the logical definitions required are similar

to those for RP_CARDREADER.COM except that the timeout definition

is not required. The commands to define these logicals are

$ DEFINE RP_READER_PUNCH_PORT portname:

$ DEFINE RP_READER_PUNCH_FILE file_specification

EDITING THE INPUT (DISK) FILE

This  option allows  editing the  input file  without leaving the

RP_MENU environment.  The default editor  specified is TPU  using

the  EDT interface.  The RP_READER_PUNCH_FILE  logical should  be

defined before choosing this option.