02-MAY-1989

This directory contains the source, object, and executable for SDFILTER, a

utility that allows system managers to more readily control the queueing

and format of VAXstation (VWS) screen dumps.

SDFILTER runs under VMS V5.0 or later with VWS V4.0.  It can be re-linked

to run under VMS V4.7 with VWS V3.2 or V3.3.  Further documentation follows.

================================================================================

Introduction

The VMS Workstation Software "print (porition of) workstation

screen" feature was developed mainly to support printers attached

locally to a workstation.

If you run a publically-accessible Local-Area VAXcluster of

workstations, the feature is difficult to control, since users can

screw up the settings. In addition, all screen dumps are

printed under username SYSTEM, making accounting impossible.

SDFilter (Screen Dump Filter) is a program designed to ease the

centralized printing of VWS screen dumps.  The program runs as a

detached process on each workstation, filtering screen dump

requests.  Through the use of logical names, SDFilter directs screen

dumps to an appropriate print queue, provides appropriate page

selection and SIXEL setup parameters, and prints a one-line header

identifying the plot origin (username, node name, and site name).

SDFilter will only allow screen dumps to be printed if a user is

logged into the workstation, and that user is charged for the print

job.

Installing SDFilter

SDFilter needs to be started up each time the workstation reboots.

The following commands should be added to the workstation's system

startup sequence:

$ DEFINE/SYSTEM SDF_QUEUE queue_name

$ DEFINE/SYSTEM SDF_HEADER header_info

$ DEFINE/SYSTEM SDF_ORIENTATION page_select_info

$ DEFINE/SYSTEM SDF_SIXEL_PARAMETERS sixel_setup_info

$ RUN/DETACHED/OUTPUT=NL:/PROCESS="SDFilter"-

/PRIV=(NOSAME,TMPMBX,NETMBX,WORLD,SYSNAM,SYSPRV,CMKRNL,PRMMBX)-

device:[directory]SDFILTER

Each of the logical names are explained in detail below.

SDF_QUEUE selects the print queue to which screen dumps should be

sent.  If not defined, it defaults to SYS$PRINT.  The print queue

should be for some Sixel-capable device.  SDFilter has been tested

with the LN03 Plus laser printer.

The queue should be set up so that a form feed occurs between jobs.

One way to do this is to use the /SEPARATE=(RESET:xx) qualifier on

the INITIALIZE/QUEUE command, where the reset sequence contains a

form feed.

On systems for which you would like to dedicate a printer only to

screen dumps, the protection on the print queue can be set up so

that WORLD has no access, since SDFilter gains access via SYSTEM

protection.

SDF_HEADER specifies the appearance of the header line that appears

at the top of each screen dump.  The form of the header information

is "jn,site", where the "j" is a letter defining the justification

to be used (L for left, R for right, C for center, or B for Both),

the "n" is a number indicating what the length of the header line

should be (usually 80 or 132), and "site" is your site name.  If

undefined, the header information defaults to "L80,Screen Output"

(left justified, width 80, "Screen Output" as site name).

Note: make sure that you have set the printer's width to some large

number with SET TERMINAL/WIDTH if you are going to print in

landscape mode, or your headers may take up more than one line.

SDF_ORIENTATION specifies the page format for the screen dump.  It

should be either "PORTRAIT" or "LANDSCAPE" or an escape sequence

that selects the page format.  Using "PORTRAIT" (which is the

default) selects DEC LN03 private tall North American format.  Using

"LANDSCAPE" selects DEC LN03 private wide North American format.

If you are going to use an escape sequence, make sure the escape

sequence includes a PFS (page format select) sequence, a font

selection sequence, and a page height sequence.

SDF_SIXEL_PARAMETERS specifies the sixel graphics setup control

sequences used for each screen dump.  They should consist of a

comma-separated list of assignment statements for sixel control

parameters as used in the following control sequences:

Set Size Unit: CSI <SSU> SP I

Enter Sixel Mode: DCS <PS1> ; <PS2> ; <PN3> q

Raster Attributes (DECGRA): " <PN4> ; <PN5> ; <PN6> ; <PN7>

SSU	      size unit selector (default is SSU=2)

PS1	      macro parameter (default is PS1=0)

PS2	      background color (default is PS2=0)

PN3	      horizontal grid size (default is PN3=2)

PN4	      vert/horiz aspect ratio numerator (default is PN4=200)

PN5	      vert/horiz aspect ratio denominator (default is PN5=100)

PN6	      height of image (default is PN6=6)

PN7	      width of image (default is PN7=200)

Not all printers use all of the parameters; see your printer's

programmer's reference for more information.

Only those parameters listed in the SDF_SIXEL_PARAMETERS string are

affected; all others take the defaults listed above.  An example for

use on an LN03 Plus in landscape mode is:

$ DEFINE/SYSTEM SDF_SIXEL_PARAMETERS "PS1=3,PN3=3"

These parameters allow a full screen dump to fit on an 8 1/2 by 11

inch page in landscape mode.

Other Setup Information

When SDFilter starts up, it creates a mailbox with a logical name of

SDF_MBOX.  You should set the workstation print destination to

"SDF_MBOX:" and have print requests handled immediately in order to

have SDFilter work properly.  You should probably disable changes to

print settings (as described in section 2.6 of the workstation

software User's Guide) to make sure that screen dumps always go

through SDFilter.  Other features you may want to disable are "Use

default settings" and "Save current settings" -- once you have saved

the settings with these print setup changes.

Changing SDF Settings

SDFilter reads the SDF logical names for each screen dump, so you

can change them "on the fly" without having to restart the SDFilter

process.  This is helpful when you want to experiment the Sixel

parameter settings right for your printer.

--------------------------------------------------------------------------------

CONTACTING THE AUTHOR

Comments, suggestions, and questions about this software can be directed to

the author at one of the following addresses:

Mail:	Matthew Madison

Engineering Computing Services

Rensselaer Polytechnic Institute

Troy, New York 12180-3590

Phone:	(518) 276-2606

E-Mail:	(Internet)  madison@vms.ecs.rpi.edu

(BITNET)    MADISON@RPIECS

COPYRIGHT NOTICE

This software is COPYRIGHT  1988, 1989 RENSSELAER POLYTECHNIC INSTITUTE.

ALL RIGHTS RESERVED.  Permission is granted for non-profit redistribution and

use.

DISCLAIMER

This software is provided "AS IS".  The author and Rensselaer Polytechnic

Institute disclaim all warranties on the software, including without limitation,

all implied warranties of merchantability and fitness.