CONTENTS

CHAPTER 1       INTRODUCTION

CHAPTER 2       INTERACTIVE ETHERNET MONITOR

2.1     MONITOR MODES  . . . . . . . . . . . . . . . . . . 2-1

2.1.1     Ethernet Protocol Mode . . . . . . . . . . . . . 2-1

2.1.2     Ethernet Map Mode  . . . . . . . . . . . . . . . 2-1

2.1.3     Ethernet Trace Mode  . . . . . . . . . . . . . . 2-2

2.2     INTERACTIVE ETHERNET MONITOR COMMANDS  . . . . . . 2-2

CHAPTER 3       ETHERNET MAINTENANCE MODULE

3.1     OVERVIEW . . . . . . . . . . . . . . . . . . . . . 3-1

CHAPTER 4       ETHERNET LISTENER AND REPORTER MODULE

4.1     OVERVIEW . . . . . . . . . . . . . . . . . . . . . 4-1

APPENDIX A      INSTALLING ETHERNET MONITOR

APPENDIX B      FILE FORMATS

APPENDIX C      CAVEAT EMPTOR

APPENDIX D      STATUS LINES

1

CHAPTER 1

INTRODUCTION

Emonitor is a collection of ethernet  monitor  programs  used  to

identify  and  monitor ethernet devices on an ethernet network.  It is

composed of the following modules:

*  Ethernet Monitor

The  interactive   ethernet   monitoring   program   provides

statistics  by protocol, displays device address and optional

node names (when  entered  via  the  maintenance  module)  of

devices sensed on the network, and traces packets to and from

a selected device.  Also able to  dump  ethernet  packets  to

EMON_DIR:ETHERNET_DUMP.DAT.

*  Ethernet Listener

Collects   statistics   to   EMON_DIR:ETHERNET_COUNTS.DAT for

processing  by  report  module.   Packet counts are dumped at

selected interval.

*  Ethernet Reporter

Produces reports from EMON_DIR:ETHERNET_COUNTS.DAT.

*  Ethernet Maintenance

Ability to list and modify contents of monitor  protocol  and

device data files.

1-1

CHAPTER 2

INTERACTIVE ETHERNET MONITOR

2.1  MONITOR MODES

The interactive ethernet monitor provides a means  of  monitoring

ethernet  traffic  on  the fly.  This program has three screen modes -

Protocol, Map, and Trace.

2.1.1  Ethernet Protocol Mode

When first invoked via  the  EMONITOR  command,  the  process  is

displaying  ethernet  statistics  by  protocol.  This mode can also be

selected by typing a "P" when in any  of  the  other  modes.   As  new

protocols  are detected by the program, they are added to the protocol

data file (EMON_DIR:ETERNET$PROTOCOLS.DAT) and displayed at the bottom

of the current screen list.  When they are added they are given a name

of "Unknown Protocol", you may change this by invoking the maintenance

module  by  typing "EMON/MAINTENANCE" at the DCL command prompt.  Each

time the program is entered, these protocols are sorted into ascending

protocol id order.  If you wish to monitor a specific protocol, simply

select the desired protocol by using the up and down arrow keys.  This

will  cause  the  packet  grabber  routine to discard all packets with

protocols different than that selected.  You will also note  that  the

counters will be reset.

2.1.2  Ethernet Map Mode

The map screen is selected by typing "M" when in the  interactive

monitor.   This  will  draw an ethernet map (schematic representation)

with the various detected ethernet devices listed.  As new devices are

sensed,  they  will  be  added  to  the  device  data  file  found  in

EMON_DIR:ETERNET$DEVICES.DAT and displayed at the bottom of the current

screen  list.   When  devices are added automagically they are given a

name equal to the ethernet address.  You should/could change  this  to

2-1

INTERACTIVE ETHERNET MONITOR

the  node  name  by  invoking  the  maintenance module.  Each time the

program is entered known devices are  displayed  in  ascending  device

name  order.   If  you  wish  to monitor a specific device, select the

desired device by using the up and down arrow keys.  This  will  cause

the  packet  grabber  routine  to  discard  all packets with source or

target addresses not equal to the selected device.  Counters will also

be  reset.   You can have a selected device and a selected protocol at

the same time.

2.1.3  Ethernet Trace Mode

The trace mode is selected by typing "T".  The  trace  screen  is

useful  for  determining  traffic flow by node address.  If there is a

selected device, the trace mode will only track packets  sent  between

the selected device and communicating nodes.

2.2  INTERACTIVE ETHERNET MONITOR COMMANDS

Ethernet packets can be viewed for analysis by typeing  "D"  when

in  any  of  the  monitor  screen  modes.   This toggles the option of

dumping ethernet packets to EMON_DIR:ETHERNET_DUMP.DAT on and off. This

file is a collection of raw ethernet packets each of which is preceded

by a time stamp record.  For analyzing formatted packets use  the  VMS

dump utility on this file, i.e.:

DUMP/RECORD EMON_DIR:ETHERNET_DUMP.DAT

2-2

CHAPTER 3

ETHERNET MAINTENANCE MODULE

3.1  OVERVIEW

The Ethernet maintenance module  is  used  to  modify  or  delete

records  contained  in  the protocol and device files.  This module is

invoked by typing "EMONITOR/MAINTENANCE" at the dcl prompt.

3-1

CHAPTER 4

ETHERNET LISTENER AND REPORTER MODULE

4.1  OVERVIEW

The Ethernet Listener is used to collect summary statistics based

on protocol type, source address, and/or target address.  The listener

is invoked by typing "EMON/RECORD" at the dcl prompt.  Output  can  be

directed  to  a  file  of  your  choice by using the /OUTPUT qualifier

(default is EMON_DIR:ETHERNET_COUNTS.DAT).  The default interval of  5

minutes  can  be  modified  by supplying a delta time to the /INTERVAL

qualifier.  Once the data  file  has  been  created,  reports  can  be

generated by typing EMON/REPORT at the dcl prompt.

4-1

APPENDIX A

INSTALLING ETHERNET MONITOR

The ethernet monitor needs to reside in a directory pointed to by

the logical EMON_DIR. The program must be run from an account with the

PHYIO privilege and write access to the EMON_DIR directory.   Copy the

executable files into this directory along with the cld file.  Execute

the following commands and you're off!

$ define emon_dir   disk1:[systems.source.emon]

$ set command emon_dir:ethernet_monitor

As the programs  execute,  the  device  and  protocol  files  are

updated as new devices and protocols are sensed.

A-1

APPENDIX B

FILE FORMATS

ETHERNET_PROTOCOLS.DAT

Field           Type                    Remarks

protocol_type       2 char                  key 0

protocol_name       25 char                 key 1

ETHERNET_DEVICES.DAT

Field           Type                    Remarks

ethernet_address    6 char                  key 0

node_name           12 char                 key 1

node_type           byte                    0 = terminal_server

1 = host

2 = unknown

remarks             80 char                 Currently not used

B-1

FILE FORMATS

ETHERNET_COUNTS.DAT

Time Stamp record:

Field           Type                    Remarks

record_type         byte = 0

time_stamp          quadword system time

active              longword integer        Active counters for current interval

counters            longword integer        Total counters for session

Counter Record:

Field           Type                    Remarks

record_type         byte = 1

destination         6 char

source              6 char

protocol            2 char

packet_count        integer

byte_count          integer

B-2

APPENDIX C

CAVEAT EMPTOR

In order to monitor ethernet  traffic,  the  ethernet  controller

must  be  kicked  into promiscuous mode.  This makes the computer work

much harder than would ordinarily  be  necessary  since  it  must  now

handle   every   packet   going  down  the  line  instead  of  packets

specifically targeted to that controller.  Ethernet  is  a  relatively

fast  channel,  and  a  busy  ethernet  can  keep the cpu busy.  IT IS

IMPORTANT TO UNDERSTAND THE RESOURCE REQUIREMENTS OF  THIS  SYSTEM  IN

ORDER  TO  GUARANTEE  MEANINGFUL  STATISTICS.   Ethernet  packets  are

collected by an asynchronous  routine  and  stuffed  into  a  circular

buffer  list.   To  prevent  this  list  from overflowing, the list is

trimmed if the current buffer count exceeds a threshold (reflected  by

discarded  packets  on  the  status  line).   Keep an eye on discarded

buffers!  Most likely, your cpu is not able to handle traffic, process

priority is insufficient (in which case you may not even catch all the

packets) or packet dumping to disk is too costly.  To prevent the  ast

routine  effectively  locking out the program, sampling is disabled if

the discarded packets exceed a threshold.

This software works best with a VT200 or VT300.

C-1

APPENDIX D

STATUS LINES

Ethernet Counters Status Line

BRC     Bytes Received

DBR     Packets Received

MBY     Mulitcast Bytes Received

MBL     Multicast Packets Received

RFL     Packets in Error

OVR     Lost - Internal Buffer Errors

LBE     Lost - Local Buffer Errors

BSN     Bytes Transmitted

DBS     Packets Transmitted

MSN     Multicast Bytes Transmitted

MBS     Multicast Packets Transmitted

BID     Packets Deferred

SFL     Packets Aborted

BSM     Packets with Several Errors

BS1     Packets with Single Error

CDC     Collision Check Errors

UFD     Unrecognized Frame Destination

D-1