Accessing RFAs From VAX COBOL

This file features code previously published in the February 1989 issue of

VAX Professional magazine. Originally run as part of Robert W. Dilworth,

Jr.'s article, "Accessing RFAs From VAX COBOL," which appeared on page 39,

the code here consists of four example programs for setting up and using

MACRO interface subroutines.

************************************************************************

*  Use these procedures at your own risk. Professional Press is not    *

*  liable for any damage that these utilities might do to your system. *

************************************************************************

*

*  Before running this example program, it is necessary to create a file

*  that corresponds to the program's FD.  Accordingly, use an editor such as

*  EVE or EDT and create a file called "TEST.FILE".  Type 10 records into the

*  file making sure that the longest does not exceed 28 characters.

*

IDENTIFICATION DIVISION.

PROGRAM-ID. RFA-TEST.

ENVIRONMENT DIVISION.

INPUT-OUTPUT SECTION.

FILE-CONTROL.

SELECT  THE-FILE ASSIGN TO "TEST.FILE"

ACCESS IS SEQUENTIAL.

DATA DIVISION.

FILE SECTION.

FD  THE-FILE

RECORD VARYING DEPENDING ON READ-LEN.

01  THE-REC                               PIC X(28).

WORKING-STORAGE SECTION.

01  NUMERIC-VARIABLES.

03  RETURN-VAL                        PIC 9(9) COMP.

03  ITS-LENGTH                        PIC 9(9) COMP.

03  READ-LEN                          PIC 99.

03  FILE-RAB                          PIC 9(9) COMP.

03  INDX                              PIC 9(4) COMP.

01  MISC-ALPHA-VARIABLES.

03  EOF-FLAG                          PIC X.

03  Y-OR-N                            PIC X.

01  RAB-ARRAY-AREA.

03  RAB-ARRAY    OCCURS 10 TIMES.

05  FILE-REC                      PIC X(28).

05  RFA-LW                         PIC 9(9) COMP.

05  RFA-W                          PIC 9(4) COMP.

01  RAC-VALUES.

03  RFA-ACCESS                        PIC 9(9) COMP VALUE EXTERNAL

SET$RAC_RFA.

03  SEQ-ACCESS                        PIC 9(9) COMP   VALUE EXTERNAL

SET$RAC_SEQ.

03  KEY-ACCESS                        PIC 9(9) COMP VALUE EXTERNAL

SET$RAC_KEY.

PROCEDURE DIVISION.

* * * * * * *

MAIN_PROGRAM.

* * * * * * * * * * * * * * * * * * * * * * * * * * * * * * * * * * * * * * * *

OPEN I-O THE-FILE.

CALL "LIB$ANALYZE_SDESC" USING BY DESCRIPTOR RMS-STS

BY REFERENCE ITS-LENGTH

BY REFERENCE FILE-RAB

GIVING RETURN-VAL.

SUBTRACT 8 FROM FILE-RAB.

MOVE " " TO EOF-FLAG.

PERFORM FILL-ARRAY VARYING INDX FROM 1 BY 1 UNTIL EOF-FLAG = "Y".

MOVE " " TO EOF-FLAG.

CALL "SET_RAC" USING BY VALUE FILE-RAB,

BY VALUE RFA-ACCESS,

GIVING RETURN-VAL.

PERFORM USE-RFA UNTIL EOF-FLAG = "Y" OR (Y-OR-N = "N" OR "n").

CLOSE THE-FILE.

STOP RUN.

* * * * * * * * * * * * * * * * * * * * * * * * * * * * * * * * * * * * * * * *

FILL-ARRAY.

* * * * * * * * * * * * * * * * * * * * * * * * * * * * * * * * * * * * * * * *

READ THE-FILE INTO FILE-REC(INDX)

AT END MOVE "Y" TO EOF-FLAG.

IF EOF-FLAG NOT = "Y"

CALL "GET_RFA" USING BY VALUE FILE-RAB

BY REFERENCE RFA_LW(INDX)

BY REFERENCE RFA_W(INDX)

GIVING RETURN-VAL

END-IF.

* * * * * * * * * * * * * * * * * * * * * * * * * * * * * * * * * * * * * * * *

USE-RFA.

* * * * * * * * * * * * * * * * * * * * * * * * * * * * * * * * * * * * * * * *

DISPLAY "ENTER A RECORD NUMBER: ".

ACCEPT INDX WITH CONVERSION.

CALL "PUT_RFA" USING BY VALUE FILE-RAB,

BY REFERENCE RFA-LW(INDX),

BY REFERENCE RFA-W(INDX)

GIVING RETURN-VAL.

MOVE SPACES TO THE-REC.

READ THE-FILE INTO FILE-REC(INDX)

AT END MOVE "Y"  TO EOF-FLAG.

IF EOF-FLAG NOT = "Y"

DISPLAY FILE-REC(INDX) BOLD

MOVE " " TO Y-OR-N

DISPLAY "RE-WRITE RECORD?"

ACCEPT Y-OR-N

IF Y-OR-N = "Y" OR "y"

DISPLAY "ENTER A NEW RECORD OF ", READ-LEN, " BYTES:"

MOVE SPACES TO FILE-REC(INDX)

ACCEPT FILE-REC(INDX)

REWRITE THE-REC FROM FILE-REC(INDX)

END-IF

DISPLAY "MORE? (Y/N)"

ACCEPT Y-OR-N.

;

;                    RFA ACCESS MACRO ROUTINES

;

;

;  Define Global equates.

;

set$rac_rfa   ==     1      ; Defines access by RFA

set$rac_seq   ==     2      ; Defines sequential access

set$rac_key   ==     3      ; Defines key access

;

.PSECT GET_RFA_CODE  NOWRT, EXE

;

;  The next line of code defines the entry point for the "CALL GET_RFA"

;    line in the accompanying COBOL program

;

.entry        get_rfa,^M<r2,r3,r4,r5>

;

get_rfa_begin:

movl          4(ap),r5             ; rab addr to r5

movl          8(ap),r2      ; rfa_lw address to r2

movl          12(ap),r3     ; rfa_w address to r3

movl          r5,r4         ; rab base addr to r4

addl2         #rab$l_rfa0,r4  ; point to first 4 bytes of rfa

movl          (r4),(r2)       ; ship 1st part of rfa to rfa_lw

movl          r5,r4         ; rab base addr to r4

addl2         #rab$w_rfa4,r4  ; point to last 2 bytes of rfa

movw          (r4),(r3)     ; ship 2nd part of rfa to rfa_w

movl          #1,r0         ; set return val to success

ret                         ; bye!

;

.PSECT SET_RAC_CODE  NOWRT, EXE

;

;  The next line of code defines the entry point for the "CALL SET_RAC:

;    line in the accompanying COBOL program

;

.entry        set_rac, ^M<r2,r3>

;

set_rac_begin:

movl          4(ap),r3             ; rab address to r3

addl          #rab$b_rac,r3   ; point it to rac "slot" in rab

movl          8(ap),r2      ; rac-access to r3

cmpl          r2,#set$rac_rfa      ; rfa access?

blssu         error         ; if a 0

beqlu         rfa           ; goto rfa

cmpl          r2,#set$rac_seq      ; sequential access?

beqlu         seq             ; goto seq

cmpl          r2,#set$rac_key      ; key access?

beqlu         key           ; goto key

bgtru         error         ; anything greater than 3 = error

;

rfa:   movb          #rab$c_rfa,(r3) ; rfa access to appropriate rab slot

brb           out             ; bye!

seq:   movb          #rab$c_seq,(r3) ; sequential access to rab slot

brb           out

key:   movb          #rab$c_key,(r3) ; key access to appropriate rab slot

;

out:   movl          #1,r0

brb           bye

;

error: clrl          r0

bye:   ret

;

.PSECT PUT_RFA_CODE  NOWRT, EXE

;

;  The next line of code defines the entry point for the "CALL PUT_RFA"

;    line in the accompanying COBOL program

;

.entry        put_rfa,^M<r2,r3,r4,r5>

;

put_rfa_begin:

movl          4(ap),r5      ; rab addr to r5

movl          8(ap),r2      ; rfa_lw address to r2

movl          12(ap),r3     ; rfa_w address to r3

movl          r5,r4

addl2         #rab$l_rfa0,r4

movw          (r2),(r4)     ; 1st part of rfa to rfa_lw

movl          r5,r4

addl2         #rab$w_rfa4,r4

movw          (r3),(r4)     ; 2nd part of rfa to rfa_w

movl          #1,r0         ; set return val to success

ret                         ; bye!

.end