PC Exchange (for VMS)

PC Exchange (PCX) is a utility for copying files to and from

MS-DOS format diskettes under VMS, using an RX23 (3.5") or RX33

(5.25") diskette drive.  For 3.5" diskettes, high-density disks can

be read or written; double-density disks are read-only. Only high-density

disks are supported on the RX33.

SYSTEM REQUIREMENTS

-------------------

* VMS V5.0 or later (it may work under V4.x)

* An RX33, RX33S, RX23, or RX23S diskette drive ("S" denotes SCSI variant)

Please read the AAAREADME.TOO file for information on bug fixes,

known problems, and other version-related information.

INSTALLING PCX

--------------

To build the executable:

* If you have the object library, PCX.OLB, available, create the

executable with the command:

$ LINK/NOTRACE PCX.OLB/INCLUDE=PCX/LIBRARY

All required object files are in that library.

* If you have the MACRO sources, execute the ASMBUILD.COM command

procedure to assemble them and link the PCX executable.  You will

probably have to first edit the file ACCESS.MAR and remove some

extraneous semicolons from in between the continuation lines of

the big .LONG directive, close to the top of the file.

* If you have the BLISS sources and the MMS description file available,

use MMS to compile and link the executable.

Once you have the PCX.EXE file, you can set up a DCL foreign command

to run PCX:

$ pcx :== $disk:[directory]PCX

To build the help library:

$ RUNOFF PCX_HELP.RNH

$ LIBRARY/CREATE/HELP PCX_HELPLIB.HLB PCX_HELP.HLP

To make the help library available to PCX:

$ DEFINE PCX_HELPLIB disk:[directory]PCX_HELPLIB

If you install the help library in SYS$HELP, this step is unnecessary.

PRIVILEGES REQUIRED

-------------------

* You may need VOLPRO privilege to use PCX.  PCX can be INSTALLed

with this privilege:

$ INSTALL CREATE disk:[directory]PCX/PRIVILEGE=VOLPRO

PCX turns off all image privileges at startup, and only activates

them when needed.

BEFORE YOU START

----------------

Before using PCX, you must define the logical name PCX_DISK as

the device name of the diskette drive.  One easy way to do that,

while at the same time ensuring that no other process has access

to the drive, is to use the ALLOCATE command, as in:

$ ALLOCATE/GENERIC RX23 PCX_DISK

Using the /GENERIC qualifier also means you don't have to know

the physical device name of the drive (assuming you only have

one diskette drive available).

AVAILABLE COMMANDS

------------------

PCX provides the following commands:

@                     executes a command file (default filetype .PCX)

ATTACH                attaches to another process

CREATE/DIRECTORY      creates a subdirectory

DELETE                deletes a file or empty directory

DIRECTORY             displays a directory listing

EXIT                  leaves PCX

FORMAT                formats the disk

HELP                  displays help

READ                  reads a file from the floppy onto VMS

SET PATH              sets current default directory path on floppy

SHOW PATH             displays current default directory path

SPAWN                 spawns a subprocess

TYPE                  displays the contents of a file on the floppy

WRITE                 writes a VMS file on the floppy

The help information explains the parameters and/or qualifiers for each

command.

--------------------------------------------------------------------------------

CONTACTING THE AUTHOR

Comments, suggestions, and questions about this software can be directed to

the author at one of the following addresses:

Mail:    Matthew Madison

Engineering Computing Services

Rensselaer Polytechnic Institute

Troy, New York 12180-3590

Phone:  +1 518 276 2606

E-Mail: (Internet)  madison@vms.ecs.rpi.edu

(BITNET)    MADISON@RPIECSVX or MADISON@RPIECS

COPYRIGHT NOTICE

This software is COPYRIGHT  1989, 1990 RENSSELAER POLYTECHNIC INSTITUTE.

ALL RIGHTS RESERVED.  Permission is granted for non-profit redistribution and

use.

DISCLAIMER

This software is provided "AS IS".  The author and Rensselaer Polytechnic

Institute disclaim all warranties on the software, including without limitation,

all implied warranties of merchantability and fitness.  USE AT YOUR OWN RISK.