ITEM$C_QDEPTH_TABLE	= 1

ITEM$C_QDEPTH_RECALC	= 2

ITEM$C_DEBUG_DYNAMIC	= 3

ITEM$C_PORT_QDEPTH	= 4

+-----------+-----------+--------------+

|   Sense   |    Set    |  Initialize  |

----------------------------------+-----------+-----------+--------------+

Runtime QDepth Table             |           |           |              |

Item = ITEM$C_QDEPTH_TABLE = 1  |           |           |              |

Buffer                          | Required  | Required  | Prohibited   |

Buffer Size                     |   >= 128  |      128  |        MBZ   |

----------------------------------+-----------+-----------+--------------+

QDepth Recalculation Counter	  |           |           |              |

Item = ITEM$C_QDEPTH_RECALC = 2 |           |           |              |

Buffer                          | Required  | Required  | Prohibited   |

Buffer Size                     |     >= 4  |        4  |        MBZ   |

---------------------------------+-----------+-----------+--------------+

Dynamic Debug Longword           |           |           |              |

Item = ITEM$C_DEBUG_DYNAMIC = 3 |           |           |              |

Buffer                          | Required  | Required  | Prohibited   |

Buffer Size                     |     >= 4  |        4  |        MBZ   |

---------------------------------+-----------+-----------+--------------+

Max Port Queue Depth             |           |           |              |

Item = ITEM$C_PORT_QDEPTH = 4   |           |           |              |

Buffer                          | Required  | Required  | Prohibited   |

Buffer Size                     |     >= 4  |        4  |        MBZ   |

---------------------------------+-----------+-----------+--------------+

These are the changes to DKDRIVER which are necessary to add SET/SENSEMODE

support. Even if we don't want to check the functionality in, I think it

would be a good idea to make the code changes and just put a DK_SExMODE stub

in to allow us to easily rebuild it for debugging either by having our own

object library or just editing the driver and cutting in the source.

1) Modify DK_FUNCTABLE to map SETMODE and SENSEMODE to DK_SExMODE, leave

SET/SENSECHAR mapped to standard routines.

2) Add UCB$R_QDEPTH_TABLE, table of XLEN_HIST_BUCKETS longwords, to hold the

run-time qdepth table.

3) Add UCB$L_DEBUG_DYNAMIC, longword, a longword we can read and write as the

driver is running.

4) Add UCB$L_XLEN_HIST_COUNT, a LW used to re-initialize UCB$L_XLEN_HIST_CYCLE

when the queue depth is being recalculated.

5) Conditionally assemble either a stub routine returning SS$_ILLIOFUNC or the

SEXMODE.MAR source code. If we make all the other changes described here,

they would be transparent to the user and the only one we'd have to make

to enable SET/SENSEMODE support for debugging is this one.

6) Change static table QDEPTH_TABLE to DEFAULT_QDEPTH_TABLE, and all references

to the former to the latter.

7) Add "DPT_STORE UCB,UCB$L_DEBUG_DYNAMIC,L,DEBUG_DYNAMIC"

8) Add static longword DEFAULT_DEBUG_DYNAMIC:: .LONG DEFAULT_DEBUG

9) Add static longword DEFAULT_XLEN_HIST_COUNT:: .LONG XLEN_HIST_TURNOVER

10) In unit init, load UCB$L_XLEN_HIST_COUNT from DEFAULT_XLEN_HIST_COUNT

11) In unit init, load UCB$L_XLEN_HIST_CYCLE from UCB$L_XLEN_HIST_COUNT

instead of from XLEN_HIST_TURNOVER

12) In unit init, load UCB$R_QDEPTH_TABLE from DEFAULT_QDEPTH_TABLE

13) In IO_RW_COMMON, use UCB$L_XLEN_HIST_COUNT to reload

UCB$L_XLEN_HIST_CYCLE instead of XLEN_HIST_TURNOVER

14) Unconditionally assemble the UCB ring buffer fields, including a new

one, UCB$L_RING_SPINS, so they are always available if we want to

allocate memory for ring buffers for a specific unit on the fly.

15) Remove SENSEMODE from the list of buffered I/O functions in FDT_BUF

16) Add the item list codes shown above to SCSILIB or some other library.

17) Add a constant, PORT_QDEPTH, defining the default value of the port

maximum queue depth.

18) Add one of the following to the DPT_STOREs:

a) DPT_STORE UCB,UCB$L_QDEPTH,L,PORT_QDEPTH

b) ASSUME PORT_QDEPTH EQ 16                ; Check assumption on next line

DPT_STORE UCB,UCB$L_QDEPTH,W,16         ; Default qdepth

19) Add a static longword, DEFAULT_PORT_QDEPTH, to act as a source for

reinitializing the maximum port queue depth to it's initial value.

20) Define a constant DEBUG_DYNAMIC ^X<FFFF@16> - this gives us a longword in

which half the bits are enabled by default and half are disabled, useful

depending on what we want a bit to mean.

21) Modify CHECK_QDEPTH to recognize a user-specified queue depth from the

high bit of UCB$L_QDEPTH, and skip establishing a new queue depth from

the table if it's set, just use whatever's in UCB$L_QDEPTH.

Notes:

1) Setting the maximum port queue depth always disables (sort of) dynamic

queue depth recalculation; actually all it does is set the cycle to -1

so it'll never turn over in the system's uptime. When you reset it to

the default value it reenables the dynamic depth recalculation counter

at it's default value. Note that you can still change the dynamic depth

recalculation counter (recalc) independently, but if you set it low

after setting the maximum port queue depth you'll lose the maximum

port queue depth you set the next time the table turns over.

2) To avoid having to code an SIO for SExMODE I took advantage of the

fact that I could have the mainline DKDRIVER code set the qdepth the

next time a read or write was performed; I set the new value in

UCB$L_QDEPTH and then just force recalculation by setting

UCB$L_XLEN_HIST_CYCLE to 1; but this does mean that the new port

queue depth doesn't take effect until the next time a read or write

is _started_.