VME KERMIT User Guide

=====================

This  document  describes  how to use the VME implementation of KERMIT

produced at the South West Universities Regional Computer Centre.

The information in this edition applies to version 1.01 of VME KERMIT.

EDITION 1.01

October 1986

ACKNOWLEDGEMENT

The layout and content of this document is derived from the  'USER GUIDE

FOR BBC KERMIT'  written by Alan Phillips of the Computing Department at

the University of Lancaster. We are grateful for his permission  to  use

this material.

VME KERMIT User Guide

CONTENTS

1.  INTRODUCTION

2.  AN OVERVIEW OF KERMIT

3.  CONTROLLING VME KERMIT

3.1  Entering VME KERMIT

3.1.1 The OPTION parameter

3.1.2 The VME_FILE parameter

3.1.3 The REM_FILE parameter

3.2  Leaving VME KERMIT

3.3  VME KERMIT command language

3.1.1  Command format

3.3.2  Abbreviating commands

3.3.3  Numeric parameters

3.3.4  The help facility

4.  TRANSFERRING FILES WITH KERMIT

4.1  Principles

4.2  Setting file type

4.2.1  How files are organised

4.2.2  Sending eight-bit data

4.2.3  Binary files

4.2.4  ASCII files

4.2.5  EBCDIC files

4.3  File transfer as a server

4.4  File transfer as a non-server

4.4.1  Sending files as a non-server

4.4.2  Receiving files as a non-server

4.5  Handling problems

4.6  Advanced facilities

4.6.1  Incomplete file transfers

4.6.2  Using timeouts

4.6.3  Protocol control

5.  COMMANDS FOR GENERAL CONTROL OF KERMIT

6.  COMMANDS FOR TRANSFERRING DATA

7.  COMMANDS FOR PROTOCOL CONTROL

8. OBTAINING, BUILDING AND MODIFYING VME KERMIT

8.1 Obtaining VME KERMIT

8.1.1 The source files

8.2 Building VME KERMIT from source

8.3 Changing KERMIT defaults

Appendix 1. VME KERMIT capabilities

Appendix 2. Valid VME control characters

Appendix 3. Running KERMIT over a JNT PAD

VME KERMIT User Guide

1: INTRODUCTION

===============

This user guide describes the VME implementation of KERMIT produced

by  the  South  West  Universities  Regional  Computer  Centre at Bath

University. It is intended to provide enough information for a  novice

KERMIT  user  to  be  able to transfer data to and from VME to another

KERMIT system. Other KERMIT systems are desribed only in passing: thus

a user would almost certainly need  to  consult  the  equivalent  user

guide for the KERMIT system on the other machine.

The guide is divided into several chapters. The first chapter is  a

general  overview of KERMIT as a whole, and explains its advantages as

a file transfer system over "dumb capture" pograms. The  next  chapter

describes  the  command  language that VME KERMIT uses. Following that

are chapters that describe how to use VME KERMIT to transfer data.

The  final chapters comprise the "reference section". They describe

in full detail the commands available in VME KERMIT, grouping them  by

functionality (i.e. "Commands for file transfer", etc).

The very last chapter of the guide  describes  how  to  obtain  VME

KERMIT,  and, having obtained it, how to build it from the S3 language

source.

VME KERMIT is, of course, freely available to anyone who wants  it.

It  can  be  obtained  from Lancaster University's KERMIT distribution

service over file transfer from their VAX 11/780 system. The Lancaster

distribution  service  also  maintains  on-line  bulletin files giving

details  new releases of VME KERMIT and of reported bugs: these can be

consulted in a public-access username.

1-1

VME KERMIT User Guide

2: AN OVERVIEW OF KERMIT

========================

KERMIT is a system, devised at the Center for Computing  Activities

at  the University of Columbia, New York (CUCCA), to permit the simple

and flexible transfer of data from a microcomputer to a  mainframe  or

another  microcomputer. CUCCA retain the copyright on KERMIT, but have

published full information on it and permit anyone to implement it  on

their own machines, provided this is not done for commercial purposes.

The  result  is  that  KERMIT is now available on a very wide range of

machines indeed: very few micros and mainframes  now  do  not  have  a

KERMIT of some sort available for them.

The  primary  design aim of KERMIT is to permit the transfer of any

data whatsoever between systems, and to make the data  usable  on  the

system that receives it if this is possible. To illustrate why this is

important,  and  not  possible with simple systems, we can consider an

ordinary terminal emulation system that allows  data  to  be  captured

into files or sent from them.

Simple  terminal  emulator  systems,  such  as  those  commercially

available  for  many  micros,  do  permit you to transfer files from a

mainframe in a rudimentary way. You would tell the  emulator  to  copy

any  characters  that  appear  on the screen into a file, then ask the

mainframe to display the file. The reverse process would let you input

data into a mainframe file from your micro's discs.

The  problems arise in the nature of the communications system that

connect the micro to the mainframe, and how the mainframe itself  uses

this  system.  A  character of data in a file occupies one byte, which

consists of 8 binary digits or "bits". If you regard  the  pattern  of

bits representing a character as a number, this allows numbers ranging

from  0  to  255 to be used. However, many communications systems will

allow only 7 of the eight bits to be transmitted along them. The  most

significant   bit,   termed   the   "parity   bit",  is  used  by  the

communications system as an error-checking device. Thus,  even  though

you  send  a byte of 8 bits to the mainframe, it may receive only 7 of

them. This immediately restricts the range of characters that  can  be

sent to those whose codes are in the range 0 to 127.

A  further  restriction may be imposed if the communications system

uses some of those characters  for  its  own  control  purposes:  thus

systems  often  will  use  the characters whose codes are 17 and 19 to

prevent overloads occurring. In  such  systems,  you  cannot  transmit

these  characters  at  all.  To make matters even worse, some machines

will (apparently arbitrarily) decide that you could not possibly  want

to  send some characters, so, if you do send them, it will change them

into something else entirely.

The  character  range 0 to 127 covers all the printable characters,

so that transferring text files should  just  about  be  possible.  Of

course,  if  the  communications  line  you are using is unreliable or

noisy (a slow dial up line, for instance) there is nothing to  prevent

data  being  garbled  in  transmission, so that you will never be sure

sure that the data that arrives is the data that you sent.

2-1

VME KERMIT User Guide

KERMIT overcomes all these difficulties by  encoding  the  data  it

sends  according  to  a  standard  set  of rules or "protocol". KERMIT

recognises  that  many  characters  cannot  be  transmitted   down   a

communications  line,  so  if  those  characters  occur,  they will be

translated into something that will be transmitted. The receiving end,

of course, will translate them back again  to  what  they  were.  This

technique  enables  you  to  send  any  data at all, even machine code

programs. It further guarantees that the data you  send  is  the  data

that  arrives,  since  KERMIT  uses  special  methods  for   detecting

garbling  and  will  repeat any transmissions that did not get through

correctly.  KERMIT's  encoding  and  checking  techniques   are   more

efficient than some other systems that offer this facility, since only

bytes that need encoding actually are encoded, thus keeping the volume

of data sent to the minimum possible.

Besides  the problems of actually transferring data corectly, there

is  the  problem  of  making  it  usable  on  the  other  end  of  the

transmission link. If you are sending, say, a SAVEd BASIC program from

a micro to VME, this isn't a problem, since VME can't understand micro

BASIC anyway. Nor does it matter if you use the VME system only as  an

archive:  it's irrelevant how the data is held on VME, as long as when

it is brought back to the micro side it looks the same as when it  was

sent.

The  usability  problem  does appear, though, if you want to move a

file from a micro to VME and then actually use it on VME.  You  might,

for  instance, word process a file on the micro then send it to VME to

be printed. In this case,  you  do  NOT  want  to  transfer  the  data

byte-for-byte,  since  the  way  the  micro and VME denote things like

the end of each line of text will almost certainly be different.  What

you require is that the file of printable lines on the micro which you

can process on that machine, becomes a file of  printable  records  on

VME, that can be processed there.

Using  a  dumb terminal emulator system would probably let you send

the data, but it would appear byte-for-byte as it was  on  the  micro.

And probably you would get a  file  on  VME  with extra line-feeds and

carriage-returns that would need laborious editing  before  you  could

use the file sensibly.

With  KERMIT  the  problem  can  easily be circumvented. The KERMIT

protocols define a standard way of indicating the end of  a  printable

line.  When  you  send  a  file  from  VME,  VME KERMIT will translate

whatever ends the lines of text in your file into this  standard  form

before  sending  the  data.  The  receiving  end, seeing this standard

end-of-line indicator, will translate it  into  however  IT  indicates

end-of-line.  You  thus  end  up  with a usable file of lines, with no

extra characters anywhere.

2-2

VME KERMIT User Guide

The requirements you must meet before using KERMIT are simple.  You

will  need a KERMIT in your micro which supports terminal emulation; a

KERMIT program in the VME system you wish to transfer data to;  and  a

way  of  linking  the machines, be it a network, an ordinary cable, or

whatever.

Micro to mainframe transfers involves entering the micro's terminal

emulator  program:  you  initially use this to log in to VME as though

the micro was an ordinary terminal. Once logged in, you start the  VME

KERMIT program, and can then flip from giving commands to VME  KERMIT,

to  giving  commands to the micro KERMIT. You tell one machine that it

is going to receive a file, tell the other to send it,  and  sit  back

and relax while it happens.

Some micro KERMIT's provide one further facility to help you  spend

time  doing  more useful things. As well as sending one file at a time

from one machine to the other, you can send them in groups: thus,  you

could  say  "send  all  the  files  on my disc to VME" in one command.

The KERMIT programs will send the files one by one until all are gone,

quite automatically. This facility is not available with VME KERMIT.

2-3

VME KERMIT User Guide

3: CONTROLLING VME KERMIT

=========================

In this section we shall look at how you start and stop VME KERMIT,

and also how its command language operates.

3-1

VME KERMIT User Guide

3.1 ENTERING VME KERMIT

-----------------------

To  invoke VME KERMIT you must log in to VME and issue the command:

KERMIT ( OPTION   = @ initial mode         @ ,

VME_FILE = @ name of VME file     @ ,

REM_FILE = @ name of remote file  @ ,

RESPONSE = @ result code variable @ )

to the VME command prompt.

3.1.1. The OPTION Parameter

This parameter defines which mode VME KERMIT is to start up in.

If  you  set  this  parameter to COMMAND (the default) then VME

KERMIT will start up in command mode and you will  be  prompted

to  type comands. You must not specify the VME_FILE or REM_FILE

parameters when you have set the OPTION parameter to COMMAND.

If  you wish to initiate a file transfer directly you may do so

by setting the OPTION parameter to either RECEIVE  (VME  KERMIT

is  to  receive a file) or SEND (VME KERMIT is to send a file).

For RECEIVE, the VME_FILE parameter is optional  but  you  must

not  set  the  REM_FILE  parameter.  For  SEND,  the  VME_FILE

parameter is mandatory and it must be set to the  name  of  the

VME  file  to  be  sent.  The  REM_FILE  parameter  however  is

optional.

Alternatively you may wish to enter server  mode  directly, you

may  do  this by setting the OPTION parameter to SERVER. As for

COMMAND mode the VME_FILE and REM_FILE parameters must  not  be

specified.

3.1.2. The VME_FILE parameter

This parameter specifies the name of the VME file to be used in

the file transfer.

If you have set the OPTION parameter to COMMAND or  SERVER  you

must not set this parameter.

This  parameter  is optional if you set the OPTION parameter to

RECEIVE. If it is given then it must contain the  name  of  the

VME  file  to  receive  the data. If you default this parameter

then  KERMIT  will  take  the  VME  file  name  from  the  file

indentified by the micro KERMIT.

This parameter is manadatory when you set the OPTION  parameter

to  SEND  and  it  must  contain the name of the VME file to be

transfered.

3.1.3. The REM_FILE parameter

This  parameter  specifies  the  file name that VME KERMIT will

send to the micro KERMIT to identify the file.

This parameter is optional when the OPTION parameter is set  to

SEND, but  must  be  omitted in all other cases. If you set the

3-2

VME KERMIT User Guide

option parameter to SEND and default  this  parameter  the  VME

KERMIT will send to the micro KERMIT a file name taken from the

VME file name.

3.1.4. The RESPONSE parameter

This parameter is used to return result codes. VME KERMIT  does

not  generate  any  result  codes  of it's own but standard VME

result codes may be returned. Normally this parameter should be

omitted.

3.2 LEAVING VME KERMIT

----------------------

To  leave  VME KERMIT you would normally use the EXIT command. This

will close down KERMIT and return to the VME command level.

Alternatively, if you are running VME KERMIT in server mode you can

close  KERMIT  remotely  and either return to the VME command level or

log out the VME terminal session.

3.3 VME KERMIT COMMAND LANGUAGE

-------------------------------

You  control  what  you want VME KERMIT to do, and how it should do

it, by giving it commands in its "command language". The format of the

command language closely  follows  that  used  on  most  other  KERMIT

implementations on other machines.

3.3.1. Command format

When you enter VME KERMIT, you will see a prompt on the screen

VME Kermit>

This indicates  that  KERMIT  is expecting you to type a command.

KERMIT commands can be typed in upper case, lower  case,  or  any

mixture of the two as you please.

KERMIT  commands  take the form of a command name, such as "SET"

sometimes followed by one or more further pieces  of  information

or  "parameters",  which  must  be  separated  from each other by

spaces.

For example, one command you might use is

SET RETRY 10

to  set  the  number  of  times an attempt will be made to send a

particular packet before giving up. Here the  command  is  "SET",

and  RETRY  and 10 are the parameters to the command. Since there

are a large number of variations to the SET  command,  these  are

often  referred  to  in  this guide as separate comands. Thus you

3-3

VME KERMIT User Guide

see references  to  the  "SET  FILE  TYPE  BINARY"  command,  for

instance.  You can regard this as either a command whose name has

spaces in it, or as a form of the  SET  command  with  parameters

FILE,  TYPE  and  BINARY.  In either case, the effect will be the

same.

At the end of each command you should press the  RETURN  key  to

send  the  line  to  KERMIT for action. If you have made a typing

mistake you can use the delete key and cancel key exactly as  for

other VME commands.

The  reference  section  at  the  end  of  this  guide gives the

complete specification  of  all  the  VME  KERMIT  commands.  The

commands  are  grouped according to their function (such as "file

transfer control"), rather than in one  alphabetical  list.  Thus

you  will  find  variations  of the SET command appearing in many

places, grouped with other commands that  function  in  the  same

area.

The  reference  section presents each command in a formal way to

show you exactly what you are allowed to type at any  point.  You

might find one command described as having a format

SET PAUSE <number>

for example. Here the fixed parts, that you would always include,

are  shown in capitals. The third part of the command, "<number>"

is enclosed in "<>" brackets to show that here you  must  provide

your  own  parameter  value.  The  fact  that  it  says  "number"

indicates that what you should type is a number of some sort: the

description of the command will tell you what sort of number  you

must  provide and why. Note that you don't type the "<>" brackets

yourself here. Thus, a valid way of using this command might be

SET PAUSE 10

where you have supplied the  numeric  value  "10"  where  it  was

expected.

Sometimes,  not  all  the  parameters  of  some commands need be

typed. In these cases, VME KERMIT will take a "default" value for

the parameter you did not supply. The reference section will tell

you which parameters can be omitted, and what values  VME  KERMIT

will assume if they are omitted.

3.3.2. Abbreviating commands

To  reduce  the amount of typing that you need to do, VME KERMIT

allows you to abbreviate all commands and  most  parameters.  You

may  abbreviate  to  as few characters as you wish, provided that

what you type is unique. Thus

HELP

HEL

HE

H

3-4

VME KERMIT User Guide

are legal, but

S

is not, since it could stand for either SEND, SERVER, SET,  SHOW

or STATISTICS. To take a full example, you could type either

SET FILE TYPE BINARY

or abbreviate it as far as

SET F T B

3.3.3. Numeric parameters

Except  where explicitely stated, numeric parameters to commands

can be typed in either decimal or hexadecimal.  By  default,  VME

KERMIT  expects  the  value to be decimal, but you can indicate a

hexadecimal number by preceding the parameter with an "X".

Thus

123

is a decimal value, and

X123

is a hexadecimal value.

3.3.4. The help facility

Quite  often,  you  may wish a reminder as to what parameters or

commands you can give. KERMIT has a "help" facility that will act

as an "aide-memoire" for you.

To use help, simply type HELP in response to the command prompt.

KERMIT will immediately give you a list of the commands available

and  the  option  of  requesting  further  information  about the

commands.

For example, if you type HELP after the prompt thus:

VME Kermit> HELP

KERMIT  will  output  a list of the commands you can use. You may

then request more information about one of the commands from  the

menu.

The  HELP  command  may  be  augmented  by  folowing  it  by  a

command name and parameters.

3-5

VME KERMIT User Guide

For  example,  if  you  require  help  for  the "SET" command you

type

VME Kermit> HELP SET

VME KERMIT will then give you an explanation ot the "SET" command

and  a  list  of the parameters available. You will then be given

the option of requesting further information about the parameters

and their values.

3-6

VME KERMIT User Guide

4: TRANSFERRING FILES WITH KERMIT

=================================

The primary use of VME KERMIT is to transfer files between it and a

microcomputer.  The  methods  used  will  be  substanstially  the same

whatever the other system is, since any KERMIT system should  be  able

to  communicate  with any other. Though the general techniques will be

same, the exact commands used to control the  different  KERMITs  will

vary  from  one  system  to another. You will need to consult the user

guides of the other system to discover how it should be controlled. In

this section we shall cover in detail how VME KERMIT is controlled.

4-1

VME KERMIT User Guide

4.1 PRINCIPLES

--------------

KERMIT differs from other "dumb" file transfer systems (such as you

might  find  in a terminal emulator ROM, for instance) in that it aims

to transfer files in a USABLE way between systems. Thus, if you have a

program  source  on  your  micro's  discs  that you can edit with some

editor, and transfer this to VME, the resulting file  should  also  be

editable  using VME's editors. KERMIT will resolve all the differences

in how files are stored on your behalf, so that, in this example,  the

VME  file  will not contain extra line-feed characters at inconvenient

places that the VME editor cannot handle.

Transferring  files with KERMIT involves several discrete steps. We

shall consider here the most common case of transfer  to  and  from  a

microcomputer.

1. The micro KERMIT is entered and set up  for  the  transfer. In

particular, you may wish to tell KERMIT what types of file are to

be moved. You will also need to set the parameters  for  terminal

emulation,  and,  depending  on the micro system's needs, you may

need to change some of the  more  detailed  aspects  of  the  VME

KERMIT's operation.

2. Terminal emulation mode is entered, and the micro is logged in

to VME as though it were an ordinary terminal.

3. Operating as a terminal, the VME KERMIT program is started.

4. Commands  can  then  be given to the VME KERMIT (from terminal

emulation mode) and to the micro's KERMIT  (from  KERMIT  command

mode).  The  two  KERMIT systems will communicate with each other

using the standard KERMIT protocol.

5. After the transfers are done, the terminal is logged out  from

the VME system.

In  practice,  the  steps  taken  will  range up and down this list as

required. For example, VME KERMIT parameters can  be  changed  at  any

time,  not  only  at  the start, and if you are moving several tyes of

file you will need to change them frequently. In the sections below we

shall consider the various actions you will need to take; the order of

doing them is up to you.

4.2 SETTING FILE TYPE

---------------------

4.2.1 How files are organised

VME  KERMIT can only operate on serial access files, and the user

must have the required access  permissions  (read  or  write,  as

necessary). The maximum record size that VME KERMIT can handle is

4096 bytes.

4-2

VME KERMIT User Guide

4.2.2 Sending eight bit data

Futher more VME KERMIT restricts data it sends to only 7 bits in

each byte: thus you can only normally send characters whose ASCII

codes  are in the range 0 to 127. However, every binary file will

contain bytes from the whole character set, with codes from 0  to

255.

KERMIT   in   general   has  a  technique  for  overcoming  this

restriction, by encoding characters  in  the  range  128  to  255

into  special  sequences  that can be sent down any communication

line. VME KERMIT and most modern KERMITs will use this technique,

which  is  known  as "eigth-bit-prefixing", but you may encounter

an older implementation on some machine that does not support it.

In  this case it will not be possible to transfer the file. There

is regrettably, no way round this problem from within KERMIT.

4.2.3. Binary files

These  files  contain  data that is not primarily printable text,

such as machine-code programs. When you transfer these files, you

wish  every byte in the file on one system to appear unchanged in

the file on the other system, regardless of what it is.

You tell KERMIT that you  are  handling  binary  files  with  the

command

SET FILE TYPE BINARY

which  tells  it  not  to change any data that it either sends or

receives. Note here that you  may  need  to  issue  a  comparable

command  to  the micro KERMIT, to prevent it trying to manipulate

the data.

4.2.4. ASCII files

These  files  contain  data that is primarily printable text, but

they also contain some characters whose codes are in the range  0

to 255, such as word processing text files. As with binary files,

the data is transferred  byte-for-byte,  but  in  this  case  the

record format is maintained.

You  tell  KERMIT  that  you  are  handling  ASCII files with the

command

SET FILE TYPE ASCII

which  tells  it  not  to change any data that it either sends or

receives, but it is to recognise end of record delimeters.

4.2.5. EBCDIC files

These  files  contain  printable  text, such as program listings.

When you transfer one of these files, you do not necessarily want

a  byte-for-byte  transfer,  since the two machines may differ in

how they store text files.  VME  for  example,  uses  the  EBCDIC

character code to store text files,  whilst  most micro's use the

ASCII character code. The KERMIT standards defines a fixed way in

4-3

VME KERMIT User Guide

which things such as end-of-line are transferred: VME KERMIT will

translate your data to this standard format, and  the  other  end

will  then  translate  the  standard format into whatever its own

specific requirements are.

You tell VME KERMIT that EBCDIC text files are to be  transferred

with the

SET FILE TYPE EBCDIC

4.3 FILE TRANSFER AS A SERVER

-----------------------------

VME  KERMIT  will  run as a KERMIT server, but it is essential that

your micro KERMIT is able to communicate with a server:  that  is,  it

must support the commands GET and BYE or FINISH.

You should put VME KERMIT into server mode with the command "SERVER".

You should then use your micro KERMITs local escape sequence to return

to  command  mode. The operation of both KERMITs can now be controlled

from your micro KERMIT, and your micro KERMITs user guide should  give

you further information regarding running with a server.

4.4 FILE TRANSFER AS A NON-SERVER

---------------------------------

Transferring  data  as  a  non-server is a little more complicated,

since you will need to continually  change  from  the  micro  KERMITs

command mode to terminal mode and back again. With a little  practice,

though, the technique becomes natural.

4.4.1. Sending files as a non-server

To send a file as a non-server you use the command SEND. However,

you must also tell the micro KERMIT that a file is on its way.

One means of doing this is as follows:

a. In terminal mode, start the VME KERMIT program, and issue

the  SEND command. This tells it to send a file to the micro

system. Normally, VME KERMIT will pause for  30  seconds  to

enable  you  to escape back and issue commands to your micro

KERMIT.

b. Use  your  micro  KERMITs local escape sequence to return

back to its command mode.

c. Issue  the  micro  KERMIT  RECEIVE  command. When the VME

KERMITs delay expires, it will start to send the file.

4.4.2. Receiving files as a non-server

If  the  VME  KERMIT system is not running as a server, you will

need to transfer files from it by the exact reverse of the above

4-4

VME KERMIT User Guide

SEND procedure:  all  you  need to do is reverse the roles of the

two machines. Thus, you could take the following steps:

a. In terminal mode, start the VME KERMIT program, and issue

its RECEIVE command. This tells it that a file is on its way

from the micro system.

b. Use  your  micro KERMITs local escape sequence to return

back to its command mode.

c. Issue   the  micro  KERMITs  SEND  command.  There  will

normally be a delay before anything happens.  The  interval

may  be anything from a few seconds upwards. When the micro

system's delay time expires, it will start to send the file

4.5 HANDLING PROBLEMS

---------------------

By  design,  KERMIT  is a highly reliable file transfer system, and

performs considerably better than any "dumb capture" facility within a

terminal emulator. The error-detection capabilities of  KERMIT  ensure

that  data  is  transmitted  correctly:  in  the  rare cases where the

communications system you are using is unreliable, KERMIT systems will

abort a file transfer rather than transfer garbage.

That said, there are some cases where you  may  need  to  give  VME

KERMIT  some assistance. The most common case will arise when one byte

is transmitted by one system but does not arrive at the other.  KERMIT

breaks  data up into small chunks called "packets", and if the missing

byte is one of the ones that the KERMIT systems use to  control  these

packets,  you  may  end  up with a machine that is waiting forever for

something that will never arrive.

The simplest way out of this possible problem is for you to keep an

eye on the progress of the transfer and see when it appears  to  grind

to a halt. Your micro KERMIT may display a continuous count of packets

as  they  are  transmitted  and  received:  normally  the  number will

increment steadily. If the number does not change  for  a  significant

time  (and  here  you  must  bear  in  mind that the VME system may be

running very slowly, so allow a good interval  to  pass),  your  micro

KERMIT  may  have  a  facility,  where  you  can  manually force it to

to  drop whatever it was doing, and retransmit its last packet of data

to the other end. If the VME system was stuck waiting  for  data  that

had   been  lost,  the  retransmission will prod it into life, and the

automatic recovery mechanisms of KERMIT should allow the two  ends  to

pick up the transfer from where it stopped.

If  you wish, you can automate this recovery by using the "timeout"

facility of VME KERMIT. This is described  below  in  the  section  on

"Advanced facilities".

4-5

VME KERMIT User Guide

4.6 ADVANCED FACILITIES

-----------------------

VME  KERMIT  permits  you  to  use some more advanced techniques to

control file transfers. Some of these  facilities  involve  the  micro

KERMIT  system,  and  it  is possible that you will find a system that

does not implement them, since they are fairly recent additions to the

KERMIT specification. However, VME KERMIT will be able to detect this,

and will act accordingly.

4.6.1. Incomplete file transfers

If  you  interrupt  the  reception of a file into VME KERMIT (or,

indeed if an error such as "file full" occurs), you will be  left

with  a  file  on  disc  that contains only part of the data that

should have been sent. By default, VME KERMIT  will  delete  this

partial  file  for you. If, though, you want to preserve whatever

data has been transferred, you can issue the command

SET FILE INCOMPLETE KEEP

after which VME KERMIT will close the file normally and  preserve

it.  You can re-instate the normal situation at any time with the

command

SET FILE INCOMPLETE DISCARD

4.6.2. Using timeouts

As we saw above, it is possible to automate  the  detection  that

the  transfer  has  stopped.  This  is  done by defining "timeout

periods": if one or other system has not received any  data  when

its timeout expires, it will try to re-establish contact with the

other.

You  control  the  use  of  timeouts  in  two ways. The first way

enables you to tell the micro  system what  timeout  interval  it

should  use when receiving data from VME KERMIT. This information

is transmitted to the micro system when  a  transfer  starts.  By

default,  VME  KERMIT asks the micro system not to use a timeout,

but you can change this with the SET SEND TIMEOUT command.  Note,

though,  that  the micro system may or may not do as it is asked:

it may not support the timeout facility; it may ignore the  value

sent  and  use its own interval; or the timeout facility may need

to be switched on by your giving it a command.

VME  KERMIT  can  also  timeout  if data does not arrive from the

micro  system within a given  interval.  By  default,  a  timeout

interval of 0 seconds is set, so VME KERMIT  will never time out.

Issuing the command

SET RECEIVE TIMEOUT 30

will set the timeout interval to 30 seconds, and VME KERMIT  will

time  out  if  it  has  not  received any data in 30 seconds. The

SET RECEIVE TIMEOUT command lets  you  change  this  interval  as

required.  You can turn the timeout facility off at any time with

4-6

VME KERMIT User Guide

SET RECEIVE TIMEOUT 0

Note  that  here  the  micro   system may ask VME KERMIT to use a

specified timeout period when a transfer starts.

4.6.3. Protocol control

The rules by which files are transferred between  KERMIT  systems

are  termed  the  "KERMIT protocol". These rules define in detail

how data should be transferred: they specify how much can be sent

in one chunk or packet; what control sequences indicate the start

and end of a packet; what character encoding is to be  used,  and

so  on.  In almost every case you will have no need to change any

of these settings, since they are carefully chosen  so  that  any

KERMIT  can  communicate  to any other KERMIT in just about every

circumstance.

However, it is possible that you may come across cases where  you

need to change some of the protocol values, either to improve the

performance of  the  file  transfer  mechanism,  or  because  the

standard settings are inappropriate and do not work.

The  protocol  values  are  changed  by  the SET command, and VME

KERMIT  allows  you  to  change  all  the  possible  values.  The

reference  section  details  all  the  SET commands concerned and

their effects. A detailed discussion of the various possibilities

is beyond the scope  of  this  user  guide,  though,  since  some

understanding  of  the  KERMIT  protocol is needed. You will find

this protocol explained in  the  "KERMIT  Protocol  Manual"  (use

issue  5  or  later);  or  you  might contact the systems support

personnel at your computer centre.

4-7

VME KERMIT User Guide

5: COMMANDS FOR GENERAL CONTROL OF KERMIT

=========================================

In this section, we shall look at the commands you can you can  use

to control the general operation of VME KERMIT.

5-1

VME KERMIT User Guide

EXIT

This  command causes VME KERMIT to return to the VME command mode

at the end of a session.

The command format is:

EXIT

There are no parameters.

Example:

EXIT

HELP

This  command  gives  information  for  the specified command, or

allows information for a number of commands to be  requested  via

a menu.

The command format is:

HELP <command>

The parameters are:

<command>

This parameter must be one of the VME KERMIT commands.

If  the  parameter  is  omitted,  the  help  command menu will be

displayed and you will be able to select th e command  for  which

you require the information.

Example:

HELP SET FILE TYPE

will display the help text for the command SET FILE TYPE.

SET DEBUG

This command allows you turn the VME KERMIT debugging facility on

or off.

The command format is:

SET DEBUG <level> <action>

5-2

VME KERMIT User Guide

The parameters are:

<level>

This  parameter  must  be one of: FILE (contents of all

file  records  read  or  written),  PROTOCOL  (protocol

states  and  events),  PACKETS (contents of all packets

sent or received), DEVICE (all  data  transmitted  over

the communications line) or ALL (all the above).

<action>

This must be either ON or OFF.

If debugging is on, VME KERMIT will output information during all

file  transfers  to  enable  you  to  trace  errors.  It  may  be

configured to record the protocol state, the contents of the file

sent or received,  the  contents  of  all  packets  received  and

transmitted, plus all the data transmitted over the communication

line. All this information is output to the users job journal.

By default, all debugging is OFF.

Example:

SET DEBUG PACKET ON

will turn on packet level debugging.

SHOW

This  command  displays  the  values  of  all  the KERMIT control

values.

The command format is:

SHOW

There are no parameters.

Example:

SHOW

STATISTICS

This  command  displays the statistics about the most recent file

transfer, such as the number of packets received etc.

The command format is:

STATISTICS

There are no parameters.

5-3

VME KERMIT User Guide

Example:

STATISTICS

5-4

VME KERMIT User Guide

6: COMMANDS FOR FILE TRANSFER

=============================

In this section we  shall  look  at  the  detailed  format  of  the

commands  that  you  use  to  transfer  files using VME KERMIT, and to

control how VME KERMIT will perform the transfers.

6-1

VME KERMIT User Guide

RECEIVE

This  command causes VME KERMIT to wait for a file transfer to be

started by the micro system. You will thus need to issue  a  SEND

command to the micro KERMIT in order to make somethong happen.

The command format is:

RECEIVE <VME-file>

The parameters are:

<VME-file>

This parameter is optional, and specifies the name of a

file on the VME system into which you wish data  to  be

transferred.

When the command is issued, VME KERMIT will wait passively for  a

signal  from the micro  KERMIT that a file transfer is beginning.

This signal will include the name of the file that is being sent:

if  you  have included the <VME-file> parameter, this name is for

information only, and the data will be written to  the  file  you

have identified.

If  you omit the <VME-file> parameter, VME KERMIT will by default

attempt to  generate  a  suitable  VME  filename  from  the  name

supplied by the micro system.

Example:

RECEIVE MYFILE

will  cause  VME  KERMIT  to receive a file, and to store it in a

file called MYFILE.

SEND

This command sends a file to the micro KERMIT.

The command format is:

SEND <VME-file> <remote-file>

The parameters are:

<VME-file>

This  parameter  is mandatory. It specifies the name of

the file on the VME system that you wish to  send,  and

can be any legal VME filename.

<remote-file>

This  parameter  is optional. It specifies the filename

that VME KERMIT  will  send  to  the  micro  system  to

6-2

VME KERMIT User Guide

identify the file: this may be used by the micro system

as the name of the file that it puts the data into, but

this  is  entirely  up  to  the  micro  system.  If the

filename is legal for the micro system, the file  would

normally be called by this name.

You  must  issue a RECEIVE command to the micro system to prepare

it for this SEND. You must issue the SEND first  and  then  issue

the  RECEIVE  to  the  other  system. To allow you to do this VME

KERMIT will pause after you issue the SEND command (by default 30

seconds)  before  it actually starts sending. You can change this

pause interval with the SET DELAY command.

If  you  omit  the  <remote-file>   parameter, VME KERMIT will by

default  generate  a  "generally  acceptable"   name   from   the

<VME-file>   parameter.   It  does  this  by  stripping  off  any

user-name, library-name, generation number etc.

Example:

SEND MYFILE YOURFILE

causes  VME  KERMIT  to send the file MYFILE to the micro KERMIT.

The file will be identified as YOURFILE by the micro system.

SERVER

This  command causes VME KERMIT to enter server mode and to wait

for commands or file transfer requests from  the  micro  system.

You  will  thus  be able to control the KERMIT session from your

micro.

The command format is:

SERVER

When  the  command  is issued, VME KERMIT will wait passively for

commands and file transfer requests from the micro KERMIT.

Example:

SERVER

SET DELAY

This command sets the period for which VME KERMIT will wait after

you issue a SEND command.

The command format is:

SET DELAY <number>

6-3

VME KERMIT User Guide

The parameters are:

<number>

The delay in seconds, in the range 5 to 300.

When you issue a SEND command, VME KERMIT will do nothing for the

specified period to enable you  to  issue  commands  to  a  micro

system.

By default, VME KERMIT will delay for 30 seconds after you  issue

a SEND command.

Example:

SET DELAY 10

causes  VME  KERMIT to wait for 10 seconds after receiving a SEND

command, before starting the file transfer.

SET FILE INCOMPLETE

This  command  defines  what  VME  KERMIT  will  do if wither you

interrupt the reception of a file, or if an error occurs while  a

file is being received.

The command format is:

SET FILE INCOMPLETE <action>

The parameters are:

<action>

This must be either KEEP or DISCARD.

If  the action is set to DISCARD, VME KERMIT will delete any file

whose reception is interrupted and is  thus  incomplete.  If  the

action  is  set to KEEP, VME KERMIT will close the file normally,

and you will be able to use whatever data was received before the

interruption or error.

By default, VME KERMIT will DISCARD an incomplete file.

Example:

SET FILE INCOMPLETE KEEP

causes VME KERMIT to save incomplete files.

SET FILE NAMING

This  command  defines how VME KERMIT interprets file names, when

the VME filename  or  the  remote  filename  are  not  explicitly

stated.

6-4

VME KERMIT User Guide

The command format is:

SET FILE NAMING <action>

The parameters are:

<action>

This must be either NORMAL-FORM or UNTRANSLATED.

If  the  action  is  set  to  NORMAL-FORM,  VME KERMIT will, when

sending a file, and the remote filename is  omitted,  attempt  to

genrate  a  "generally acceptable" name from the VME filename. It

does  this  by  stripping  off   any   user-name,   library-name,

generation  number etc. Similarly, when receiving a file, and the

VME filename is omitted, VME KERMIT will attempt  to  generate  a

suitable name from the name supplied by the remote KERMIT.

If  action  is  set  to UNTRANSLATED, VME KERMIT will not attempt

to generate a standardised name.

By default, VME KERMIT will normalise filenames.

Example:

SET FILE NAMING UNTRANSLATED

will turn off file normalisation.

SET FILE OVERWRITE

This command specifies the action that VME KERMIT  must  take  on

reciving  a  file  if  the  name it tries to store the data under

clashes with an existing file.

The command format is:

SET FILE OVERWRITE <action>

The parameters are:

<action>

This must be either ON or OFF.

If  the  file  overwrite  facility  is  ON, VME KERMIT will check

whether the name it wishes to use to store incoming data  clashes

with an existing file name. If it does, VME KERMIT will overwrite

the file with the new data.

If file overwrite is OFF, VME KERMIT will not overwrite the file,

but will report an error.

By default, file overwrite is OFF.

6-5

VME KERMIT User Guide

Example:

SET FILE OVERWRITE ON

turns on the file overwrite facility.

SET FILE TYPE EBCDIC

This  command  tells  VME  KERMIT  that  files  it  transmits and

receives  are  to  taken  as  containing  printable text  and  to

transform them accordingly.

The command format is:

SET FILE TYPE EBCDIC

There no parameters.

After  using  this  command,  VME  KERMIT  treats  all  files  as

containing printable text. When sending a file, it will translate

all EBCDIC text to ASCII, and transform every end of record  into

KERMIT's  standard "end of line" terminator, and the micro KERMIT

should then change this into whatever the standard representation

of  end-of-line  is  on its own system. When receiving files, VME

KERMIT will translate all ASCII text to EBCDIC, and  treat  every

occurrence  of  the  standard KERMIT "end-of-line" indicator as a

record terminator.

Note,  you  may  need  to give an equivalent command to the micro

KERMIT system to make it treat the data correctly.

By  default, VME KERMIT treats files as EBCDIC.

Example:

SET FILE TYPE EBCDIC

makes VME KERMIT treat files as containing EBCDIC data.

SET FILE TYPE ASCII

This  command  tells  VME  KERMIT  that  files  it  transmits and

receives are to taken as containing ASCII text and  to  transform

them accordingly.

The command format is

SET FILE TYPE ASCII

There no parameters.

After using this command, VME KERMIT will transmit the characters

from  the  file  exactly  as they are, and will not change any of

6-6

VME KERMIT User Guide

them. Similary the data it receives will be written to file  with

no  alteration.  However,  as  with EBCDIC files, VME KERMIT will

maintain the record format.

Note,  You  may  need  to give an equivalent command to the micro

KERMIT system to make it treat the data correctly.

By default, VME KERMIT treats files as EBCDIC.

Example:

SET FILE TYPE ASCII

makes VME KERMIT treat files as containing ASCII data.

SET FILE TYPE BINARY

This command tells VME KERMIT that all files it receives or sends

should be treated as containing binary data.

The command format is:

SET FILE TYPE BINARY

There are no parameters.

After using this command, VME KERMIT will transmit the bytes from

a file exactly as they are, and will  not  change  any  of  them.

Similarly,  the  data it receives will be written to file with no

alteration.

Note,  You  may  need  to give an equivalent command to the micro

KERMIT system to make it treat the data correctly.

By default, VME KERMIT treats files as EBCDIC.

Example:

SET FILE TYPE BINARY

makes VME KERMIT treat files as containing binary data.

SET RECEIVE TIMEOUT

This  command  defines  the  interval after which VME KERMIT will

take action if it has received no data from the micro system.

The command format is:

SET RECEIVE TIMEOUT <interval>

6-7

VME KERMIT User Guide

The parameters are:

<interval>

Specifies the timeout period in seconds, in the range 0

to 94.

The command defines the timeout that VME KERMIT is to use.

The  choice  of  the  timeout period will depend on many factors.

Regard should be paid to how fast the VME system is operating, it

could  be  running slowly due to heavy use and also to the delays

that  operating  over a network may  impose. Care should be taken

not to set the value too  low,  or  VME  KERMIT  will  repeatedly

timeout unnecessarily.

By default, a timeout interval of 0 seconds is set.

Example:

SET RECEIVE TIMEOUT 10

causes  VME  KERMIT to timeout if no data has been received after

10 seconds.

SET SEND TIMEOUT

This command sets the timeout interval that VME KERMIT  asks  the

micro  system to use. The micro  system should timeout if no data

has been received from VME KERMIT after this interval.

The command format is:

SET SEND TIMEOUT <interval>

The parameters are:

<interval>

specifies  the  interval  required  in  seconds, in the

range 0 to 94.

VME KERMIT will pass this value  to  the  micro   system  at  the

beginning  of  every  file  transfer.  Note,  though,  that it is

entirely up to the micro  system  whether  it  uses  this  value,

ignores  it, or uses a value of its own. You may need to give the

micro  system commands to turn its timeout facility on.

By default, VME KERMIT requests the micro system to set a timeout

value of 0 seconds.

Example:

SET SEND TIMEOUT 10

causes  VME  KERMIT  to ask the micro  system to timeout after 10

seconds if no data has been received from the VME system.

6-8

VME KERMIT User Guide

7: COMMANDS FOR DETAILED PROTOCOL CONTROL

=========================================

The commands described in this section are used to  exert  detailed

control over the KERMIT protocol. As explained earlier, it is unlikely

that you would ever need to use these commands. You should consult the

KERMIT  Protocol  Manual  for a detailed description of the facilities

they control.

7-1

VME KERMIT User Guide

SET PAUSE

This command defines a "turn-round" delay that  VME  KERMIT  will

use  in  packet transmission. It defines the time that VME KERMIT

will wait, after receiving  a  packet  from  the  micro   system,

before  transmitting the next to it. This overcomes problems with

some systems that cannot accept input that comes too  soon  after

output.

The command format is:

SET PAUSE <number>

The parameters are:

<number>

The pause time in tenths of a second, in the range 0 to

50.

By  default, VME  KERMIT does not pause (i.e. the pause value is

0).

Example:

SET PAUSE 1

causes VME KERMIT  to  pause  for  1  tenth  of  a  second  after

receiving one packet before sending the next.

SET RECEIVE END-OF-LINE

This  command defines the value that VME KERMIT request the other

end to use to signal the end of a packet.

The command format is:

SET RECEIVE END-OF-LINE <number>

The parameters are:

<number>

The number of the control character in the range  0  to

31 or 127. Note, a number of the control characters are

not accepted by VME as a valid  end-of-line  terminator

(see Appendix 2).

By  default,  VME  KERMIT  asks for packets to be terminated with

Carriage Return (hex 0D).

7-2

VME KERMIT User Guide

Example:

SET RECEIVE END-OF-LINE X09

requests the micro KERMIT to terminate  packets  with  Horizontal

Tab (code hex 09).

SET RECEIVE PACKET-LENGTH

This command defines the maximum packet size that VME KERMIT will

ask the micro  system to send.

The command format is:

SET RECEIVE PACKET-LENGTH <number>

The parameters are:

<number>

The  maximum  size  of  packet  wanted  from the micro

system, in the range 30 to 94.

By default, VME KERMIT asks the micro  system to  use  a  maximum

packet size of 80 bytes.

Example:

SET RECEIVE PACKET-LENGTH 60

causes  VME  KERMIT to request the micro  system to use a maximum

packet size of 60 bytes.

SET RECEIVE PAD-CHARACTER

This  command  defines  the  control  character  that  VME KERMIT

requests the other to use when padding before sending a packet.

The command format is:

SET RECEIVE PAD-CHARACTER <number>

The parameters are:

<number>

The  numeric  code  of the control character to be used

for padding in the range 0 to 31 or 127. Note, a number

of  the  control  characters are not accepted by VME as

valid pad characters (see Appendix 2).

By default, VME KERMIT will ask for a pad character of code 0.

7-3

VME KERMIT User Guide

Example:

SET RECEIVE PAD-CHARACTER X00

requests the micro  KERMIT to pad with NUL characters (code 0).

SET RECEIVE PADDING

This command defines the number of pad characters that VME KERMIT

will ask the other end to send before each packet.

The command format is:

SET RECEIVE PADDING <number>

The parameters are:

<number>

The number of pad characters to be  requested,  in  the

range 0 to 94.

By default, VME KERMIT asks for no pad characters to be sent.

Example:

SET RECEIVE PADDING 10

will  cause  VME  KERMIT  to request the micro  system to precede

each packet with 10 pad characters.

SET RECEIVE START-OF-PACKET

This  command  defines  to VME KERMIT the control character that

the micro system will use to indicate the start of each packet.

The command format is:

SET RECEIVE START-OF-PACKET <number>

The parameters are:

<number>

The  numeric  code  of  the character the micro  system

will use, in the range 0 to 31 or 127. Note,  a  number

of  the  control  characters are not accepted by VME as

a valid start-of-packet marker (see Appendix 2).

By default, VME KERMIT will expect the micro  system  to  precede

each packet with a hex 1E character.

7-4

VME KERMIT User Guide

Example:

SET RECEIVE START-OF-PACKET X07

tells  VME KERMIT that the micro  system will precede each packet

with a hex 07 character.

SET RETRY

This  command  sets  the maximum number of times that VME KERMIT

will attempt to send or receive a packet before  abandoning  the

transfer.

The command format is:

SET RETRY <number>

The parameters are:

<number>

The  maximum  number of retries to be attempted, in the

range 0 to 10.

By default, VME KERMIT will retry transfers 4 times.

Example:

SET RETRY 7

sets the packet retry limit to 7.

SET SEND EIGTH-BIT-PREFIX

This command defines  the  eigth-bit-prefix  character  that  VME

KERMIT will ask the micro system to agree to use.

The command format is:

SET SEND EIGTH-BIT-PREFIX <character>

The parameters are:

<character>

The printable character to be used.

By  default,  VME KERMIT will attempt to use the "&" character as

its eigth-bit prefix.

7-5

VME KERMIT User Guide

Example:

SET SEND EIGHT-BIT-PREFIX %

sets the eigth-bit prefix character to be "%".

SET SEND END-OF-LINE

This  command  defines the control character that VME KERMIT will

use to terminate the first packet it sends to the micro system in

each  transfer.  Subsequent  packets  will  be  terminated by the

character that the micro system requests in its SEND-INIT packet.

The command format is:

SET SEND END-OF-LINE <number>

The parameters are:

<number>

The  numeric  code  of the character to be used, in the

range 0 to 31 or 127. Note, a  number  of  the  control

characters   are   not  accepted  by  VME  as  a  valid

end-of-line terminator (see Appendix 2).

By default, VME KERMIT uses hex 0D (Carriage Return).

Example:

SET SEND END-OF-LINE 9

causes the first packet sent in each transfer to  be  ended  with

Horizontal Tab (code 9).

SET SEND PAD-CHARACTER

This  command defines the padding character with which VME KERMIT

will  precede  the  first  packet  to  be  sent  in  a  transfer.

Subsequent packets will be preceded by the character requested by

the micro  system in its SEND-INIT packet.

The command format is:

SET SEND PADCHAR <number>

The parameters are:

<number>

The  numeric  code  of the character to be used, in the

range 0 to 31 or 127. Note, a  number  of  the  control

characters  are  not  accepted  by  VME  as a valid pad

character (see Appendix 2).

7-6

VME KERMIT User Guide

By default, VME KERMIT uses a pad character of NUL (code hex 0).

Example:

SET SEND PADCHAR X1A

sets the pad character used to be hex 1A.

SET SEND PADDING

This command defines the number of pad characters VME KERMIT will

send before the first packet in a  transfer.  Subsequent  packets

will  be preceded by the number requested by the micro  system in

its SEND-INIT packet.

The command format is:

SET SEND PADDING <number>

The parameters are:

<number>

The number of pad characters to be sent, in the range 0

to 94.

By default, VME KERMIT sends no pad characters.

Example:

SET SEND PADDING 21

causes VME KERMIT to send 21  pad  characters  before  its  first

packet.

SET SEND QUOTE

This  command  defines  the character that VME KERMIT will use to

prefix control characters in data packets that it sends.

The command format is:

SET SEND QUOTE <character>

The parameters are:

<character>

The printable character to be used.

By  default,  VME KERMIT uses the "#" character (hash). Note that

the "hash" character appears as "pound"  on  some  keyboards  and

printers.

7-7

VME KERMIT User Guide

Example:

SET SEND QUOTE $

causes VME KERMIT to prefix control characters it sends  in  data

packets with a "$".

SET SEND START-OF-PACKET

This  command  defines the control character that VME KERMIT will

send to indicate the start of every packet.

The command format is:

SET SEND START-OF-PACKET <number>

The parameters are:

<number>

The numeric code of the character to be  sent,  in  the

range  0  to  31  or 127. Note, a number of the control

characters  are  not  accepted  by  VME  as   a   valid

start-of-packet marker (see Appendix 2).

By default, VME KERMIT sends a hex 1E character in front of every

packet.

Example:

SET SEND START-OF-PACKET X08

causes  VME KERMIT to precede every packet it sends with a hex 08

character.

7-8

VME KERMIT User Guide

8: OBTAINING, BUILDING AND MODIFYING VME KERMIT

===============================================

This chapter describes how you can get hold of VME KERMIT and build

it  from  the  S3  and  MTUP  language  sources, or modify an existing

version.

8-1

VME KERMIT User Guide

8.1 OBTAINING VME KERMIT

------------------------

You can obtain the VME KERMIT files by file transfer over the JANET

or  PSS   networks,   or  by  using  KERMIT  itself,  from   Lancaster

University.

You  will eventually have a set of S3 and MTUP source files from which

you  need  to  generate compiled OMF. There are two SCL files: KERMIT,

an interface procedure, and KMT_SCL which is executed to  compile  and

collect your version of VME KERMIT.

8.1.1 The source files

VME  KERMIT  source  is held in a fair number of files and is now

quite  large  (and  still  growing).  The  source  is  split   by

functionality.  The organisation is not fixed, we shall refine it

with experience and as the system changes, so the names  are  not

guaranteed to be unchanged from one release to another.

For version 1.01 and later, the files that make up VME KERMIT are

are as follows:

VME  KERMIT  as  issued consists of four types of file. The first

type consists of the S3 source files:

KMT_DATA_MODULE        Global declarations.

KMT_DH_MODULE          Communications device handling code.

KMT_EH_MODULE          Error handling code.

KMT_FH_MODULE          File handling code.

KMT_MAIN_MODULE        Entry code.

KMT_PH_MODULE          Protocol handling code.

KMT_PP_MODULE          Packet handling code.

KMT_SP_MODULE          Support procedures.

KMT_UI_MODULE          User interface code.

The second type of the file consists of the MTUP source files:

KMT_HELP_MTM           Help message texts.

KMT_SP_MTM             Support procedures message texts.

The  third  type of file consists of SCL files.

KERMIT                 Interface procedure.

KMT_SCL                Compilation and collection SCL.

The fourth type of file consists of the User Guide:

VME_KMIT_DOC           User Guide documentation.

8-2

VME KERMIT User Guide

8.3 BUILDING VME KERMIT FROM SOURCE

-----------------------------------

Prior  to  generating  VME  KERMIT  check  that  the SCL interface

procedure KERMIT meets your requirements. In particular,  ensure  that

the  BOOL  value  ASG  is  set appropriately. It should be set TRUE if

terminal connection to VME is via ASG, but FALSE  if  via  some  other

communications  controller,  eg NIC or CSC. If in doubt set ASG FALSE.

The purpose of this BOOL is to allow  checking  of  control  character

setting  for  START-OF-PACKET  etc.  VME I/O is not transparent to all

control characters. The ASG can handle a superset  of  the  characters

that can be handled by CSC or NIC.

If you wish to change the default VME KERMIT option (see 3.1)  this

can be altered most easily in the SCL interface procedure KERMIT.

Generating VME KERMIT from the S3 and MTUP sources is a fairly easy

process.  Having  transferred  the files to your VME filestore you can

compile them by executing the SCL file KMT_SCL. Edit this file to suit

your VME filestore, then execute the SCL file as follows:

EXSCL(FIL=KMTSRC.KMTSCL)

8.4 CHANGING KERMIT DEFAULTS

----------------------------

If you have the S3 compiler, you can change the defaults by altering

the  values  held  in  the  source  files (in particular the procedure

KMT_SP_SET_DEFAULTS in KMT_SP_MODULE) and re-compiling the source.

8-3

VME KERMIT User Guide

Appendix 1. VME KERMIT capabilities

===================================

Local operation:                 No

Remote operation:                Yes

Transfer text files:             Yes

Transfer binary files:           Yes (prefix only)

Wildcard send:                   No

^X/^X interruption:              Yes (receive only)

Filename collision avoidance:    Yes

Can timeout:                     Yes

8th-bit prefixing:               Yes

Repeat count prefixing:          No

Alternate block checks:          No

Terminal emulation:              No

Communications settings:         No

Transmit BREAK:                  No

IBM communication:               No

Diagnostic logging:              Yes

Session logging:                 No

Raw transmit:                    No

Act as server:                   Yes

Talk to server:                  No

Advanced server commands:        No

Local file management:           No

Handle file attributes:          No

Command/init files:              No

Host commands:                   No

Printer control:                 No

A1-1

VME KERMIT User Guide

Appendix 2. Valid VME control characters

========================================

The control characters that are acceptable to VME depend on the

communications device through which the terminal running Kermit is

routed. The ASG permits a greater range of control characters than

do CSC or NIC.

Character              Input via               Output via

name   dec   hex          ASG     CSC/NIC          ASG     CSC/NIC

NUL    0     00           +         -              +         +

SOH    1     01           +         -              -         -

STX    2     02           +         -              +         +

ETX    3     03           +         -              -         -

EOT    4     04           +         -              -         -

ENQ    5     05           +         -              +         +

ACK    6     06           +         -              +         +

BEL    7     07           +         +              +         +

BS     8     08           +         +              +         +

HT     9     09           +         -              +         +

LF    10     0A           +         -              -         -

VT    11     0B           +         -              +         +

FF    12     0C           +         -              +         +

CR    13     0D           +         -              +         +

SO    14     0E           +         +              +         +

SI    15     0F           +         +              +         +

DLE   16     10           -         -              +         +

DC1   17     11           -         -              +         +

DC2   18     12           -         -              +         +

DC3   19     13           -         -              +         +

DC4   20     14           +         +              +         +

NAK   21     15           +         -              +         +

SYN   22     16           +         -              +         +

ETB   23     17           +         -              +         +

CAN   24     18           +         -              +         +

EM    25     19           +         +              +         +

SUB   26     1A           +         +              +         +

ESC   27     1B           +         +              +         +

FS    28     1C           +         +              +         +

GS    29     1D           +         +              +         +

RS    30     1E           +         +              +         +

US    31     1F           +         +              +         +

DEL  127     7F           +         +              +         +

A2-1

VME KERMIT User Guide

Appendix 3. Running KERMIT over a JNT PAD

=========================================

KERMIT  may  be  ran  over a JNT PAD, but special attention must be

made to the KERMIT configuration parameter settings  and  to  the  PAD

settings.  The  choice of the KERMIT start of packet marker and end of

line terminator will dictate the PAD settings required. Normally,  the

additional  PAD  settings  of  TRANSPARENT  and  NOPRINTMASK  will  be

sufficient. However, the FORWARD and LFINSERT PAD  settings  may  also

require consideration.

A3-1

VME KERMIT Questionaire

Please reply to the following questions and return to:

David Lord,

S.W.U.R.C.C.

The University Of Bath,

Claverton Down,

Bath.

BA2 7AY.

1. Name of organisation:

2. Address of organisation:

3. Specify  the  KERMIT  implementations  with which VME KERMIT is to

operate.

4. List  any  problems  encountered  during  the  installation of VME

KERMIT.

5. Comment on the contents and structure of the User Guide.

VME KERMIT Questionaire

6. Other comments.