MODCOMP MAXIV

KERMIT USER'S MANUAL

by

Richard D. Burke

Robert A. Borgeson

of

SETPOINT Inc.

10245 Brecksville Rd.

Brecksville, Ohio 44141

KERMIT is a copyrighted protocol of Columbia Univ. The authors of

this version hereby grant permission to copy this document

provided that it is not used for any explicitly commercial

purpose and that proper credit be given. SETPOINT Inc. makes no

warranty whatsoever regarding the accuracy of this document or

the software described within and will assume no liability

resulting from its use.

TABLE OF CONTENTS

________

Page No.

Introduction     ..........................    I.1

Overview         ..........................   II.1

KERMIT commands  ..........................  III.1

HELP            ..........................  III.1

STATUS          ..........................  III.1

SET             ..........................  III.1

DELAY          ..........................  III.2

EOL            ..........................  III.2

ESCAPE         ..........................  III.2

MYQUOTE        ..........................  III.2

PACKET         ..........................  III.2

SOH            ..........................  III.2

USL            ..........................  III.3

QUIT / EXIT      ..........................  III.3

SEND             ..........................  III.3

RECEIVE          ..........................  III.3

Technical

Appendix       ..........................    A.1

I.1

A.   INTRODUCTION

KERMIT provides an easy, reliable way to transfer text files

between a MODCOMP Classic running under MAXIV and other machines

which have a KERMIT implementation on them. KERMIT is a

packet-oriented telecommunications protocol copyright by Columbia

University. Documentation available from them thoroughly covers

the formal details of the protocol.(1) The purpose of this manual

is to detail the specific features of this implementation and to

provide instruction on how to use KERMIT on a MODCOMP. This

manual assumes that the reader has a general knowledge of a

MODCOMP running under MAXIV. A Technical Appendix follows which

details how to allocate resources and install KERMIT on your

machine.

MAXIV KERMIT is adapted from a version written by John Lee

of RCA Labs for an HP-1000. It is implemented in Fortran IV

except for one subroutine in assembler.

(1)        KERMIT Distribution

Columbia University Center for Computing Activities

Seventh Floor, Watson Laboratory

612 West 115th Street

New York, N.Y.  10025

II.1

II.  OVERVIEW

MAXIV KERMIT is run by activating a job control procedure

which makes the necessary logical assignments and then, executes

the program. Each site will probably wish to create their own job

control for this, taking into account the requirements addressed

in the Technical Appendix. The original MAXIV KERMIT distribution

contains a simple command file which may be used as a guide. The

file's name is KERMIV. The syntax for activating this procedure

is:

$KERMIV co,oc

where  co   -  logical assignment for file transfer output

(Default = CO)

oc   -  logical assignment for file transfer input

(Default = OC )

Operator input is always from OC and informational output is to

CO. Therefore, if no parameters are specified, KERMIT will expect

operator input and file transfer to occur at the same ports.

MAXIV KERMIT may be run in either remote or local mode. In

remote mode, the data transfer occurs at the same terminal port

which is controlling the program. Presumably, the user is

accessing the MODCOMP from some computer which has the ability to

emulate a terminal as part of a communications package. Local

mode implies that KERMIT is controlled by a terminal which is on

an I/O port different from the port assigned to the data

transfer. This is most likely to be used when neither machine has

a communications software package. When MAXIV KERMIT is initially

run, it senses the current logical assignments and chooses the

appropriate mode automatically. In the initial release ( Version

1.0 ), it is not possible to alter the file transfer port or the

local/remote mode once KERMIT is invoked.

MAXIV KERMIT is designed with a command structure that

reflects MAXIV's division of files into libraries. KERMIT is

always logically assigned to a directorized source library. (by

default, USL) If the user does not intervene, all files to be

sent must be cataloged in USL. All received files will by default

be cataloged to USL. The methods to modify these defaults are

discussed later.

KERMIT's choice of <CTL-A> (ASCII 01) as the start-of-packet

character presents special problems under MAXIV since this is the

standard break character for getting the TMP prompt. Short of

requiring that the break character be redefined in the SYSGEN,

there is no bulletproof way to always pass the <CTL-A> through to

KERMIT without waking up TMP.

II.2

The user must be aware that all input including the <CTL-A>s

will be passed through into a 132 byte buffer. This is sufficient

for at least one packet. However, if the other KERMIT sends a

number of unexpected packets it could fill up the buffer. One

possible scenario that could cause this is when the other KERMIT

begins to send initialization packets before MAXIV KERMIT is told

to receive. If this buffer is ever filled up an outstanding read

request is terminated and the next <CTL-A> received will be

interpreted as the break character. In this case, the user has no

choice other than to exit KERMIT and start over. In local mode,

the buffer is activated immediately upon execution but, no check

for incoming packet data is made until MAXIV KERMIT is told to

SEND or RECEIVE. In remote mode, it is activated as soon as the

SEND or RECEIVE command is entered. Once the initial connection

is made, the buffer is checked periodically for packet data and

refreshed after each packet so that it should not overflow. It is

important to remember that this checking only occurs in SEND and

RECEIVE mode. At other times the user must exercise discretion

regarding sending unexpected packets.

The conclusion is that MAXIV KERMIT is more sensitive to the

initial connection sequence than many other KERMITs. The

following general principles may be helpful:

Local mode

Invoke MAXIV KERMIT first. In the case of MODCOMP to

MODCOMP transfer start the receiving KERMIT first.

Remote mode

Issue the command to the remote machine first.

III.1

III. MAXIV KERMIT COMMANDS

After executing KERMIT, the prompt "MAXIV KERMIT>" will

appear. The user may enter the following commands:

HELP

STATUS

SET

EXIT / QUIT

SEND

RECEIVE

CONNECT

These commands may or may not require additional parameters. In

all cases, the parameters must be separated by spaces. The

commands and parameters may be abbreviated so long as they may be

uniquely identified. For example, HELP may be shortened to H, but

SEND can only be shortened to SEN so that it may be distinguished

from SET. The CONNECT command is recognized but the function is

not supported in Version 1.0 . The other commands are discussed

below.

HELP

The command HELP prints out a description of each

available command to the terminal.

STATUS

The command STATUS prints out the current values for a

number of user-definable parameters.

SET

The command SET allows the user to change various

parameters according to your current needs. The syntax

of the command is:

SET option value

III.2

The options and their applicable values are:

SET DELAY x

This command controls how long KERMIT waits before

sending an initialization packet. The values 1 -

60 seconds are acceptable.  The default is 10

seconds.

SET EOL x

This command sets the end-of-line character which

this KERMIT is expecting to terminate each

received packet. This value does not have to match

the EOL expected by the other KERMIT. The values

1 - 31 are acceptable with the default being 13

(<CR> ).

SET ESCAPE x

This command sets the character which terminates

CONNECT mode. Since CONNECT is not currently

supported, this command has no operational effect.

SET MYQUOTE x

This command sets the value for the character

which prefixes all ASCII translated control

characters. The valid values are 33 - 126. The

default is 35 (<#>).

SET PACKET x

This command sets the maximum packet length we

want to receive.  The valid values are 31 - 94

characters. The default is 90.

SET SOH x

This command sets the start-of-packet character.

The acceptable values are 1 - 31. The default

value is 1 (<CTL-A>). This is not a parameter

which is adjusted by the initial exchange.

Therefore, both KERMIT's must agree before the

connection is made.

III.3

SET USL usl

This command changes the library which KERMIT is

logically pointed at. This library must be a SED

directorized partition or dataset. The default is

USL.

QUIT / EXIT

Either command will end this KERMIT session. If any files

were received, the user will then be prompted to catalog them

away properly.

SEND

This command has two forms:

SEND file_id

SEND @usl

The first command will send a single file with the specified

name. The file must be cataloged in the current USL. (see the SET

USL command) The second command will send every file in the

specified library. This library is not required to be the same as

the current source library. It may be a library in a scratch

partition. This allows the operator to create a directorized

library in scratch and catalog files from several libraries to be

sent as a group.

RECEIVE

When the command RECEIVE is issued, MAXIV KERMIT will

receive as many files as are sent until the sender KERMIT issues

an end-of-transmission. The files are placed in a scratch

partition to be cataloged at the end of the session. When MAXIV

KERMIT is EXITed, the user is prompted to name and catalog the

received files. The answers to these questions are used to create

a Job Control procedure which places the files in the desired

libraries. The default file name is the received file name

truncated to 8 characters with any non-cancodeable characters

replaced by "$". The user may elect to  accept all the default

names. In this case, you will be asked whether all the files

should be CATaloged or RECATaloged to the current source library.

If all the defaults are not accepted, you will be prompted to

name each file, specify the library to use, and whether to CAT or

RECAT. When the questions have all been answered, the command

procedure is automatically invoked to perform the desired file

operations.

III.4

In the event that a file name conflict causes the cataloging

operation to fail, all is not lost! The command procedure is in

KE9 under the name "STORE" and can be edited to correct the

problem. Then reinvoke the procedure to complete the operation.

Care must be exercised to preserve the received text while the

procedure is being modified.

A.1

TECHNICAL APPENDIX

___________________________

A.  Building KERMIT from Source

The KERMIT source files are intended to reside in a single,

directorized SED USL library.  Procedures are provided to

compile, assemble, and link all modules and to create KERMIT as a

load module overlay.  All subroutines should be compiled to a

single, directorized object library.  This library will then be

referenced to link the main program.  The procedure which builds

KERMIT is called BLDKER and it requires 3 arguments.  BLDKER

could be invoked as follows:

$ASSIGN JC=KER,UJC=KER

$BLDKER KER,KUL,BM

where -   KER is the source library containing KERMIT

source,

KUL is a directorized object library to contain

object versions of all KERMIT subroutines,

BM is the load module library to contain the

KERMIT overlay.

The fourth argument to BLDKER is an optional switch which

tells whether or not to generate listing output.  For example, to

generate hardcopy listings during the build invoke BLDKER as

follows:

$ASSIGN JC=KER,UJC=KER,LO=LP

$BLDKER KER,KUL,BM,LO

__________________________________

B.  Logical file assignments and usage

KERMIT requires several logical files which must be assigned

to I/O channels and scratch disk partitions.  They are as

follows:

KE1 - Terminal output

KE2 - Terminal input

KE3 - KERMIT data output

KE4 - KERMIT data input

KE5 - List of file names received during the current session

KE8 - Received text files

KE9 - List of file names to be sent to the remote KERMIT,

and Job Control procedure to catalog received files

away

KEH - Directorized source library containing HELPFILE

A.2

One or two full duplex or half duplex asynchronous I/O ports

may be used for KERMIT.  If KE2 and KE4 are assigned to the same

device (port) then remote operation is assumed.  If they are

assigned to different ports then KERMIT is set up for local

operation.

As files are received, the text is written to the disk

partition to which KE8 is assigned, and the name of the file is

written to KE9. When the user EXITs, several questions are asked

concerning the disposition of the files.  The file name list in

KE9 is then used as the basis for creating a Job Control

procedure to catalog all the received files to directorized

libraries.

KE9 is also used to contain a list of file names to be

transmitted to the remote KERMIT.  This file will contain either

a single name, specified by the "SEND filename" command or the

names of all the files in a directory if the "SEND @usl" command

is used.

For most applications, the disk resources of a BATxxx task

are adequate for KERMIT.  The procedure KERMIV which is provided

as a template uses SCA for keeping received files, SCB for the

received file list, and SC for the sending file list and Job

Control procedure.  KERMIV also provides the capability of

designating the logical device(s) to be used for KERMIT file

transfer input and output.

__________________________________________

C. Control-A collision between KERMIT and TMP

Many systems use Control-A as the TMP attention character.

It is also used by KERMIT as the start-of-packet character.  This

causes problems if the asynch port used for KERMIT transfers is

on the MAX IV break list.

This is only a problem when KERMIT is operating in local

mode. When KERMIT is invoked in local mode, a non-standard binary

read is immediately issued to intercept any unsolicited packets

which could cause a BREAK.  However if the number of unsolicited

characters exceeds the size of the read (132 bytes) the read will

complete and the next control-A will cause a BREAK.  To avoid

this anomaly it is necessary to initiate the SEND and RECEIVE

operations at roughly the same time.  The user may want to

customize KERMIT by increasing the size of the incoming data

buffers, but must take note that only the first packet in the

buffer is interpreted.

A.3

_______________________________

D.  Warning about SEND @usl command

This command causes the specified directory to be read and

the names of all active file entries to be extracted.  This logic

will only work correctly for 128 word-per-sector disks.  Systems

with sector sizes different from 128 words should avoid this

command.

________________________________________

E.  BATxxx task resources required by KERMIT

The number of concurrent I/O operations and the number of

vacant logical file assignments for Batch tasks which will be

running KERMIT may have to be increased.  3 I/O operations and 15

vacant logfiles are adequate for KERMIT's use.

The partitions assigned for scratch use for KERMIT must be

large enough to hold the data that KERMIT will temporarily store

there. The critical file is KE8 which holds the received text

files. Received files are written in compressed format so as to

conserve disk resources, but the user must still make sure that

KE8, or any other disk partitions are not overfilled.

________________________________

F.  Communications port conditioning

The configuration (baud rate, frame size, parity, and stop

bits) of the I/O port used for KERMIT file transfers must be set

up to match the parameters of the remote KERMIT before the MAX IV

KERMIT is invoked.  Modification of those parameters is not

supported in this release of KERMIT.

___________________

G.  Future Enhancements

This version of KERMIT implements only some of the basic

features of a KERMIT program.  Time permitting several additions

are planned. Those include:

1)  SET LINE and CONNECT commands to allow going from

local to remote operation after KERMIT is invoked

and to support a dumb terminal emulation.

2)  SERVER commands to simplify the transfer of files

to KERMIT versions which support server commands.

3)  TAKE command which allows command sequences to be

stored in an external disk file and invoked from

inside KERMIT.

4)  Use of the SET command to modify the KERMIT I/O

port parameters such as baud rate, parity, frame

size, or stop bits.

A.4

5)  A STATISTICS feature which would summarize

information about the current session and the

current transaction, including bytes transferred,

packets transferred, and effective transfer rate.

#