NPS Modifications for KERMIT Installation

The primary communication requirements for KERMIT operation

are that async lines on the DATANET pass all printable ASCII

characters (as spececified in the KERMIT protocol manual) and

the CTL-A (SOH) character be passed to Time Sharing.  As released

NPS does not support these requirements, therefore modifications

must be made prior to KERMIT installation.  NPS modules NHMP and

NTAB must be modified to change the character delete from the

@ character.  Most sites will use the backspace character.  The

NPS Software Release Bulletin addresses the required changes in

Section 18 for the NT4 release.  In the base release the CTL-A (SOH)

is used as a connect character for a private DAC line.  The SOH

must be replaced with an alternate control character (ie: CTL-T).

This modification is made in the module NHMP.  The code to be

changed resides in the DACSOH locmop.  Any site customization

which could effect transparency of any ASCII characters or the

CTL-A character through the DATANET should be reworked prior

KERMIT installation.

DAVE HARSHBARGER

HONEYWELL-FSD (202) 696-6016