P2DOS User's Guide

Release Version 2.3

15 February 1988

After extracting and uncrunching this library, in addition to this file,

you should have the following files:

---> DATE    .COM     CRC  =  1D 32	Displays the current date

---> DATE    .MAC     CRC  =  E6 E4	Source for DATE.COM

---> DDIR    .COM     CRC  =  C7 45	Lists directory with time stamps

---> P2DOS   .MAC     CRC  =  BF 2A	First of three P2DOS source files

---> P2DOS1  .MAC     CRC  =  31 B6	Second of three P2DOS source files

---> P2DOS2  .MAC     CRC  =  10 CD	Third of three P2DOS source files

---> INITDIR .COM     CRC  =  74 E1	Initializes directory for time stamps

---> PUBLIC  .COM     CRC  =  1B 71	Sets/resets the public file bit

---> PUBLIC  .DOC     CRC  =  26 00	Documents PUBLIC.COM

---> PUBLIC  .SRC     CRC  =  8E C3	Turbo Pascal source for PUBLIC.COM

---> SET     .COM     CRC  =  04 44	Displays file/disk statistics

---> TOUCH   .COM     CRC  =  9A DF	Updates a file's time stamp

Installation Procedure:

1.  Extract all files from P2DOS23.LBR onto a blank disk in drive B.

Uncrunch all type *.?z? files and erase the crunched versions.

2.  Create a disk file containing your CP/M or ZCPR3 system image. Do

this by running SYSGEN as follows:

A>sysgen

SYSGEN VER 2.2

SOURCE DRIVE NAME (OR RETURN TO SKIP)a

SOURCE ON A, THEN TYPE RETURN<ret>

FUNCTION COMPLETE

DESTINATION DRIVE NAME (OR RETURN TO REBOOT)b

FUNCTION COMPLETE

This step serves two very important functions. First, the system

boot tracks on the disk in the B: drive are initialized. Second,

a complete image of the contents of the system tracks is left

in memory. Save the system track image for later use:

A>save x cpm.bin

x is size of your CP/M system in 256-byte "pages". If you are not

sure what this number is, try 50.

One important note; SYSGEN is a hardware/system dependent program.

Not to worry if the text above does not match your screen exactly.

3.  Find the ultimate location of P2DOS and the point in CPM.BIN

where the P2DOS image is stored by using ZSID (or SID or DDT). Do

the following:

A>zsid cpm.bin

ZSID VERS 1.4

NEXT  PC  END

3300 0100 BCFF

#d0

0000: C3 03 ED BC 00 C3 00 BD .......

(There will be 12 lines printed at this point, but

the 0000 line contains what is important. The second

and third byte contain the location of the BIOS

warm boot entry point, which is exactly 0E03H above the

start of the BDOS. The BDOS jump no longer contains

the BDOS address because ZSID is now intercepting

BDOS calls. Now we will find the start of the BDOS and

your serial number.)

#ded03-e03

(ZSID will perform the subtraction for you, DDT will

not. In any case, the start of the BDOS will now be

displayed.)

DF00: SE RI AL NU MB ER C3 16 DF 94 E2 95 E2 AE E2 9A

(Again, there will be more lines, but the first line

contains all the important information. First, the

address on the left is the ultimate destination

of your P2DOS, the value for the DOSSTRT equate in

the source code. Second, the next six bytes are your

serial number. Note both DOSSTRT and your serial number

so you can include them in the source code. Now to

find the location of the BDOS image in CPM.BIN.)

#d1280

(Again, several lines will be displayed. Find the

one that exactly matches the line containing your

serial number. If it is not on the screen, repeat

the display command until you find a match.)

#d

12F0: 46 75 .....

1300: SE RI AL NU MB ER C3 16 DF 94 E2 95 E2 AE E2 9A

1310: E2 00 ....

(You may have to look around 1300H, but eventually,

you will find a line identical to the start of

the BDOS. Always use an address ending in 0 for the

display command. When you find the matching line,

the address at the left will be the point in CPM.BIN

where the image of the BDOS starts. Note this address,

which we'll call "image_location". We'll need this

address in step 6. You are temporarily done with ZSID.)

4.  Now edit the source code to customize it to your tastes. The

file P2DOS.MAC contains all the customizing selections. The

other two files, P2DOS1.MAC and P2DOS2.MAC, do not need to

be changed unless you have special requirements (such as a

special location for the BDOS stack.)

Enter the start of P2DOS and the serial number into the source

code. Modify the assembly time options to suit your tastes

and assemble P2DOS.MAC to create a P2DOS.HEX file. The source

files are set up for M80 because it is one of the more difficult

assemblers to generate a proper .HEX file. For M80, the steps are:

A>b:

B>a:m80 =p2dos

B>a:l80 p2dos,p2dos/x/n/e

B>a:

Different assemblers will have different methods of generating

the P2DOS.HEX file, but all assemblers should be capable of making

such a file. Your will have to change the .Z80, ORG, ASEG,

.PHASE, and $INCLUDE statements to match the requirements of your

assembler. Many assemblers will just require the ORG statement set

to the ultimate P2DOS location.

5.  Examine the P2DOS.HEX file and note two things on the first line:

the serial number and the load address of the serial number.

For M80, the load address was specified in the ORG statement in the

source code (100H). If you changed the org to match DOSSTRT

because that is what your assembler requires, then the DOSSTRT

address should appear on the first line. Note this load address,

which we'll call "linker_load_address", for use in step 6. For

example, if the .HEX file contains:

:20addr00serialnumberC316DF92E293E2ACE2......

:20012000AF327CEC327DECED738FE......

:2001400021ECFE29D02151.....

:

:

then you can see the load address in the 3rd through 6th digit and

the serial number in the 9th through 20th digit of the first line.

If you don't find your serial number on the first line, then the

assembler/linker didn't make a proper, useable hex file. Find out

what went wrong and make a new .HEX file.

6.  Combine CPM.BIN and P2DOS.HEX using ZSID (or SID or DDT) as follows:

A>zsid cpm.bin

#>f<image_location> <image_location>+dff 0

(This zero-fills the old BDOS location. Check that

it worked by displaying the BDOS image. image_location

is from step 3.)

#d<image_location>

<image_location>: 00 00 00 00 ...

(Here we are using the image location found in step 3.

Now move in the new image.)

#ip2dos.hex

#r<image_location>-<linker_load_address>

(You may have to perform the subtraction yourself if you

are not using ZSID. linker_load_address is from step 5.)

#d<image_location>

<image_location>: SE RI AL NU MB ER C3 ....

(This completes the merge of the two files. Now save the

new operating system.)

#^c

A>save x p2dos.bin

(Here x is the same value you used to make CPM.BIN.)

7.  You now have a copy of P2DOS in a file called P2DOS.BIN. Install it

on a few test disks by typing:

A>zsid p2dos.bin

#^c

A>sysgen

SYSGEN 2.2

SOURCE DRIVE NAME (OR RETURN TO SKIP):<ret>

DESTINATION DRIVE NAME (RETURN TO REBOOT):b

FUNCTION COMPLETE

Please note that SYSGEN is a system dependent program. The text

you see may not match exactly the text given here, but the

questions and answers should be the same.

8.  Warm boot your system with P2DOS in A. If it boots correctly,

continue testing by copying files, running programs, until you are

satisfied that it works. Try this: Warm boot with P2DOS, PIP, and

a test file in A. Change the disk in B without hitting Control-C.

PIP the test file to B. No "Bdos Err on B: R/O"!

If your system hangs or goes completely nuts upon warm booting with

P2DOS, repeat the installation again.

USAGE

-----Changing Disks Without Control-C

P2DOS should work just like your old BDOS, quietly and out of your way.

It has some additional features which make it much nicer than your old BDOS.

For example, let's say you're writing the "Great American Novel" with

WordStar. It's late, you've written all day, and you want to move on to

other things. You hit ^K^D to save your work, and WordStar burps:

**Disk Full. Press <ESC>**

With your old BDOS, if you switched disks, you'd get

Bdos Err on A: R/O

and you'd say bye-bye to Chapter 5 unless you had a text recovery program.

With P2DOS, move to the beginning of file using ^R, not ^Q^R, and mark the

beginning of a block. Move to the end of file using ^C, not ^Q^C, and mark

the end of block. Hit ^K^W to write the block to disk. Enter a filename on a

drive not containing your novel (probably your WordStar disk). Remove the

disk from that drive, and insert a blank disk. Hit return, and your work will

be saved to a new disk. Finally, replace the old disk. Try this with your

old BDOS, and you'd be quite sorry.

P2DOS's "Control-C-less" operation is not limited to Wordstar. You may

change disks at any time, with any program, with one exception. IF A FILE IS

OPEN FOR WRITING, DO NOT CHANGE DISKS UNTIL THE FILE IS PROPERLY CLOSED. IF

YOU DO, YOU WILL TRASH YOUR DISKS!  This is why, in the example above, we

changed the disk containing WordStar, not the disk containing the novel. No

files were open for writing on the Wordstar disk, so it was ok to change it.

The novel file was open, so its disk could not be changed.

-----Public Files

If you use user areas, you'll love this. Public files are files which are

accessible from all user areas. Normally, if Wordstar was in A7: and

your text files in A8:, you could not edit your files with Wordstar. If

Wordstar and its overlays were made public files, you could be logged into

A8:, B6:, or any other user area and still run Wordstar.

Public files are invisible to the use of wildcards, so you may not copy them

with PIP *.*. They must be specified by name. Public files are also

read-only, so they may not be erased.

To make a file PUBLIC, run the PUBLIC program which is included with P2DOS.

Ex: A>public ws.com

To restore a file to normal (make it private),

A>public ws.com x  <---the X makes the file private.

----Error Messages

P2DOS may produce the following error messages:

Disk Error on A : error message

Function xx ; File = FILE.TYP

where the error message may be

Read error

Write error

Non existent drive

Read-Only (this happens only if disk was set to r/o with bdos function 28,

not when disks are changed.)

File is Read-only

The BDOS function number and the file involved (if applicable) are given, so a

full message may be:

Disk Error on A: Write error

Function 22 ; File = CHAPTER.5

In this case, there was a write-error trying to create file CHAPTER.5.

-----Advanced Features

P2DOS has optional built-in time stamp capabilities. To use time stamps, you

must have a real time clock and a bios routine which access the clock. You

must install P2DOS with DOTIME set to TRUE. You must also initialize

the directory for time stamps, using the INITDIR.COM program included

in the library. You will also find a directory display program, DDIR.COM,

that will display the time stamps.

P2DOS can be installed so that functions 13 and 37 allow the free

interchange of single and double sided disks. On many machines, such

as the Kaypro IV, changes between single and double sided disks are

recognized only at warm boots. Obviously, this is a limitation on the

switching of disks without Control-C. If your machine is one of these

and if you have the source to your bios, you can remove this limitation.

Add code to your bios to detect disk formats without warm booting.

Suppose this routine is DSKFMT. Change your BIOS jump table to:

:

:

SECTRN:	JP	TRANS			; existing sector translate

TIME:	JP	DOTIME			; get/set time

SETDSK:	JP	DSKFMT			; determine disk format

This code may not be necessary if your BIOS always knows the proper

disk type. However, if a different disk type is recoginzed only at

warm boot, this routine will need be added to recognize disk type

changes between warm boots.

This feature is extremely useful to have when using disk cataloging programs,

the file utility NSWP207, the library manager NULU 1.5, and many others

with which expect disk changes and call functions 13 and 37 to change

disks.

To my knowledge, all components of the P2DOS.LBR are public domain

for non-commerical use only. P2DOS23 is a modification of P2DOS21,

by H.A.J. Ten Brugge from the Netherlands. It is subject to the original

author's restrictions.

No liability is assumed. The code is beleived to be correct, but use

at your own risk.