K E R M I T - 1 1   f o r   R T - 1 1 / T S X - P L U S

I N S T A L L A T I O N   a n d   R E L E A S E   N O T E S

V  0  3 . 6  2 - 8        2 7 - J U L - 9 3

KRTTSX is  a  special  version  of  Kermit-11 (PDP-11 Kermit written by Brian

Nelson of the University of Toledo) enhanced for use with TSX-Plus.  This was

begun primarily to support its use on a public bulletin board system and then

was expanded to produce a  reliable,  general purpose communications program.

Numerous bugs have been fixed and several  new  routines  added.  All changes

are documented in the edit history in KRTCMD.MAC,  or may be read from within

Kermit by HELP Release-Notes.  Please note not only has KRTTSX's on-line HELP

text been greatly enhanced and expanded, lookups in it now execute at maximum

speed, thus this document is rather short.  Use HELP for operating info.  You

may use HPRINT to generate hard copy of anything needed for handy reference.

For an in-depth discussion of Kermit fundamentals there's no substitute for:

"Kermit, A File Transfer Protocol" by Frank da Cruz, 1987

Digital Press, order number: EY-6705E-DP  (in USA phone: 800 343-8321)

DP ISBN 0-932376-88-6, PH ISBN 0-13-514753-0

This Kermit does not yet offer character set translation, labeled file types,

sliding windows, a script programming language or command macros, but it does

provide everything else one would expect after reading the above.

This program is for use with RT-11 V4.0 or TSX-Plus V5.0 and up, though a few

new features use EMTs added in slightly later releases.   KRTTSX runs what it

can with earlier releases of either RT-11 or TSX.   Debugging was done on TSX

V6.5 and RT-11 V5.5, under which its operation has been thoroughly tested and

is extremely reliable.  All RT-11 V5.6 monitors are also supported, including

separate I&D space.

Thanks for some of the ideas, additions, fixes and above all, bugs eliminated

in this program are due to Tim Clarke, Tom McCoy, Bill Walker, Roger Wallace,

Jim Whitlock and Chester Wilson.  Additionally,  appreciation is owed all who

contributed  to  VMS-Kermit  and C-Kermit, from which many other new features

were derived, and to Brian Nelson, author of the original Kermit-11.  Some of

the most  obscure  yet  serious  bugs  were  discovered by Bill Walker, Roger

Wallace and Jim Whitlock, without  whose  help this Kermit would hardly be as

bullet-proof as it now is.

Billy Youdelman                 youdelman@eisner.decus.org

PO Box 1207                     billy@mix.com

Culver City, California 90232

USA

Installation and Release Notes	KRT V03.62-8			Page Two

T A B L E   O F   C O N T E N T S

---------   ---   ---------------

I N S T A L L A T I O N				Three

K R T . I N I   E x a m p l e s			Five

R T - 1 1   S p e c i f i c   I n f o		Six

A   "K e r m i t   M o d e m"   H a n d l e r	Eight

T S X - P l u s   S p e c i f i c   I n f o	Ten

M O D E M S					Thirteen

Anchor Automation MK24			Fourteen

Telebit T2500/T3000			Fifteen

US Robotics "Courier" 2400/2400e	Seventeen

USER-DEFINED MODEM			Eighteen

SET DIAL Commands		TwentyOne

R E S T R I C T I O N S				TwentyNine

O b t a i n i n g   U p g r a d e s		TwentyNine

F i l e s   R e q u i r e d			Thirty

W h a t ' s   N e w				Appendix A

Installation and Release Notes	KRT V03.62-8			Page Three

I N S T A L L A T I O N

-----------------------

This program will run under RT-11 or TSX-Plus.  The system clock must be

running.  As  distributed  KRT  is built to run on a non-EIS CPU, KRTTSX

is built for EIS processors.  These monitors and handlers are supported:

RT-11SJ   V5.0 + up   TT  KM  XL	; SYSGEN for TIMER support

RT-11FB   V4.0 + up   TT  KM  XL	; SYSGEN for..

RT-11XM   V5.1 + up   TT  KM  XC  XL	;  ..SPCPS support

RT-11..   V5.6 (all)  TT  KM  XC  XL	; including separate I&D space

TSX-Plus  V5.0 + up   TT  CL  KM  XL	; see HELP INSTALL TSX

NOTE:  XL first appeared in RT-11 V5.1.  Prior releases must use either

TT (the console terminal port) or KM (a "Kermit Modem" handler supplied

with this program) for communicating with the outside world.

KRTTSX.SAV uses extended memory overlays for XB,XM,ZB,ZM,TSX and KRT.SAV

low memory overlays for  FB,SB,SJ,TSX  to accommodate small systems.  FB

is the ONLY RT-11 V4 monitor which can run this program.  VBGEXE or VRUN

may be used under mapped RT-11 monitors.  It's best to load any handlers

required first, even for a background task.  KRT's maximum packet-length

is 600., KRTTSX's is 1920., this is determined by the NONEIS conditional

code for MAXLNG in KRTMAC.MAC.

KRTTSX requires 166kb of memory to load, KRT only needs 42kb.  Under TSX

it's 172kb and 48kb for V6.5 or 171kb and 47kb with V6.4.  KRT increases

disk I/O as its overlays are not kept resident.

If you already have some form of Kermit on your system, you may try this

one out by copying to a disk containing no other Kermit files, assigning

that disk DK, deassigning HLP, INI and KRT, and running the new program.

Assuming LD0 is a scratch disk and you have an 8" floppy:

.COP DY0:*.(SAV,HLP,INI) LD0:

.ASS LD0 DK

.DEA HLP

.DEA INI

.DEA KRT

.SET USR SWAP	! if running RT-11

.RU KRTTSX	! use KRT if KRTTSX won't load

To dial out, edit the copy  of  KRT.INI  on LD0 as needed for your site.

It is not necessary to use a command file for such things as attaching a

CL unit to a TSX line before running this Kermit - everything may now be

easily handled during its initialization.  At this point it is suggested

you use the running Kermit to read through its own HELP,  especially the

SET LINE and SET CL topics.  If you opt to use any sort of indirect .COM

file to actually run  KRT/KRTTSX,  please  note that under TSX using the

^>R KRT.. option to force command file input to the running program will

cause the test for TT input during the strike any key abort when dialing

to hang and you won't be able to dial out from the command file.  Kermit

does process indirect command (take) files as well as arguments from the

command line used to run it, either of which may be used instead of ^>R:

Installation and Release Notes	KRT V03.62-8			Page Four

.KRTTSX DIAL TOMMY  ! dials a phone number defined as "tommy"

.KRTTSX TAKE TOMMY  ! runs the indirect command file tommy.com

You may also have to force arguments for the program to it when run from

a command file under TSX:

R KRTTSX ^1 ^2 ^3   ! as needed, up to 6 args

There is an additional problem when forcing command line input to Kermit

in that comment delimiters "!" and ";" are processed as normal chars and

will cause errors.  This allows them to be passed to a remote system for

RSX/VMS version numbers and the like.  They are treated as delimiters in

Kermit's internal command file processing, thus the best procedure is to

dump the "^>R KERMIT" line from such files and treat them as Kermit take

files.  If typing in arguments when starting Kermit is a problem one may

use a separate command file run from the monitor containing only:

R KRTTSX TAKE TOMMY

This allows tommy.com to contain comments without crashing when run.

If it's decided to permanently install this program all old Kermit files

(K*.HLP, K*.INI, K*.SAV, etc) should be backed up then replaced by those

from this package.  Dialing from a command file (if in use) will need to

be treated per the above.

After ensuring RT-11 or TSX requirements are met patch KRT.INI (examples

follow) for your site, then copy these to SY:

KRTTSX.SAV  -  or KRT.SAV, the executable image, rename as desired

KRTHLP.HLP  -  HELP text file, may also be on HLP, KRT or DK

and this to the logical INI, KRT or a default disk of your choice:

KRT.INI  -  initialization command file

When  started  KRT/KRTTSX  will  check  INI,  KRT, DK and SY for KRT.INI.

Keeping it off SY allows others to use a different init file, and permits

the program to be easily run in the remote mode by callers to the system.

Or, one may SET LINE TT to accomplish the same thing.  For multiple users

sharing a group of lines check HELP SET CL before editing KRT.INI.

The HELP text file search (for KRTHLP.HLP) path is HLP, KRT, SY and DK.

KRTTSX.SAV/KRT.SAV  may  be  renamed as is convenient (K11 or KERMIT) but

KRT.INI  and  KRTHLP.HLP  are called (by these names) by Kermit, and thus

must not be renamed.  If the HELP text (KRTHLP.HLP) is altered in any way

Kermit must be rebuilt.  See KRTHLP.PRE for details.

Kermit is then run by its name, and an initial command line may be passed

to it as argument(s) to the KMON command:

.K11		! run Kermit

.K11 DIAL DEC	! run Kermit, dialing number set in init file

Also check the specific info for your operating system if Kermit will not

be the only task running.

Installation and Release Notes	KRT V03.62-8			Page Five

K R T . I N I   E x a m p l e s

- - - - - - -   - - - - - - - -

! Multiple users under TSX-Plus with possibly assorted modems

! This seeks the first available line/port from those specified

! May be installed on SY to be shared by all users, see HELP SET CL

SET CL PORTS 9/19200/T25/3 11/2400/H/6 8/2400/H/4

SET CL UNITS 2 1 0

SET CL PRIORITY 71	! don't drop chars at high speeds

SET CL LINE *	! search for first available combination of the above

! For efficiency's sake, the above may be reduced to:

!SET CL POR 9/19200/T25/3 11/2400/H/6 8/2400/H/4,UNI 2 1 0,PRI 71,LIN *

SET VLSWCH REMOTE	! emulate SET HOST/PORT if needed

... other items as required ...

! Single user under TSX-Plus

SET CL0 LINE 9		! includes "SET LINE CL0," unneeded by PRO-series

SET SPE 2400		! CL or KM only, others must do from monitor

SET PHO XMODE 4		! enable CONNECT/speed messages for fall back

SET MOD HAYES

...

! Generic RT-11 ..

SET LINE KM		! to call out..

SET FLOW-CONTROL RTS	! if preferred, must be enabled in hardware too

SET MOD HAYES

...

! System-independent items

SET LONG ON		! enable long packets

SET SL ON,KED		! turn on SL editor, enable its KED mode

SET PHONE TONE

SET PROMPT "Kermit-11> "

SET PHO NUM DECUS 1 800 521-8950	! Username: INFORMATION for info..

By default init file commands and informational responses to them are not

echoed to the screen.  After the init completes the default terminal mode

becomes NOQUIET, unless set in said file, which overrides these defaults.

If you wish to display the entire init process, make SET TT NOQ the first

line in KRT.INI.  A file may be tested by running it as a take file:

Kermit-11>@dev:KRT.INI

SHOW FILE to check the name of the init file Kermit-11 used at start-up.

Installation and Release Notes	KRT V03.62-8			Page Six

R T - 1 1

---------

The RT-11 FB,XB,XM,ZB,ZM monitors must include .SPCPS support if you want

^C to abort anything in the program without actually stopping the program

itself.  If KRT is not the only job running it should be FRUNed to ensure

it will be able to process incoming data from the remote system.  RT-11SB

and SJ must be sysgenned for .mrkt and .twait (TIMER) support.

If you are using RT-11 V4, the ONLY monitor which can run KRT is RT-11FB.

SET USR SWAP if it is not already there, this program needs the space.

RT-11 multi-terminal support has been pulled to keep the size reasonable.

Only a handler, or the controlling terminal line, may be used to exchange

files with the outside world.  Use XL with RT-11, XC with PRO/RT.  KM may

be used with either, it adds speed fallback and support for TSX functions

that make life with Kermit more enjoyable.

As distributed KRTTSX has just enough buffer to load the KM handler built

for a DLV11-E in a PDP-11 system.  This may be altered by the extend size

(octal number of bytes) specified in KRTLNK.COM:

krttsx,krttsx=krtxm/t/e:3010//	! APR0, the root   extending HNBUFF

WARNING:   Because it's impossible to bomb an outstanding read completion

routine once KM/XC/XL has been assigned the link the only way to deassign

it is to exit Kermit.

WARNING:   Any SET from KMON on KM/XC/XL while its interrupts are enabled

will OVERWRITE THE QUEUE and hose any data then in it.

Note XC/XL is NOT an eight-bit device thus Kermit's parity will be forced

to SPACE under RT-11.  If you have hacked it into handling 8-bits you may

SET PARITY NONE either in the init file or from the keyboard after Kermit

is running and AFTER you have SET LINE XL or XC.

HANGUP (DTR control) isn't available unless XL is from RT-11 V5.2 and up,

or the KM handler is used.

To use a handler, you must have an DL-11/DLV-11 interface (in addition to

the console interface).  The XL handler supports two DCL commands:

.SET XL CSR=n

.SET XL VECTOR=m

Where "n" is the address of the CSR (control status register)  and "m" is

the  interrupt  vector address.  The defaults are 176500 for the CSR, and

300 for the interrupt vector.  The DLV-11 speed is set with wire wrapping

and it's not settable on the XL handler.  If you have a DLV-11E XL may be

built with XL$DVE=1 and the speed set from KMON as with XC.

Installation and Release Notes	KRT V03.62-8			Page Seven

The XC handler is a special version of XL for the PRO/300 series, its CSR

is 173300 and vector is 210(8).  You must SET XC SPEED=nn from KMON prior

to running  Kermit  to  change XC's 1200 baud default.  You may also need

to UNLOAD XC before Kermit can be FRUN:

.UNLOAD XC

.FRUN KRTTSX.SAV/PAUSE

.LOAD XC=KRTTSX

.RESUME

Also, when running in the foreground

Kermit-11>SET TT QUIET

allows one to work in the background during file transfers.

To operate properly at high speeds (2400 and up) and with Kermits

not supporting XOFF flow control the XL or XC device handler MUST

be modified to increase the internal buffer size and repress XOFF

transmission within a packet.  This is very easy to do.

First copy XM.MAC, XL.MAC, XM.ANS, SYSGEN.COM and for the PRO/300

series XC.MAC from the RT-11  distribution  to DK, ASSIGN DK SRC,

and then in the copy of the file XL.MAC change the line:

from:	BUFSIZ	= 64.

to:	BUFSIZ	= 256.

Then IND SYSGEN and tell it to use the saved answer file XM.ANS.

When SYSGEN exits, it  will  have created, among other things, a

file called XM.CND which is needed for the driver assembly.  For

the PRO/300 series substitute XC for XL in the following command

procedure to build the handler:

.COPY SY:XLX.SYS SY:XLX.OLD		! save a copy

.REN/NOPRO SY:XLX.SYS SY:XLX.SYS	! unprotect file

.UNLOAD XL				! unload handler

.REMOVE XL				! and remove it

.MACRO/OBJ:XLX (XM,XM.CND,XL)		! assemble new

.LINK/EXE:SY:XLX.SYS XLX		! RT V4, handler on SY

.LINK/NOBIT/EXE:SY:XLX.SYS XLX		! if RT V5, do this

.INSTALL XL				! install it and

.LOAD XL				! load into memory

Last, but not least, if there is no way to get an  additional  interface

into your system (perhaps you have a four slot Q-bus backplane), you can

force Kermit to use the console.  This implies, of course, that it won't

be possible to dial out from the RT-11 system - it may only be used with

the remote Kermit connected via the console port.  Kermit-11 defaults to

TT as its link device and sets the packet length to 60 bytes.  Otherwise

the command

Kermit-11>SET LINE TT

does the same thing.  You may increase the packet size if larger packets

will work, or modify RT-11's default SPACE parity, AFTER the SET LINE is

done.

Installation and Release Notes	KRT V03.62-8			Page Eight

The KM Handler

KM is a "Kermit Modem" communications handler provided to support

TSX-Plus features not otherwise available under RT-11, as well as

older releases of TSX that preceded its CL handler.   KM combines

the best of both worlds, including the following under RT-11:

o  Works with DL(V), Falcon and PRO serial interfaces

o  Speed may be SET and SHOWn from within Kermit

o  Automatic fallback to the connected speed

o  An eight-bit data path

o  Selectable hardware (RTS/CTS) flow control

o  Emulation of TSX's CLCLR (flush pending I/O)

o  DTR toggling to support Kermit's HANGUP command

o  Usable with RT-11 V4.0 and up, and TSX-Plus V5.0 and up

To use KM you must first edit the appropriate conditional assembly

files for the serial line interface in your system:

KRTSJ .CND  -  for RT-11FB, RT-11SB, and RT-11SJ

KRTXM .CND  -  for RT-11XB, RT-11XM, RT-11ZB and RT-11ZM

KRTTSX.CND  -  for TSX-Plus

Then assemble and link using KRTSYS.COM if being done under RT-11

V5.0 or above, or KRTSV4.COM if using any RT-11 V4 release.  This

will create KM.SYS, KMX.SYS and KM.TSX, which should be copied to

your system device (SY) where they may be INSTALLed and LOADed as

needed.  Note that KRTKM.MAC (the handler source file) must be on

the default disk for this assembly.  Under TSX you'll have to use

TSXMOD to add a definition for KM as shown below, or define it in

TSGEN.MAC and rebuild the system.  In either case you must reboot

as it only loads handlers when started.  KM may be mapped to save

low memory:

DEVDEF	<KM>,MAPH

To build KM under RT-11 V5 (KRTSYS.COM):

MAC/OBJ:KM KRTSJ.CND+KRTKM

LINK/NOBIT/EXE:KM.SYS KM

MAC/OBJ:KMX KRTXM.CND+KRTKM

LINK/NOBIT/EXE:KMX.SYS KMX

MAC/OBJ:KMTSX KRTTSX.CND+KRTKM

LINK/NOBIT/EXE:KM.TSX KMTSX

To build KM under RT-11 V4 (KRTSV4.COM):

MAC/OBJ:KM KRTSJ.CND+KRTKM	! KRT will only run under RT-11FB if V4

LINK/EXE:KM.SYS KM

MAC/OBJ:KMTSX KMTSX.CND+KRTKM

LINK/EXE:KM.TSX KMTSX

NOTE:  Comments must be stripped to run these command files under RT-11.

Installation and Release Notes	KRT V03.62-8			Page Nine

KM supports the following SET commands from the monitor (KMON):

SET KM CSR=octal-address	! except on the PRO for

SET KM VECTOR=octal-address	! which these are fixed

Hardware flow control REQUIRES pins 4 and 5 be carried through in

the modem cable, 4 to 4 (RTS) and 5 to 5 (CTS), DTE (the port) to

DCE (the modem), in addition to those otherwise normally present.

The port should be set up for 8 data bits and NO parity as Kermit

does parity in software.

WARNING:  If the interface provides selectable interrupt priority

and your max speed will be greater than 4800 you will likely have

to use BIRQ 5 to avoid dropping chars.

NOTE:  This handler does NOT support VTCOM, nor is it intended to

be used with anything other than KRT V03.62 or future releases.

WARNING:  Because it's  impossible  to  bomb  an outstanding read

completion routine once KM has been assigned the  link  the  only

way to deassign KM is to exit Kermit.

See the top of KRTKM.MAC for detailed installation instructions.

Installation and Release Notes	KRT V03.62-8			Page Ten

T S X - P l u s

---------------

One may communicate via TT when calling in,  and  use CL0 to CL7, KM or XL

when calling out.  The CL handler is suggested as it's more efficient with

TSX-Plus.  See HELP SET CL for details.   On PRO/300 series systems Kermit

will try to assign CL0 or CL1 to TSX Line #3 at startup.

If there is any chance of swapping on your system use TSX's MEMLOCK option

when running the program.  This is easily done via the INSTALL facility.

WARNING:  If you build XL with XL$DVE (set speed) enabled be advised under

TSX-Plus (as of V6.5) the KMON command SET XL SPEED=nnnn will modify the

hardware port which XL is serving instead of the handler itself and thus

fail to produce the expected result.  And regardless, if you must use XL

see HELP INSTALL RT-11 regarding this handler's limitations.  Mapping it

is recommended to save low memory.

KRTTSX uses  208.  blocks  in the  TSX  PLAS  swap file.  The TSGEN/TSXMOD

parameter  determining  its size is SEGBLK.  If modified, TSX must then be

restarted to create the new swap file.

In the CONNECT mode 11.  activation  characters  are declared so the TSGEN

parameter MXSPAC must be => 11.

Under TSX one must SET IO ABORT so errors won't cause waits for the I/O to

rundown.  In  TSGEN  this may be accomplished by making IOABT = 1.  It may

also be done as a keyboard command.

XOFF operation without dropping characters in the CONNECT mode at 2400 and

up may require enlarging TSX's silo and input buffer on lines to which the

CL units are attached.

BUFSIZ	1024.,1024.

SILO	255.,64.,16.

in the LINDEF does the job though this is a bit big if 2400 is the fastest

speed used.  Kermit also can raise its priority when actually talking to a

CL unit so as not to drop chars at high speeds, see HELP SET CL.

The above also applies to file transfers via the controlling terminal line

(TT).   When sending to a TSX system you may also need to set its priority

below any other interactive job and use TSX's /HIGHEFF switch when running

Kermit-11 to avoid excessive degradation of response time for other users.

This is due to the overhead involved in processing input by TSX's terminal

handler, and is not a problem when sending output from a TSX system.  Note

this only concerns I/O via TT - comm handlers (CL, XL, etc) are no problem

in either direction.

Binary file transfers  via  TT  might require you to SET TT 8BIT.  You may

also need to SET TT BITS=8 but this is a hardware function and thus should

ONLY be done from a primary line first, before any subprocess that may run

Kermit is spawned.  Setting this via a subprocess may write garbage in the

line-parameters word.

TERMINAL privilege is required to assign a CL unit to a time-sharing line.

Modem control is an extremely significant system security issue under TSX.

The problem is if a dial-in user's phone connection fails that job must be

Installation and Release Notes	KRT V03.62-8			Page Eleven

killed before a following call can be answered and connected to it, and to

the still-logged-on session of the previous caller.

Beginning with V6.4 TSX was given improved modem control, with which these

"tailgating" incidents are easily prevented.  The following parameters are

suggested for MNP/V.42 auto-answer operation (modem time-out = 40 secs):

.SH MODEM

PHONE=1    Other modem control parameters (in 0.5 sec):

ONTIM=80 OFFTIM=1 TIMIN=80 TIMLOC=1 TIMOUT=1

When no error correction is in use, ONTIM and TIMIN may be halved, and the

modem's wait time for carrier (S7 register in Hayes-style units) set to 20

seconds.  If the modem's wait time is not settable such that it's restored

when reset (ala ATZ), then ONTIM and TIMIN must be set to match the modem.

OFFTIM, TIMLOC and TIMOUT must = 1 (the minimum, don't use zero) to ensure

the job is logged off immediately.  The $PHONE flag must be defined in the

LINDEF (in TSGEN.MAC) for any line connected to an auto-answer modem.

One then includes this in the start-up command files for "phone" lines:

SET ERROR FATAL

!ACCESS SY:CLEAR.COM/READ		! add if other access limited here

SET LOGOFF FILE=SY:CLEAR.COM		! add this..

SET TT 8BIT,BITS=8			! ..and this for Kermit

...					! other necessary items

R LOGON

and CLEAR.COM contains:

SET ERROR NONE

SET PRIO 70				! be sure this runs before next call

SET TT NOPAGE,NODTR,PHONE,AUTO,XON

...					! anything else needed should follow

This may seem a bit extreme on the SET TT end of things, but it definitely

gets the job done, even after in and out buffers are overflowed, XOFF'd in

both directions (deadlocked) and the caller has to hang up.  The rationale

for the various options used above is:

NOPAGE	ensures possible noise induced XOFF on DTR drop is harmless

NODTR	resets the modem

PHONE	ensures port is not in local mode, just in case..

AUTO	ensures port not stuck at any particular speed

XON	resets any XOFF which may have snuck through..

Note that TERMINAL privilege is required for the above to drop DTR.  As of

TSX V6.5 installing a .COM file with TERM privilege doesn't work, the user

must be given it or DTR will not be dropped and the phone line may not get

hung up.

If your version of TSX-Plus is earlier than 6.4 and you must allow dial-in

operation there is a program named NOTAIL which runs as a detached job and

provides the same protection against tailgating.  See KRTAIL.MAC, included

with this Kermit's distribution.

Installation and Release Notes	KRT V03.62-8			Page Twelve

It's also a good idea when using a Hayes-style modem to disable its escape

character means of invoking the modem's command mode.  If one phones in to

a TSX system and then uses another line on it to call out or talks to some

one such that the other person's data are going into the modem (the one at

the TSX system, to which your local system is connected) via its DCE port,

there's no other way to prevent someone from using the escape sequence and

taking control of that modem, after which they've got your phone line..

Installation and Release Notes	KRT V03.62-8			Page Thirteen

M O D E M S

-----------

Generally, connecting a modem requires an appropriate interface such

as a DL-11E, which supports DTR and software speed selection.  It is

possible to get by without such things, however (and particularly on

TSX) this may pose a security risk in that the system has no control

signals with which to log out a disconnected job.

Keeping the above in mind, note that if your modem requires  DTR  to

be present, you must use an interface that asserts it, force it high

internally in the modem, or build a cable to force it high.  Without

an appropriate interface HANGUP will not function and TSX is open to

serious security problems if incoming calls are allowed.

WARNING:  If the interface provides selectable interrupt priority

and your max speed will be greater than 4800 you will likely have

to use BIRQ 5 to avoid dropping chars.  The port should be set up

for 8 data bits and NO parity as Kermit does parity in software.

The modem cable must at a minimum carry through pins 2 and 3 (data),

7 (ground), 8 (DCD) and 20 (DTR).  Under TSX the modem may be set to

answer the line (pin 22, the ring indicator, must also be connected)

then when Kermit is started if a CL unit is attached to the line and

a modem set Kermit retains control of it all until dropping the line

or exiting the program,  when the modem and line will be restored to

their idle (auto-answer) state.

The SET MODEM command is used to establish the type of modem you are

using for the DIAL command.  Enter SET MODEM ? for a current list of

of pre-defined modem types.  See HELP SET DIAL for information about

entering a USER-DEFINED modem structure.

This Kermit is capable of matching the DTE (system) speed to that of

the DCE (modem) in response to a message by the modem (CONNECT nnnn)

only if using the CL or KM handler.  Currently the generic Hayes and

Telebit T2500/T3000 modem definitions support this.  Under XL/XC the

speed can't be changed after Kermit is running, it must be done from

the monitor first otherwise the queue will be lost.  The DCE must be

locked to the same speed, which must be at least the highest desired

carrier speed.  The MNP and V.42 versions of the above support this.

If this is not suitable, a USER-DEFINED modem structure may be used.

Be aware that file transfers FROM this Kermit via a modem who's DTE

speed is greater than the connected speed (with the distant system)

will require XOFF flow control, and servicing the interrupts caused

by it may substantially increase the cpu's loading.  If the maximum

connected speed possible is 2400 or less, dropping the DTE speed to

9600 (when same must be locked for MNP/V.42) is a good idea.  Using

the KM handler's hardware flow control is another option.

NOTE:  This program toggles DTR to reset the modem thus this option

must be enabled as part of the modem's default configuration.

The following examples have been taken from actual daily use with

this program to both dial in and out under TSX-Plus.  Note the S7

register (wait time to connect) is set to 255 when Kermit inits a

modem to dial out so Kermit's SET DIAL TIME-OUT takes precedence.

Installation and Release Notes	KRT V03.62-8			Page Fourteen

Anchor Automation "Signalman Lightning 24" Model MK24:

CONFIGURATION

S00=002    S01=000    S02=128    S03=013  NOTE: S0=2 here because this

S04=010    S05=008    S06=002    S07=020  modem occasionally glitches

S08=002    S09=006    S10=005    S11=040  DCD when answering the first

S12=050    S13=000    S14=044    S15=000  ring, which TSX will catch &

S16=000    S17=000    S18=000    S19=000  then hang up on when it's not

S20=000    S21=016    S22=118    S23=054  there 0.5 sec later..

S24=000    S25=005    S26=000    S27=000

S28=000    S29=000    S30=000    S31=255

S32=001    S33=000    S34=000    S35=001

B=0  Y=0  &M=0   &D=2   &C=1  &J=0  &T=0

X=4  M=1   F=1    E=0    Q=1   V=1

DIAL=TONE    SPEED=2400     PARITY=NONE

The Anchor modem will save all of the above, including its "S" registers

in RAM (backed up with a lithium battery), via AT&W.  Then one need only

send ATZ to restore its auto-answer operation, which this Kermit will do

when exiting.  SET PHONE XMODE 4 for all result codes, and use the HAYES

modem type (SET MODEM HAYES).  Note that because this modem restores its

time-out (S7) when reset it is set to 20 secs for answering which allows

for a more timely response following a failed call.  If using MNP modems

on the same system, S7 should be set to 40 seconds.  If you have the MNP

model of this modem, stick with the HAYES definition as there is not any

provision for locking the DTE speed.

Configuration switches:

1 - as req'd, ON for RJ-12/13 busy support

2 - ON   DCD/DSR follow detected carrier signal

3 - ON   DTR from the computer controls modem (manual says OFF, in error..)

4 - OFF  remote digital loopback

5 - OFF  enables command recognition by modem

6 - OFF  no CTS support

7 - unused

8 - unused

Installation and Release Notes	KRT V03.62-8			Page Fifteen

Telebit T2500/T3000 Modems

Three Telebit T2500 definitions are provided.  T2500 is intended for

use without MNP or V.42 LAP-M error correction.  It uses the modem's

connect messages to match the DTE speed to that of the data path and

that requires using either the CL or KM handler.  Set up the T2500's

default profile in the "enhanced" command mode as shown below:

E0 F1 M0 Q3 T V1 W1 X3 Y0 &P0 &T5     Version GF7.00-T2500SA

S00:001 S01=000 S02:128 S03=013 S04=010 S05=008 S06=002 S07=040 S08=002

S09=006

S10=007 S11:050 S12=050 S18=000 S25=005 S26=000 S38=000

S41=000 S45=000 S47=004 S48:001 S49=000

S50=000 S51=255 S52:002 S54:002 S55=000 S56=017 S57=019 S58=003 S59=000

S61=150 S62=003 S63=001 S64=000 S65=000 S66=000 S67=000 S68=255 S69=000

S90=000 S91=000 S92=000 S93=008 S94=001 S95=000 S96=001 S97=000 S98=003

S100=000 S101=000 S102=000 S104=000 S105=001 S106=000 S107=020

S110=255 S111=255 S112=001

S121=000 S130=002 S131:001

S150=000 S151=004 S152=001 S153=001 S154=000 S155=000 S157=000 S158=000

S160=010 S161=020 S162=002 S163=003 S164=007 S169=000 S255=000

Then, when MNP operation is desired, set the DTE port (via SET SPEED

if using TSX's CL handler) to 9600 or 19.2kb if your cpu can keep up

with it, then SET MODEM T2500MNP.  This enables MNP only as attempts

at a V.42 connection first with some MNP modems appear to leave them

in a state where nothing following works.  When an MNP link can't be

established operation falls back to the normal mode.

V.42 and LAP-M facilities are enabled by setting the modem T2500V.42

which operation is just like that of the MNP only above, except V.42

is tried first, then MNP, and then normal operation if an error free

path can not be established.  Negotiation of these protocols can add

considerably to the time needed to connect.

All of this should be done on the same  default  profile with which

the modem will  automatically  answer  incoming calls to ensure the

modem will be properly reset to  its auto-answer state when exiting

Kermit-11 to the monitor.  The "enhanced" command mode is suggested

because the "conventional" mode does not allow XOFF flow control in

the command mode.  While this is not normally a problem with result

messages it does make listing the modem's status impossible at high

speeds.

Installation and Release Notes	KRT V03.62-8			Page Sixteen

Under RT-11 either the T2500MNP to T2500V.42 definitions may be used

to work around XL/XC's inability to modify its speed as is necessary

for the T2500.  Both will connect normally if error correction isn't

supported by the remote system.  The KM handler may also be used.

SET PHONE XMODE 3 to enable all possible result messages, or use 14

if you prefer to force display of the carrier speed and protocol in

all cases, even when an error correcting path is not established.

If by accident the modem is sent something at an unwanted speed and

becomes stuck there, toggle DTR (HANGUP or SET DTR) to reset it.

If you are using PEP/MNP/V.42 Telebit's Kermit protocol spoofing is

the fastest of all possible options even though it only uses normal

94 byte packets between modems.  It is set in the above to run when

requested by the other end.  To force it on first SHO LINE to check

the DTE speed (if a fallback speed is shown then one must first SET

SPEED 9600 or 19200 if using a mode where the DTE speed is locked),

then CONNECT and issue the command ATS111=n, when "n" is one of:

0    No protocol supported

10   Kermit protocol with no parity

11   Kermit and odd parity

12   Kermit and even parity

13   Kermit and mark parity

14   Kermit and space parity

20   XMODEM/YMODEM protocol

255  Use protocol specified by other end

Then, disable long packets (SET NOLONG will suffice), so the modems

run most efficiently, and proceed with the transaction.

Telebit T3000

Three Telebit T3000 definitions are provided.  T3000 is for disabling

MNP/V.42 error correction.  It checks the modem's connect messages to

match the DTE speed to that of the carrier.  T3000V.42 enables V.42 &

MNP modes and locks the speed.  Set up the T3000's defaults as listed

below.  This definition  (T3000V.42)  is for use with hardware  (RTS)

flow control.  This is ONLY available under the KM handler, where you

have an option to SET FLOW-CONTROL RTS before setting the modem-type.

WorldBlazer - SA - Version LA5.00W- Configuration A

B1  E0  L1  M0  Q1  T   V1  X12  Y0

&C1 &D2 &G0 &J0 &L0 &Q0 &R3 &S0 &T5 &X0

S000=0   S001=0   S002:0   S003=13  S004=10  S005=8   S006=2   S007:255

S008=2   S009=6   S010=14  S011=70  S012=50  S018=0   S025=5   S026=1

S038=0   S041=0   S045=0   S046=0   S047=4   S048:1   S050=0   S051=255

S056=17  S057=19  S058:2   S059:15  S060=0   S061:0   S062=15  S063:1

S064=0   S068=255 S069=0   S090=0   S092=0   S093=8   S094=1   S100=0

S104=0   S105=1   S111=255 S112=1   S151=4   S155=0   S180=2   S181=1

S183=25  S190=1   S191=7   S253=10

Installation and Release Notes	KRT V03.62-8			Page Seventeen

The T3000 does NOT do XOFF flow control in the command mode even when

it's enabled and functioning for data between modems unless S48=0 and

this of course kills an 8-bit data path.  A third modem definition is

provided (T3000X.42) which enables XOFF rtesraint - Kermit quotes the

8-bit counterparts of XOFF and XON so this will only be a problem for

binary data transfers using the Xmodem protocol.  This defintion will

force S48 to 0 and enable XOFF restraint when initializing the modem.

US Robotics "Courier" 2400/2400e

C1  E0  F1  M1  Q1  V1  X6

BAUD=02400  PARITY=N  WORDLEN=8

DIAL=HUNT   ON HOOK  TIMER

&A1  &B0  &H0  &I0  &M0

&N0  &R1  &S0  &Y1

S00=001  S01=000  S02=043  S03=013

S04=010  S05=008  S06=002  S07=030

S08=002  S09=006  S10=007  S11=070

S12=050  S13=032  S14=252  S15=000

S16=000  S17=000  S18=000  S19=000

S20=000

Configuration switches:

1 - UP    DTR controls modem

2 - UP    verbal result messages

3 - UP    quiet mode, for auto-answer

4 - DOWN  don't echo local commands, for auto-answer

5 - UP    enable auto-answer, modem answers on the first ring

6 - UP    DCD on pin 8 follows action of received carrier

7 - as req'd, DOWN for RJ-12/13 busy support

8 - UP    to recognize "AT" command set

9 - UP    drop phone connection when escape code received

10 - UP    load power-on defaults from NVRAM

QUAD - as req'd for your system and modem cable..

Use the Hayes modem type (SET MODEM HAYES).  Set the "&" registers per

the above and save them with AT&W.  The "S" registers aren't preserved

so ONTIM and TIMIN must = 60. (TSGEN.MAC)  if incoming calls are being

accepted (auto-answer enabled) under TSX-Plus.  If wish to use the MNP

version, lock the DTE speed as desired to at least 4800 (AT&B5&W) then

SET MODEM Hayes-MNP.

US Robotics modems such as the Courier 2400 or 2400e will occasionally

glitch when answering the phone and start a TSX port at 19.2kb instead

of 2400.  This is more likely to happen when the modem is not properly

cooled.  Replacing its rubber feet with some that hold it higher above

the surface on which it resides may help.  Setting the modem to answer

after the first ring (that is, on the second) may also help.  Starting

with TSX V6.5, 2400 is tried first, before 19200, which also may help.

SET PHONE XMODE 6 enables all possible result messages from the modem.

Installation and Release Notes	KRT V03.62-8			Page Eighteen

U S E R - D E F I N E D   M O D E M

-----------------------   ---------

Kermit-11 contains operating parameters for several of the more common

auto-dial modems.  To find out if your modem is directly supported use

the SET MODEM ? command.  If it is not, you'll need to use SET DIAL to

load the data used to control the modem and implement the various DIAL

routines.  These involve formatting commands to the modem, and what to

expect in the way of modem responses.  As an example the Vadic VA212PA

modem is awakened from an idle state by the character sequence (octal)

005 015

which is Ctrl-E followed by carriage return.  The modem responds with

HELLO: I'M READY

*

Thus Kermit needs to know when it sends the wakeup sequence it should

wait for the asterisk to be returned by the modem, after which it may

presumed the modem is awaiting further commands, such as that to dial

a phone number.

Suppose we had to tell Kermit about the Racal Vadic VA212PA (though in

reality Kermit already knows this modem).  Checking the owner's manual

for it, we find that:

To wake the modem up, we type a Ctrl-E followed by a return.

To dial a number, we type the letter D  followed by a return.  At

this point, the modem prints a NUMBER? prompt and we then type in

the desired number.  The number  is  then reprinted and the modem

waits for a return from us to confirm it's correct.

When it completes dialing it will print "ON LINE" or "ONLINE" for

a successful call, otherwise it may return "BUSY", "FAILED CALL",

"NO DIAL",  "VOICE"  or  "TIME OUT".  While it is waiting for its

call to be answered it may print the line "RINGING" several times

to tell you it is working on the call.

The Kermit commands required would be:

Kermit-11>SET DIAL WAKE-STRING \005\015

Kermit-11>SET DIAL ECHO

Kermit-11>SET DIAL WAKE-ACK *

Kermit-11>SET DIAL INITIATE D\015

Kermit-11>SET DIAL FORMAT %P%S\015

Kermit-11>SET DIAL DIAL-PAUSE 9K

Kermit-11>SET DIAL CONFIRM \015

Kermit-11>SET DIAL SUCCESS ONLINE

Kermit-11>SET DIAL SUCCESS ON LINE

Kermit-11>SET DIAL RINGING RINGING

Kermit-11>SET DIAL FAILURE BUSY

Kermit-11>SET DIAL FAILURE FAILED CALL

Kermit-11>SET DIAL FAILURE NO DIAL

Kermit-11>SET DIAL FAILURE TIME OUT

Kermit-11>SET DIAL FAILURE VOICE

Installation and Release Notes	KRT V03.62-8			Page Nineteen

The notation "\005\015" indicates a Ctrl-E followed by a return.  5 is

octal for Ctrl-E,  15 is octal for return.   Octal numbers may also be

specified by placing the value inside of  inequality  characters as in

SET DIAL WAKE <005><015> though the former is preferred.  Octal values

may be obtained from HELP ASCII.  If for any reason you need to pass a

"\" or "<" to your modem, simply prefix the character with another "\"

or "<" as in "\\" or "<<".

"%P%S\015" indicates the phone number from the  DIAL  command is to be

followed by a return.  "%S" is  a format effector for the phone number

string itself,  which causes it to be inserted in the dial string sent

to the modem.   "%P" inserts the dial pause string, as in this case we

need to dial 9 and wait for a second dial tone.  The "K" is the  Racal

Vadic  code to get the modem to pause.  If you are dialing on a direct

line or your modem supports pausing via something embedded in the dial

string itself, the DIAL-PAUSE command is unneeded.

Many modems require only WAKEUP, WAKE-ACK, FORMAT and result strings.

The Digital DF112 is a good example.  Its definition would look like:

Kermit-11>SET DIAL WAKEUP \002

Kermit-11>SET DIAL WAKE-ACK READY

Kermit-11>SET DIAL FORMAT %S#

Kermit-11>SET DIAL SUCCESS ATTACHED

Kermit-11>SET DIAL FAILURE BUSY

Kermit-11>SET DIAL FAILURE DISCONNECTED

Kermit-11>SET DIAL FAILURE ERROR

Kermit-11>SET DIAL FAILURE NO ANSWER

Any undefined result string is considered informational only thus this

class of response does not need to be expressly defined.

If you are using a "Hayes" style modem, its extended response mode may

be set by using the %X format effector in the WAKE-STRING.  The number

desired is then specified by SET PHONE XMODE n, where n is 0 to 14, or

"OFF" to disable it.  %X may also be used in the FORMAT (dial) string,

allowing modification without re-initializing the modem.

Here's a sample Hayes definition:

Kermit-11>SET PHONE TONE,XMODE 4

Kermit-11>SET DIAL COMMENT Generic Hayes Command Set

Kermit-11>SET DIAL TIME-OUT 45

Kermit-11>SET DIAL WAKE-ACK OK

Kermit-11>SET DIAL WAKE-STRING AT E1 Q0 V1 S0=0 S7=255 %X \015

Kermit-11>SET DIAL ECHO

Kermit-11>SET DIAL INIT-ONCE

Kermit-11>SET DIAL FORMAT AT %X %A D%M %S \015

Kermit-11>SET DIAL ABORT \015

Kermit-11>SET DIAL CON19200 CONNECT 19200

Kermit-11>SET DIAL CON19200 CONNECT 14400

Kermit-11>SET DIAL CON9600 CONNECT 9600    ; fallback only works with

Kermit-11>SET DIAL CON4800 CONNECT 4800    ; the CL or KM handlers..

Kermit-11>SET DIAL CON2400 CONNECT 2400

Kermit-11>SET DIAL CON1200 CONNECT 1200

Kermit-11>SET DIAL CON9600 CONNECT FAST

Kermit-11>SET DIAL CON300 CONNECT	   ; embedded, must be last!

Installation and Release Notes	KRT V03.62-8			Page Twenty

Kermit-11>SET DIAL FAILURE BUSY

Kermit-11>SET DIAL FAILURE ERROR

Kermit-11>SET DIAL FAILURE NO ANSWER

Kermit-11>SET DIAL FAILURE NO CARRIER

Kermit-11>SET DIAL FAILURE NO DIALTONE

Kermit-11>SET DIAL FAILURE NO DIAL TONE

Kermit-11>SET DIAL FAILURE VOICE

Kermit-11>SET DIAL RINGING RING		; includes RINGING

Kermit-11>SET DIAL RINGING RRING	; Telebit style..

Kermit-11>SET DIAL PULSE P

Kermit-11>SET DIAL TONE T

Kermit-11>SET DIAL ANSWER S0=1

Kermit-11>SET DIAL NOANSWER S0=0

Kermit-11>SET DIAL IDLE AT Z \015

Some modems are unable to accept data at the line speed, in which case

one must employ SET DIAL WAKE-RATE and SET DIAL DIAL-RATE.   These two

functions accept a delay time in clock ticks,  which  are 1/50 or 1/60

second units depending on where in the world this program is running.

If your modem has a settable timer it should be either disabled or set

to the maximum possible (example: the Hayes wakeup string contains S7=

255) to allow Kermit-11's internal TIME-OUT to prevail.

If your modem hangs, see HELP SET DIAL SETTLE-TIME.  This and TIME-OUT

are two special parameters here in that they are global, and used even

with Kermit's internal modem definitions.  However, default values are

loaded by SET MODEM, thus these must be altered AFTERWARDS, except for

the USER-DEFINED modem where what is set BECOMES the default.  This is

also the case for SET PHONE XMODE.

NOTE:  This program toggles DTR to reset the modem thus the modem must

have this option enabled in its default configuration.

If the modem echoes command strings sent to it SET DIAL ECHO to enable

display of anything output by it.

Once all pertinent data have been entered, it is then necessary to SET

Kermit's modem-type:

Kermit-11>SET MODEM USER-DEFINED

If a line (and speed if necessary) has been previously set this causes

Kermit to try initializing the modem, the result of which is reported.

If successful, you may then use the DIAL command to call out.

SET DIAL -- Additional information is available on:

ABORT             ANSWER            BINARY-RESPONSE   BLIND

COMMENT           CON300            CON1200           CON2400

CON4800           CON9600           CON19200          CON38400

CONFIRM           CONFIRM-ACKNOWLEDGE                 DIAL-ACKNOWLEDGE

DIAL-PAUSE        DIAL-RATE         ECHO              FAILURE

FORMAT            Format-effectors  IDLE              Important-Notes!

INITIATE          INITIATE-PROMPT   INIT-ONCE         NOANSWER

NOBINARY-RESPONSE NOECHO            NOINIT-ONCE       PULSE

RINGING           SETTLE-TIME       SUCCESS           TIME-OUT

TONE              WAKE-ACKNOWLEDGE  WAKE-RATE         WAKE-STRING

Installation and Release Notes	KRT V03.62-8			Page TwentyOne

SET DIAL ABORT

Defines a string to force the modem to stop dialing.  This is passed

to the modem by the strike-any-key abort function of Kermit-11 while

waiting for a dialed  call  to complete.  Non-printing chars must be

specified in octal notation.

Kermit-11>SET DIAL ABORT \003	; use ^C to stop modem

SET DIAL ANSWER

Defines a string the %A format effector will insert in the dial FORMAT

string when it contains said %A effector and you have SET PHONE ANSWER

to allow answering a call back from a security modem requiring it.

Kermit-11>SET DIAL ANSWER S0=1	; Hayes does it thusly

SHOW MODEM to display its current setting and status.

SET DIAL BINARY-RESPONSE

If your modem responds with single character result messages (sans any

terminator ala a return) set this so Kermit will as well.  The default

may be restored with SET DIAL NOBINARY-RESPONSE, which enables testing

of strings of more than one character.

SET DIAL BLIND

Defines a string to force the modem to dial regardless of its detecting

a dial tone or recognizing tone dialing capability on the line.  Actual

use of this string in dialing is enabled by SET PHONE BLIND.

Kermit-11>SET DIAL BLIND string

SET DIAL COMMENT

Specifies a brief description of the modem definition which is included

in the SHOW MODEM display.  Embedded blanks are ok, but multiple spaces

will be reduced to a single blank.

Kermit-11>SET DIAL COMMENT Hayes-MNP with DTE speed locked

SET DIAL CON300

Define a message from the modem which, as a result of a dialed call

will cause Kermit-11 to set its DCE port (ONLY when using the CL or

KM handler) to 300 baud and then CONNECT to it:

Kermit-11>SET DIAL CON300 CONNECT  ; Hayes defaults to 300

SET DIAL CON1200

Define a message from the modem which, as a result of a dialed call

will cause Kermit-11 to set its DCE port (ONLY when using the CL or

KM handler) to 1200 baud and then CONNECT to it:

Kermit-11>SET DIAL CON1200 CONNECT 1200

Installation and Release Notes	KRT V03.62-8			Page TwentyTwo

SET DIAL CON2400

Define a message from the modem which, as a result of a dialed call

will cause Kermit-11 to set its DCE port (ONLY when using the CL or

KM handler) to 2400 baud and then CONNECT to it:

Kermit-11>SET DIAL CON2400 CONNECT 2400

SET DIAL CON4800

Define a message from the modem which, as a result of a dialed call

will cause Kermit-11 to set its DCE port (ONLY when using the CL or

KM handler) to 4800 baud and then CONNECT to it:

Kermit-11>SET DIAL CON4800 CONNECT 4800

SET DIAL CON9600

Define a message from the modem which, as a result of a dialed call

will cause Kermit-11 to set its DCE port (ONLY when using the CL or

KM handler) to 9600 baud and then CONNECT to it:

Kermit-11>SET DIAL CON9600 CONNECT 9600	; Hayes

Kermit-11>SET DIAL CON9600 CONNECT FAST	; Telebit

SET DIAL CON19200

Define a message from the modem which, as a result of a dialed call

will cause Kermit-11 to set its DCE port (ONLY when using the CL or

KM handler) to 19,200 baud and then CONNECT to it:

Kermit-11>SET DIAL CON19200 CONNECT 19200

Kermit-11>SET DIAL CON19200 CONNECT 14400

SET DIAL CON38400

Define a message from the modem which, as a result of a dialed call

will cause Kermit-11 to set its DCE port (ONLY when using the CL or

KM handler) to 38,400 baud and then CONNECT to it:

Kermit-11>SET DIAL CON38400 CONNECT 38400

SET DIAL CONFIRM

Some modems use a command syntax wherein the number to be dialed is

echoed back to the user, after which the user must confirm it is ok

by returning an appropriate string:

Kermit-11>SET DIAL CONFIRM \015   ; usually just a return

SET DIAL CONFIRM-ACKNOWLEDGE

If your modem issues some sort of confirmation to the DIAL CONFIRM

sent to it, and there is  some  reason  to  test that in fact said

confirmation has actually been received by Kermit, you may specify

what to expect thusly:

Kermit-11>SET DIAL CONFIRM-ACK string

Installation and Release Notes	KRT V03.62-8		      Page TwentyThree

For example, older Vadic modems would say "DIALING" here.

SET DIAL DIAL-ACKNOWLEDGE

Specifies a string issued by the modem to acknowledge it is dialing.

Use of this may be  required  for  error  detection  with some older

modems, but it's normally not necessary.

Kermit-11>SET DIAL DIAL-ACK \015   ; usually just a return

SET DIAL DIAL-PAUSE

Defines the string for dial tone pause, which string is passed to the

modem by placing the %P format effector in the actual dialing string,

which in turn is defined with the SET DIAL FORMAT command.

Kermit-11>SET DIAL DIAL-PAUSE string

If you are dialing on a direct line or your modem supports pausing via

something embedded in the dial string itself the DIAL-PAUSE command is

unnecessary.  As long as the first char is numeric, commas and similar

tokens will be passed to the modem as part of the number from the DIAL

command.

SET DIAL DIAL-RATE

Allows pacing of chars in the dial string sent to the modem.  There is

an inherent wait of one clock tick (1/50 or 1/60 second, per your LTC)

to ensure an echo is not missed.  If this remains too fast for a modem

additional delay between each char may be specified in ticks:

Kermit-11>SET DIAL DIAL-RATE 3

SET DIAL ECHO

If the modem echoes command strings, SET DIAL ECHO causes the display

during DIALing to be driven directly from the modem's output.  If the

modem doesn't support this, SET DIAL NOECHO forces display of what is

sent, regardless of what may be received from the modem.

SET DIAL FAILURE

Define a message from the modem which, as a result of a dialed call,

indicates failure to complete the connection.  More than one message

may be defined if necessary to include all possibilities.

Kermit-11>SET DIAL FAILURE BUSY

Kermit-11>SET DIAL FAILURE NO ANSWER

SET DIAL FORMAT

This defines the actual DIAL string sent to the modem.  In  addition

to specifying the entire thing verbatim several format effectors are

provided to make life easier.  Most notable of these is %S, which is

the number to dial taken from the argument to the DIAL command.  See

HELP SET DIAL Format-effectors for detailed info.

Kermit-11>SET DIAL FORMAT %P%S\015

Kermit-11>SET DIAL FORMAT AT %X D%M %S \015

Installation and Release Notes	KRT V03.62-8		       Page TwentyFour

Most modems will require %S be the last item in the list (other than

the return as an input terminator), as otherwise a following command

will be taken as a request to abort dialing.  If blanks are embedded

they will be passed to the modem, although contiguous blanks will be

reduced to a single space.

SET DIAL Format-effectors

FORMAT (dialing) string format effectors supported:

%A - SET PHONE [NO]ANSWER string

%B - Dial blind

%M - Mode, pulse or tone

%P - Insert pause string

%S - Phone number string, passed by DIAL command

%X - Insert Hayes xmode string, unless set off

WAKE-STRING (initialization) string format effector supported:

%X - Insert Hayes xmode string, unless set off

See  HELP SET DIAL ANSWER, NOANSWER PULSE, TONE  and  HELP SET PHONE *

for info concerning exactly what each of these does.

If you are using a "Hayes" style modem, its extended response mode may

be set into the init string via SET PHONE XMODE n, where n is a number

between 0 and 14, or the word "OFF".  The  format  effector  for XMODE

is %X.  It may also be used in the dial string, allowing modifications

without re-initializing the modem.

Strings to be inserted will be displayed in SHOW MODEM when the format

effectors (SET PH BLIND,PULSE,TONE,XMODE) for them are enabled, except

for the phone number string, which is not built until DIALing.

SET DIAL IDLE

Defines the string used to return the modem to idle state, which is

sent to the modem upon exiting Kermit-11 or dropping the connection

(ala SET LINE) to it.  If the modem was preset to auto-answer, this

restores it to that state.

Kermit-11>SET DIAL IDLE ATZ \015

SET DIAL Important-Notes!

The SUCCESS, CONnnnn, RINGING  and  FAILURE fields are built as linked

lists thus you can have as many as necessary.  At least one SUCCESS or

CONnnnn message must be defined or any call is going to fail!

NOTE: Where a modem response string (SUCCESS, CONN, FAILURE) is a sub-

string of another response string the substring MUST FOLLOW the string

containing it as the comparison scheme used here ignores the return at

the end of the result string from the modem.  Thus, when you are using

CONNECT 1200 and CONNECT, "CONNECT 1200" MUST be defined first, BEFORE

"CONNECT" on its own.  All tests are done without respect to case.

Use of CONNECT messages to conform the speed of the DCE port (the one

on the PDP-11) to that of the modem (DTE port) is only possible using

the CL (TSX-Plus) or KM (RT-11/TSX-Plus) handlers.  These are for use

Installation and Release Notes	KRT V03.62-8		       Page TwentyFive

with modems which alter the speed of their data port to the connected

(telephone port) speed.  The old speed will be restored when the next

call is placed, unless SET otherwise first.  See HELP SET DIAL CONnn,

where nn=300 ,1200 ,2400 ,4800 ,9600 ,19200 or 38400.

Once you have  established  a  working set of parameters for your user

defined modem,  you  may  include  them  in  KRT.INI, Kermit's startup

initialization command file.  If memory runs out before everything you

need can be entered, increase the value of ALSIZE in KRTMAC.MAC.  This

same buffer is used to store phone numbers.  Also note malloc as it is

here does not deallocate anything, thus while re-entering data updates

pointers to them the superseded data's memory is NOT recovered.

The following pseudo code was extracted from the dialer source module,

with the hope of clarifying the functions of and relationships between

the seemingly excessive number of parameters available here.  Anything

that can modified by SET DIAL is shown in UPPER CASE.  Items listed as

optional should only be used when really necessary, or performance may

not remain as fast as it could be.

format_dial_string		; build the modem's dial_command_string

using FORMAT string and

%B BLIND dial string	; if set phone blind has also been done

%M PULSE or TONE string	; set phone pulse,tone selects which

%P DIAL-PAUSE string	; used if defined

%S the_phone_number		; as passed by the dial command

%X XMODE			; depending on set phone xmode

call tty_initialize		; open the link device

call fix_speed			; restore set speed (b4speed) if need be

test INIT-ONCE			; need to re-init modem?

if <>, goto do_dial

send WAKE-STRING	; ya, send it an init string

pace WAKE-RATE ticks	; pause between chars if <>

call check_abort	; check strike any key to abort loop

wait_for WAKE-PROMPT	; now wait 2 secs for an ack

on error, loop a maximum of three times then give up

do_dial:			; try to dial a number

; these two are optional and only for modems which need them

send INITIATE		; dump the string to initiate dialing out

wait_for INITIATE-PROMPT; if defined, wait up to 4 secs for a response

send dial_command_string	; this is from FORMAT, et.al. above

pace DIAL-RATE ticks		; pause between chars if not zero

test DIAL-ECHO		; does it echo?  if not, Kermit will do it

call check_abort		; strike any key to abort loop

; these three are optional and only for modems which need them

wait_for DIAL-ACK	; wait up to 4 secs for numb to echo perhaps

send CONFIRM		; stuff a confirm

wait_for CONFIRM-ACK	; if defined, wait up to 5 secs for the string

get_response:			; get the modem's response

wait_for SUCCESS ,RINGING ,FAILED ,CON300..CON19200

test BINARY-RESPONSE		; exit loop on a single character?

call check_abort		; strike any key to abort loop

wait_until TIME-OUT secs

Installation and Release Notes	KRT V03.62-8			Page TwentySix

call_fails_or_was_aborted:

send ABORT			; get the modem to STOP

wait SETTLE			; let modem recover from abort!

call_succeeds:

; if under TSX-Plus with CL handler and a CONnnnn result message

call b4speed		; save speed before fallback..

call set_speed		; then set it to what modem wants it to be

jump c$connect		; and connect automatically..

SET DIAL INITIATE

Some modems use a command syntax wherein the number to be dialed must be

preceded by a command to first initiate the dial mode itself.   Normally

these modems will also require using a DIAL CONFIRM string after sending

the number string.

Kermit-11>SET DIAL INITIATE D\015

SET DIAL INITIATE-PROMPT

If after use of the DIAL INITIATE option for some obscure reason you

find it necessary to check for proper response to it from your modem

the desired string may be specified thusly:

Kermit-11>SET DIAL INITIATE-PROMPT string

SET DIAL INIT-ONCE

If once initialized your modem remains in that state (some such as the

Concord 224 will time out and return to an idle mode) you may skip any

further unnecessary and time consuming inits by calling this function.

This is tested when dialing, and will speed up the redial looping time

if your modem allows its use.

Kermit-11>SET DIAL INIT-ONCE

To restore Kermit-11's default operation where the modem is re-initted

before trying anything and everything else SET DIAL NOINIT-ONCE.

SET DIAL NOANSWER

Defines a string the %A format effector will insert in the dial FORMAT

string when it contains said %A effector and you've SET PHONE NOANSWER

to disable the modem from answering an incomming call.

Kermit-11>SET DIAL NOANSWER S0=0  ; Hayes does it thusly

SHOW MODEM to display its current setting and status.

SET DIAL NOBINARY-RESPONSE

Enables testing modem result messages consisting of strings greater than

one character.  If your modem responds via single character result codes

SET DIAL BINARY-RESPONSE so Kermit will as well.

Installation and Release Notes	KRT V03.62-8		      Page TwentySeven

SET DIAL NOECHO

If the modem echoes command strings, SET DIAL ECHO causes the display

during DIALing to be driven directly from the modem's output.  If the

modem doesn't echo, SET DIAL NOECHO will force Kermit to display what

is sent to the modem in place of what would be echoed back.

SET DIAL NOINIT-ONCE

If your modem, once initialized for dialing out, doesn't remain in such

a state, particularly after a dial out call fails, SET DIAL NOINIT-ONCE

will force Kermit-11 to re-initialize the modem before each dial/redial

attempt.  This is the default operation.

SET DIAL PULSE

Defines a string to force the modem to use pulse dialing regardless of its

recognizing tone dialing capability on the line.  This string is passed to

the modem by the %M format effector (which must in the DIAL FORMAT string)

when one has SET PHONE PULSE to enable it.

Kermit-11>SET DIAL PULSE P	; "Hayes" uses this

SET DIAL RINGING

Defines a ring response, as returned by the modem to report the status of

a call in progress.  Kermit-11 maintains a count of received RINGS, after

seven of which the call will be considered failed due to no reply.

Kermit-11>SET DIAL RINGING RING   ; "RING" includes "RINGING"

Multiple messages may be defined as required to cover all possibilities.

SET DIAL SETTLE-TIME

Allows modifying the delay between sending a reset string to a modem and

dumping the handler connected to it, which also drops DTR.  If the modem

doesn't recover in time to resume its auto-answer mode, increase this to

something beyond the 20 tick default by setting a new value as a decimal

number of ticks.  One tick is 1/50 or 1/60 second (hardware dependent).

Kermit-11>SET DIAL SETTLE 30

IMPORTANT NOTE:  This is a global function effective for ANY modem type,

not just USER-DEFINED, but for KRT's built-in modem types this will only

prevail when this is done AFTER setting the MODEM-TYPE.

SET DIAL SUCCESS

Define a message from the modem which, as a result of a dialed call,

indicates successful completion and a data path has been established

between modems, and which will force Kermit to CONNECT your terminal

to it.  More than one message may be defined if necessary to include

all possible responses.

Kermit-11>SET DIAL SUCCESS CONNECT

NOTE:  This particular function neither checks nor changes speed.

If this is something you need, see HELP SET DIAL Important-Notes.

Installation and Release Notes	KRT V03.62-8		      Page TwentyEight

SET DIAL TIME-OUT

Defines the period in seconds after which if there is no conclusive

response (excluding informational messages) from the modem the call

will be considered to have failed.

Kermit-11>SET DIAL TIME-OUT 60	; for overseas calling..

IMPORTANT NOTE:  This is a global function effective for ANY modem type,

not just USER-DEFINED, but for KRT's built-in modem types this will only

prevail when done AFTER setting the MODEM-TYPE, and the modem's internal

time out is set to something greater than the desired time period here.

SET DIAL TONE

Defines a string to enable the modem to use tone dialing regardless of its

recognizing tone dialing capability on the line.  This string is passed to

the modem by the %M format effector (which must in the DIAL FORMAT string)

when one has SET PHONE TONE to enable it.

Kermit-11>SET DIAL TONE T	; "Hayes" uses this

SET DIAL WAKE-ACKNOWLEDGE

Defines the expected response to the WAKE-STRING (initialization to dial

out) sent to the modem.  This is tested three times over a period of six

seconds, after which the init will be considered to have failed.

Kermit-11>SET DIAL WAKE-ACK OK

SET DIAL WAKE-RATE

Allows pacing of chars in the init string sent to the modem.  There is

an inherent wait of one clock tick (1/50 or 1/60 second, per your LTC)

to ensure an echo is not missed.  If this remains too fast for a modem

additional delay between each char may be specified in ticks:

Kermit-11>SET DIAL WAKE-RATE 5

SET DIAL WAKE-STRING

Defines the wakeup (initialization) string sent to the modem to place

it in the ready-to-dial-out state:

Kermit-11>SET DIAL WAKE-STRING \005\015

Kermit-11>SET DIAL WAKE-STRING AT E1 Q0 S0=0 S7=255 V1 %X \015

Installation and Release Notes	KRT V03.62-8		       Page TwentyNine

R E S T R I C T I O N S

-----------------------

A problem was found with versions of Kermit-11 prior to  3.49  regarding

the  sending and processing of attribute packets.  Basically, two of the

attribute types were improperly implemented.  The support for  attribute

packets  was  added  back  in April 1984, at a time when testing against

other implementations was not possible.  At this writing, in March 1986,

some problems have been found.

The corrected version of Kermit-11 is V3.49.  This version will  NOT  be

able  to exchange attribute packets with previous versions.  In order to

make an initial download of Kermit-11 V3.49 to a host running  an  older

version, you must explicitly disable attribute support, as in:

Kermit-11>SET NOATTR

and then force both ends to binary mode, as in:

Kermit-11>SET FILE TYPE BINARY

The best way to accomplish the updating of remote sites is to use a pre-

3.49 version of Kermit-11 at both sides,  transfer the image appropriate

to your system, then replace the old executable image on the remote side

and your own side and transfer the help text and other needed files.

U p g r a d e s

---------------

This special version of Kermit-11 for  RT-11  and TSX-Plus may be from

time to time updated/upgraded, or various bugs may be found and fixed.

The latest revision may be had by calling the system on which the work

is done:

Host:             Billy's Place, Culver City, California

Available:        24 hours, 3 lines, immediate access

Phone:            +1.310.837.0892

Logon (username): 1000

Password:         moving_target

Modem speeds:     9600 (V.32) and down, or PEP (modems are T-2500s)

Protocols:        Kermit, VTCOM/TRANSF, Xmodem

Access is free other than whatever it costs to place the call.  Any bug

found will be fixed.  Please leave mail regarding same to account #1 on

the above.  Also on-line here are the complete Kermit-11 T3.60 sources,

along with a massive amount of RT-11 SIG and other public domain/gratis

software.  Mail to youdelman@eisner.decus.org or billy@mix.com may also

be used to report bugs.  A fairly recent edition of this package may be

gotten from ftp.utoledo.edu in their [PDP11-KERMIT.K-RT11] directory.

Installation and Release Notes	KRT V03.62-8			Page Thrity

The following files comprise everything required for KRT/KRTTSX:

KRT   .BWR	"Beware" file, known bugs and possible things to come..

KRT   .COM	Links low memory overlays image, KRT.SAV

KRT   .DOC	Installation and Release Notes

KRT   .IND	Bob Schor's Kermit-11 init file manipulator, an IND process

KRT   .INI	Sample initialization file

KRT   .MAP	Link map of low memory overlays image, KRT.SAV

KRT   .SAV	Low memory overlays image, for RT-11FB,SB,SJ and TSX+

KRTAIL.MAC	Tailgating protection for TSX+ V6.31 and below

KRTASM.COM	Assembles all source modules for an EIS CPU

KRTASM.NON	Assemble all sources for a non-EIS CPU using macros in KRTNHD

KRTATR.MAC	Attribute packet processing

KRTCDF.MAC	Command macro definition .INCLUDE file

KRTCMD.MAC	Main program loop, commands calling APR4

KRTCM1.MAC	APR4 overlay for the rest of the commands

KRTCOM.MAC	Main loop commands data table

KRTCON.MAC	CONNECT terminal emulator

KRTCVT.MAC	File name conversion routines

KRTDAT.MAC	Global read/write data

KRTDEB.MAC	EXAMINE command, display memory by symbol or address

KRTDEF.MAC	Packet definition .INCLUDE file

KRTDIA.MAC	DIAL command, also SET MODEM and SHOW DIAL

KRTDIR.MAC	Directory operations

KRTDSP.MAC	I/O dispatching to handler or terminal

KRTEDI.MAC	SL command line editor

KRTERM.MAC	I/O support for the controlling terminal line (TT)

KRTERR.MAC	Error messages

KRTHLP.HLP	HELP text file

KRTHLP.MAC	HELP command processor

KRTIDX.BAS	Builds KRTIDX.MAC from KRTHLP.HLP

KRTIDX.MAC	HELP text file index for high speed lookups

KRTINI.MAC	Initialization, packet logging

KRTKM .MAC	"Kermit Modem" handler

KRTLNK.COM	Links extended memory overlays image, KRTTSX.SAV

KRTMAC.MAC	Macro definitions .INCLUDE file

KRTMDM.MAC	Pre-defined modems database

KRTNHD.MAC	MUL, DIV, SOB, ASH macros for non-EIS CPU

KRTOID.COM	Link for RT-11 V5.6 separate I&D space, low memory overlays

KRTOSI.MAC	One-shot initialization at program start

KRTPAK.MAC	Low level packet processing

KRTREC.MAC	Incoming file protocol processing

KRTRMS.MAC	File access, RMS emulation

KRTSEN.MAC	Outgoing file protocol processing

KRTSER.MAC	Server support

KRTSHO.MAC	SHOW, COPY, DELETE, PRINT, RENAME commands

KRTSJ .CND	RT-11FB,SB,SJ conditional assembly file for KM handler

KRTSJ .MAC	Root module for low memory overlays image

KRTSTD.MAC	SET command data table

KRTST0.MAC	SET command overlay 0

KRTST1.MAC	SET command overlay 1

KRTSUB.MAC	Misc common utility routines

Installation and Release Notes	KRT V03.62-8			Page ThrityOne

KRTSV4.COM	Build KM handler under RT-11 V4

KRTSYS.COM	Build KM handler under RT-11 V5

KRTTRA.MAC	TRANSMIT command

KRTTSX.CND	TSX conditional assembly file for KM handler

KRTTSX.MAP	Link map for extended memory overlays image, KRTTSX.SAV

KRTTSX.SAV	Extended memory overlays image, for RT-11XB,XM,ZB,ZM and TSX+

KRTUTL.MAC	Misc utility routines, mount for TSX+

KRTVID.COM	Link for RT-11 V5.6 sep I&D space, extended memory overlays

KRTXL .MAC	I/O support for CL, KM, XC and XL

KRTXM .CND	XM conditional assembly file for KM handler

KRTXM .MAC	Root module for extended memory overlays image

KRTXMO.MAC	XMODEM, also SEND command processor

Installation and Release Notes	KRT V03.62-8			Appendix A

The following updates/fixes have been made to Kermit-11 for RT-11/TSX-Plus

since my edit of 10-Jul-90 and are included in this revision of 27-Jul-93.

Numerous changes were made before this list was begun, please refer to the

top  of  the  edit  history  in  KRTCMD.MAC  for  a  detailed  list of all

differences between this program and Kermit-11 T3.60.

BBS Edit: (3.61)

From edit of 10-Sep-90:

1.  The long packet maximum size was increased to 1024. bytes.

2.  KERMIT.INI is no longer echo'd - instead the name of the file being read

is briefly displayed.  This provides a cleaner, more direct sign-on.

3.  TSX+ window processing is now suspended when TT is init'd for data xfrs

and then restored when TT is deassigned.  This allows the program to be run

without the /NOWINDOW switch (previously done to avoid windowing overhead on

packets).

4.  An addressing bug in the EXAMINE (K11DEB) module causing incorrect values

to sometimes be displayed for integer (word) data was fixed.

5.  The help text file (K11HLP.HLP) has been thoroughly updated by reading

through the sources to make sure all but the most obvious functions are

covered.

6.  The escape sequence to exit the CONNECT mode is now displayed when

entering same.

7.  It is now possible to specify only a device as the output file name in

order to place a single (unambiguous) file being transferred on a device other

than the default.  When sending a file the receiving Kermit must not strip the

device from the file name (SET FILE NAM FULL).  All other wildcarded renaming

remains unsupported.

8.  The handling of extremely long VMS-type file names, up to and including

the max possible under VMS, is now done without writing past the end of any of

the various buffers involved.  Unix and pc subdirectory delimiter parsing has

been included too.  Owing to the rather limited nature of RT-11 file names,

it is strongly recommended the default file naming conversion not be off'd

when talking to such foreign systems.

9. The recpkt buffer size was increased from 130. bytes to accommodate a full

size long packet, which can be encountered when things get out of sync.  This

was causing a trap to 4 when the end of the buffer was reached, and was also a

problem when a burst of line noise inadvertently forced an incoming ACK/NAK

packet's size beyond 130. bytes as rpack$ checks overflow against MAXLNG.

10. The link command file has been modified to combine segments where possible

for the sake of reducing memory and mapping requirements.

Installation and Release Notes	KRT V03.61			Appendix A

11. The need for eight-bit quoting is now tested during the initialization for

each transaction.  Thus, once it has been used it may be off'd without exiting

the program by setting parity on BOTH sides (if the other Kermit accommodates

reseting eight-bit quoting once it has been invoked) to none.  This presumes

the problem in the path which made parity necessary has been resolved, or one

has selected a new path/system to call.  A warning message is now displayed at

the beginning of each transaction if eight-bit quoting has been forced on, as

its use increases the time required to transfer a file.

12. Comment delimiters ";" and "!" may now be used anywhere in a command file

text line.  In the past, their use was limited to the first column only.  From

the keyboard these remain treated as normal chars for the sake of VMS version

numbers and the like.

13. The help file index routine was modified to accommodate topic names longer

than 18. bytes, ala VMS help.  The "?" function as used to display the various

command dispatch name tables was similarly modified, thus preventing the list

scrolling off screen before one has adequate time to stop it.

14. The ccast (^C abort) was modified to send an error packet if called amidst

exchanging packets, and to reset the default block checking.  This makes it

fairly robust, though ^E is still preferable as it waits for the packet in

progress to complete before the abort.

15. A bug in setting the tsxcl flag, which caused DTR to be dropped when

exiting the CONNECT mode when using CL4 through CL7, has been fixed.

16. Several other small bugs, errors, omissions and typos have been fixed.

Some commands with duplicate functions, a few of which could be misleading

(ex: SET TIME-OUT, SET SEND TIME-OUT and SET RECEIVE TIME-OUT all did the

same thing because there are no separate send and receive registers) have

been pulled (only SET TIME-OUT remains).

From edit of 2-Oct-90:

1.  Handling of TSX's VLSWCH char (^W) during the CONNECT mode is selectable

by SET VLSWCH LOCAL (windowing remains on if in use and ^W will switch between

processes on the local machine) or REMOTE (the default, in which ^W is treated

as any other normal character and window buffering is, if in use, suspended).

SHOW VLSWCH may be used to determine its current status.

2.  TTYOUT is now used to echo received data in K11TRA.  Calling writ1char

would sometimes cause chars to be lost..

From edit of 7-Oct-90:

1.  Fixed TRANSMIT (in K11TRA) to display last char(s) echo'd from the

remote after the EOF is reached.  This introduces a small (1/4 sec) delay

just before returning to Kermit's command line to ensure nothing is missed.

2.  Packet size display in log files will now accommodate four digit numbers,

as the maximum packet length is now 1024. bytes.

Installation and Release Notes	KRT V03.61			Appendix A

3.  The logical INI is now tested for KERMIT.INI, before DK and SY.

From edit of 9-Oct-90:

1.  Action of ^B under TSX in the CONNECT mode now tracks ^W, allowing one

to print the appropriate window.

From edit of 11-Oct-90:

1.  Indexing was added to the help text file read routines (K11HLP.MAC).

This makes the response as fast as the first block needed can be read in off

the disk, regardless of its position in the file.  If the help file is edited

one will then need to conform K11HLP.MAC to the new file.  Please refer to

it for a description of the procedure involved.  A short BASIC program is

included which will do it all for you..

From edit of 19-Oct-90:

1.  Disallow typing TSX lead-in char to TT when running under TSX in CONNECT

and xreply modes.  TYPE already provides this protection.

2.  K11HLP.HLP (help text) received a massive update, and K11TSX.DOC was

created.

3.  Modified action of TRANSMIT on EOF to reconnect to the remote system.

From edit of 21-Oct-90:

1.  Cleaned up and clarified the help text file.

2.  Made ".COM" the default TAKE file type.

From edit of 25-Oct-90:

1.  Added %X dial string format effector to insert Hayes style extended mode

response commands into the dial string, and SET PHONE XMODE n to support same,

for which the Hayes modem definition is now patched as well.

2.  Cleaned up the K11HLP.HLP and K11TSX.DOC files yet again ..

From edit of 29-Oct-90:

1. Fix an error (of my own) wherein SET NOATTR would also kill long packets.

2. SHOW PARAMETERS modified to indicate if SET NOLONG/SET LONG OFF has been

done.

3. SET REC PAC > 94. now also sets long packets on (if previously off'd).

From edit of 30-Oct-90:

1.  Added a warning message if long packets are enabled sans CRC block check.

Installation and Release Notes	KRT V03.61			Appendix A

From edit of 23-Nov-90:

1.  Modified SET LONG-PACKETS ON to SET BLO 3 and SET REC PAC 1024, which

provides an easy way to enable them.  Previously this only reset a flag

allowing long packets if they were otherwise enabled.

2.  Fix a typo in K11RCO wherein the TSX lead-in char filter was not getting

same.

3.  Add an error message to the fatal error table (faterr:) to accommodate

-19. (355) in the error byte returned when trying to do a .lookup on a non-

initialized device, such as when HELP tries VM first.  Previously this

returned without an error, unless by chance there was an entry in the first

word of the rtque buffer.

4.  Allow time-out to be set as low as 1 second.

5.  Ring the bell at the end of RECEIVE, so one will know it's done.

6.  PRINT now works ala RT-11, sends a single (no wildcards) file to LP.

This shares the COPY code, so you may now also COP FILE DEV: to place a

single file on the output device while retaining the input file's name.

7.  NAMCVT fixed to not output a "." on a null file name input, then some

file name parse routines patched to use it (instead of coding the same thing

over a few times).

8.  Dropped the CONNECT sign-on message when re-connecting after a successful

transfer via TRANSMIT, which leaves the display and cursor position intact.

The bell is rung when the EOF is reached.

9.  MOUNT now looks for .DEV extent if it can't default to a .DSK file name.

10. Dump the raw I/O mode from SET DEB ALL, rendering logs files easier to

read and follow.  Cleaned up all the debug routines, made debugging to TT

without a disk file open possible, and fixed various small bugs.

11. Added tests for "strike any key" abort during DIAL so it now works even

before arriving at the point where it's waiting for a response from the called

system.  A DTR drop was added to the abort after the modem is waiting for a

response form the remote system to ensure some types of Hayes clones are hung

up even when aborting just after the dial string has been typed at the modem.

12. Modified DIAL to actually echo from the modem, allowing init error

messages and other such stuff previously hidden to be seen.

13. Added CONNnnnn (where nnnn=various speeds) to SET DIAL options.  These

allow falling back to the indicated speed upon connecting, if this is how

your modem handles that.  Also added SET DIAL ABORT, COMMENT, IDLE and

RINGING, allowing one to fully specify a user-defined modem.

14. Display the HOME directory in SHOW ASSIGNS.

Installation and Release Notes	KRT V03.61			Appendix A

15. Added a SET FILE NAMING [NO]LOWER-CASE option which allows sending lower

and mixed case file names to a remote system (Unix) that is case-sensitive.

The default is NOLOWER-CASE, and its status is included in SHOW FILE.  This

has the happy side effect of allowing lower case in user-defined modem strings

and many other useful places.

16. Help text file was again updated, particularly its debug and SET DIAL

sections.

17.  Various small bugs/errors/etc were found and fixed.

From edit of 27-Nov-90:

1.  Added support for Telebit T2500 modem, including a separate definition

for PEP/MNP/V.42 operation in which the DTE speed must remain fixed.

2.  SET MODEM now will init the modem, if there is a line assigned on which

to do so.  This takes it out of the auto-answer mode when attaching a CL

unit to a line also used for incoming calls.

3.  Connect result messages from modem are now displayed, as the data path

speed is not necessarily also the DTE speed.

4.  Various small bugs fixed and misc cleanup work done.

From edit of 3-Dec-90:

1.  Fixed display formatting for SHOW PHONE.

2.  Sped up the testing of modem responses whilst trying to connect.

3.  Init.once support for the dialer was added in edit of 27-Nov-90, and

here in this edit was added to SET DIAL to allow its use in a user-defined

modem as well.  Make this non-zero in definitions of modems that do not

require an init string before each retry when dialing out.

From edit of 16-Dec-90:

1.  Sped up CONNECT processing of input from TT by calling ttdone once per

clock tick instead of 2 ticks.  This started out at 3 ticks..  This makes a

noticeable difference in the speed at which data may be sent, particularly

when from a function key generating a long string without any delay between

chars.  Modified TT input processing to also pass 8-bit chars to the remote

if SET CONSOLE 8 (default is 7) and no parity is in use.  Sped up exit to the

command line by skipping TT input routines once the done flag is set.  This

permits one to type ahead immediately after the escape sequence without any

of that getting hosed by completion routines that execute before the done

flag is checked.

2.  Made selection of a VT-200 terminal (during start-up or when set) also

enable 8-bit CONNECT operation (as if SET CON 8).

Installation and Release Notes	KRT V03.61			Appendix A

3.  In a recent edit a DTR drop was added to the dialer for the strike any key

abort when done after the modem has dialed and is waiting to connect.  This

is because some Hayes clones do not reliably hang up from just keyboard input.

As doing this resets many more of these clones, the modem is re-initialized

after said DTR drop.  This ensures it doesn't sit in its auto-answer mode

under TSX when it should be available to Kermit.

4.  The modem's init status is now saved thus the init string (rather long

for the Telebit definitions) is not sent when dialing unless it's needed.

The Hayes-style extended mode format effector is now in both the init and

dial strings, allowing it to be modified without re-initting the modem.

5.  The Telebit MNP (T2500MNP) definition was changed to enable MNP only.

That is, V.42 LAP-M operation is disabled because some MNP modems get so

confused by the V.42 stuff nothing else has a chance, even though the

Telebit grinds through them all..  A third Telebit definition (T2500V.42)

was added which enables everything possible.

From edit of 19-Dec-90:

1.  Added missing fields to SET DIAL for user-defined modem, and to SHO MODEM

too.  SHOW MODEM now displays speeds for result messages which change the DTE

speed to match.

2.  SET TT TTY or VT100 now clears the 8-bit mode in CONNECT.  It may be set

back on after if desired..

From edit of 25-Dec-90:

1.  Patched SHO MODEM to display format effector values when same have been

defined and set to be included in the init or dial strings.

2.  Fixed a problem with the modem result message processing where a connect

message containing the substring "ERROR" would return as an error due to the

"ERROR" result message also being defined.  Now, the match is only valid when

it begins at the first byte of the strings tested.

3.  Increased the size of the buffer used to store phone numbers and the

user-defined modem to 1kb (from 384 bytes), making it possible to fully define

a modem and store about 25 phone numbers as well..

4.  Cleaned out most unused/obselete/left-over stuff from source modules.

5.  Added fixwild from K11RMS.MAC, operation flipped to translate "?" into "%"

for RT-11 wildcarding syntax.

From edit of 1-Jan-91:

1.  Added support for file creation date/time and protection (within limits

of RT-11) attributes.  This uses the "#" attribute, as opposed to 4 16-bit

words in ascii'd octal, which was never supported under RT-11.  See HELP SET

ATTRIBUTES for details.  This works with C-Kermit, MS-Kermit.

Installation and Release Notes	KRT V03.61			Appendix A

From edit of 3-Jan-91:

1.  Xmodem size display now uses 32-bit number for RT-11 files >16k blocks.

From edit of 7-Jan-91:

1.  Modified error handling to use code in K11RTE.MAC, which allows init/take

file failures to be displayed properly, and will make script file support

possible.

2.  SET [NO]QUIET made SET TT [NO]QUIET to conform with RT-11 syntax, and

added missing tests for it as needed in the program.

3.  SPACE/REMOTE SPACE now display the device, and file spec if used, with

which the reported numbers of files and blocks used were calculated.

4.  Shuffled some sub-routines to segments with more room, as some regions

were getting close to their page boundaries.  DIRER$ moved to the root to

support above mentioned global error handling.

5.  Disallow running the server unless TT is the link device.  Although

the server is quite capable of talking through a handler, this can be very

confusing when the expected path is the connected terminal line, as the

server is normally run on the remote system (as running it locally leaves

one with no means of passing commands to it).

6.  Created K11MIN.COM, a link command file to create K11MIN.SAV, which uses

the minimum possible memory (110kb) by doing the maximum possible swapping of

segments.  This saves 48kb, at the expense of increased disk I/O..

From edit of 11-Jan-91:

1.  If a modem is on-line via a TSX CL unit, the modem will be reset to its

idle (auto-answer) state if a new line (link device) is SET.  This is to

ensure a time sharing line is not unnecessarily kept from handling incoming

calls.

2.  A TSX CL unit is now set to its default speed (9600) when SET as the

link device, thus a following SET SPEED command is not needed unless a

different speed is desired.

From edit of 17-Jan-91:

1.  Modified file date/time creation attribute to be sent under RT-11 too.

From edit of 30-Jan-91:

NOTE:	As of this edit, no .COM file is needed to run Kermit under TSX-Plus.

My suggestion is K11TSX.SAV be renamed to K11.SAV when copied to SY.

Then, if K11.COM was used by you in the past, it should be deleted,

along with the file it called (the old K11TSX.SAV)..

Installation and Release Notes	KRT V03.61			Appendix A

1.  Fix typo in K11RTI so RT-11 init code that checks for available handlers

(XL or XC) and sets the line accordingly now runs.

2.  Added SET CLn LINE x facilities, including an automatic scan through TSX

lines (ports) and CL units until an available combination is found, as well

as specifying a speed and modem-type to try on each possible port.  This

provides some convenience in a multi-user environment, especially when the

lines are also used for incoming calls, and in particular when using an odd

lot of modems.  See HELP SET CL for details and operating instructions.

3.  SET LINE now allocates the device (if it was not before) under TSX.  It's

deallocated when exiting Kermit or if the line is later set to something else,

along with restoring what ever else (modem, TSX line, etc) is necessary.

4.  Modified getcdt (get creation date and time) and sn.cdt (its companion

which writes the appropriate packet) to omit sending create time under RT-11.

It's determined by using the "misc" word in the directory ala TSX even if

running under RT-11, thus there is no way to distinguish between "00:00:00"

(a valid time) and a null entry.

5.  This program is now self-contained, in that no .COM file is necessary as

Kermit itself now supports all commands needed for its operation which were

previously only available from the monitor under TSX-Plus.  In addition to the

above SET CL and SET LINE, a CL assign is released and deallocated on exiting

or when modifying the line, and the job's priority will be raised to 71 when

doing I/O through a handler (so as to not drop chars at higher speeds).  This

assumes the normal default priority is 50 or something close to it.

6.  Fixed BINWRITE so it actually does wait for completion before returning.

Also patched to XON and retry if it doesn't in 5 secs, as opposed to XON and

then bailing out.  ^C aborts if need be..  Also dropped appending a LF when

sending a string terminated with a return.

7.  Went back to calling ttdone in CONNECT once every two ticks because once

in a great while at high TT speeds a current byte would be output twice, the

first of which would eat the previous byte..  This was only observed at 19200

however the best course seems to be better safe than sorry.

From edit of 3-Feb-91:

1.  Fixed a small typo in the help text file, then fixed yet more stuff here..

2.  Fixed bogus error after SET MODEM ? that would also eat the variable

containing the modem's ready status.

3.  Made SET DEBUG NONE dump ALL (even TT-based) debugging.  SET DEB OFF

still only closes disk-based logging, without affecting terminal displays.

4.  Made .LOG the default extent for log files and .LST for PRINT.

5.  Added XMODE option to SET CL PORTS/UNITS/* auto assign code.  This allows

painless use of an odd lot of Hayes-style modems..

6.  Added TSX line #, modem type (if on-line) and its DCD status to sho$line.

Installation and Release Notes	KRT V03.61			Appendix A

From edit of 5-Feb-91:

1.  Dump ascdat in k11rtu and use cvtdat in k11rtd.  The only thing that calls

ascdat is the DATE/SHO DATE command, from which it doesn't matter what's where

in the overlays.

2.  Fix various bugs in PRINT, COPY, RENAME and DELETE, and added some

inadvertently omitted upper case routines to their arguments..

From edit of 7-Feb-91:

1.  Fixed non-initialized repeat count register bug in bufunpack where any

char repeat-quoted an even number of times would be decoded as rpt_cnt+1.

This was rendering a file name such as CK~$A.HLP as CKAAAAA.HLP instead of

the expected CKAAAA.HLP.

2.  Added SET CONSOLE [NO]MILNET enabling which sends a couple XONs when first

CONNECTing to automatically wake up a MILNET Terminal Access Controller or any

other hardware needing ^Q.

3.  Fixed problem where calling a take file from the init file would blow up

due to using the stack for the new file name buffer.

4.  Patched the TSX routines to not drop a CL cross-connection unless it was

attached from within Kermit.

5.  Dumped the K11MIN version and created K11RT5, a disk overlaid version,

instead.  This one should run on anything.  It is stressed RT-11SJ MUST be

sysgenned for .mrkt and .twait support (or, you'll have to use RT-11FB) in

order to do anything useful.

6.  Fixed "Warning: Parity is possibly being introduced" to only appear once.

A small glitch setting the hi bit once left it repeating with every packet..

Bug reports are in particular solicited for K11RT5.  I don't often use RT-11

but will gladly fix anything brought to my attention..

From edit of 10-Feb-91:

1.  Reset the fixed (per above) counter for parity found in the incoming data

although set NONE on the local Kermit warning message before each transaction.

2.  Shuffled some routines around and forced some psects into the root for the

disk-overlaid version.  This fixes xmodem and the rem dir display formatting

and reduces K11RT5's hi limit to 19714. words (including its buffers uses 48kb

plus whatever the monitor adds) which is hopefully enough.

From edit of 12-Feb-91:

1.  Modified BINWRITE to not wait for completion, but to XON and retry once

when an error occurs, then give up.  This fixes ^C abort during things sent

from a server (remote dir being a good example) although ^E remains a better

choice for such aborts..

Installation and Release Notes	KRT V03.61			Appendix A

2.  Removed a few unused things in the root uncovered in the course of the

above, and made another pass at cleaning up the source modules.

3.  Restore SET TT [NO]QUIET after a ^C abort, if during something that mutes

the display..

From edit of 16-Feb-91:

1.  Fixed the "DK" in "assign ld0 dk" to accept lower case "dk" too.

2.  Fixed a couple typos in K11TSX.DOC/K11HLP.HLP ..  Also expanded the

information on using initialization (KERMIT.INI) files.

3.  Fixed warning messages to distinguish between:

A) long packets are off locally but the other Kermit says it can do them

B) long packets are on locally but unsupported or off at the other end

4.  Allow init file to be echoed after SET TT NOQ is done in it.  The default

remains quiet until the init file is closed at which point it becomes noquiet,

unless set in said file, which overrides the defaults.  If you wish to display

the entire init process, make SET TT NOQ the first line in KERMIT.INI.  Init

files may also be tested by running them as command files:

Kermit-11>@dev:KERMIT.INI	; "dev" default is SY

From edit of 17-Feb-91:

1.  Cleaned up the init code (K11RTI) error handling.

From edit of 19-Feb-91:

1.  Fixed COPY error handling, which included fixing same for the file close

routine, and did a final clean-up on the whole COP/PRI/REN/DEL collection.

COPY now passes the file size, and dies immediately if there is not enough

free space instead of grinding through it all until it can't write any more,

and will dump an incomplete copy on the output disk if the input file dies..

2.  Fixed BINFIL writing trailing spaces into file extent and sending same

to the remote where they would be replaced with Xs, making it tough to send

anything with a type of less than three chars (sorry about this!).

3.  Dropped CONNECTing to a device specified as an argument to that command

and in general disallow setting a new link device without dropping one that

may be currently in use first until it's reasonably well debugged.  The idea

here is to not keep a TSX line unable to answer unless it's actually in use,

which for now may done with the appropriate keyboard commands and dealing

with any error when it occurs.  This is completely separate from the reset

done when exiting the program.

From edit of 21-Feb-91:

1.  Pulled yet more unused stuff from the program.

Installation and Release Notes	KRT V03.61			Appendix A

2.  SOH and EOL chars are now displayed (SHO PARAM) in the same number

base (octal) with which they may be modified.

From edit of 23-Feb-91:

1.  Fixed trap to 4 for SET CL on any bad parameter name.

2.  Modified sending attributes to do it all in a single packet, ala C-Kermit.

From edit of 25-Feb-91:

1.  Fixed error (of my own) regarding testing RT-11SJ for timer support.

2.  Cleaned up some other odds and ends, such as the error handling for SET

DTR when no line is set, added a comma between entries for rcv,calc checksums

in logging errors for them, etc..

3.  Made the LOCAL mode the default for SET VLSWCH.  This is how it's turned

out to be most used, and it's also less confusing to first time users of this

program.  Those of you already using this can drop "SET VLSWCH LOCAL" from

your init/command files..

4.  Added a test when opening the help text file to check for the correct

version of K11HLP.HLP, and warn if incorrect version(s) found.

5.  Changed the VM logical in the help search path to HLP and added a new

K11 logical to it and the init path.  Paths are now:

For HELP (K11HLP.HLP):  HLP, K11, SY, DK

For Init (KERMIT.INI):  INI, K11, DK, SY

From edit of 28-Feb-91:

1.  Reset keypad to numeric mode for SET TT TTY if it was previously in the

application mode.  Also cleaned up display in that nothing will now be over-

written on a hard copy terminal.

2.  Noted in the docs (HELP INSTALL) a problem and work arounds where using

the ^>R K11.. option under TSX to force command file input to Kermit causes

the strike any key test for an abort during dialing to hang.  Dialing may be

done either from a Kermit take file or by passing a command for it to Kermit

when it's started.

3.  Added a search path for take files:  TAK, K11, DK, SY

4.  Fixed problem with getting clock rate under SJ/FB.

From edit of 3-Mar-91:

1.  Make take file default type (TAKDEF) a global so it's easily patchable.

Installation and Release Notes	KRT V03.61			Appendix A

2.  Fixed the RT-11 code which clears the CR/LF bit and toggles the PAGE bit

in $TCFIG so it all actually works as intended, especially under RT-11XM.

This was then extended to check the status of SET TT [NO]SCOPE (as may have

been done under RT-11) when the program is started, and if it's SCOPE Kermit

will call it a VT-100.  Kermit's SET TT was also extended for the [NO]SCOPE

option (only does it locally, does not affect the monitor setting).  The

choices now are:

NOSCOPE	= a printing terminal

TTY	= a tube terminal, but not a VT-xxx

VT100	= itself

VT200	= ditto..

Under TSX the terminal type will be gotten and used per the above.

3.  Fixed a problem with SET ATTR ON concerning sharing long-packets code.

4.  Enhanced the SHO FILE display to include misc items even when at their

defaults.

5.  Misc other small things fixed or pulled..

From edit of 4-Mar-91:

1.  Added SHO TERMINAL.

2.  Use Kermit's default for DK in the HELP file path.

From edit of 10-Mar-91:

1.  Fixed bug in detecting where comments begin when a "!" preceded a ";"

in a line in a command (take) file.

2.  Dumped a bit more unused stuff, fixed a help file typo..

From edit of 14-Mar-91:

NOTE:  Under TSX or RT-11SJ for some reason the XL handler does not like to

.close under this program until it gets a byte of input through the port it is

serving.  This can be a real pain, as when it's thus hung, there is NO other

way to un-hang things.  Until this is fixed, avoid such use of the XL handler.

That is, do NOT use it with TSX or RT-11SJ.  If you do get stuck, and have

run the server on the other end, you can wait for the server to time out and

send a NAK packet, which will definitely do the trick..

1.  Rearranged some stuff to make the XM (K11TSX) root smaller (2886. words).

This uses a few kb more memory, PLAS swap file space, etc. in order to make

life easier for XM use.

2.  Moved disabling XL's interrupts to exiting program as it seemed to get

get confused under XM when doing this every time link was closed, after which

weird things would happen, such as ^\I in CONNECT failing not only to drop

DTR, but to even trap the "I" on some systems, or "?" on others..

Installation and Release Notes	KRT V03.61			Appendix A

From edit of 17-Mar-91:

1.  Fixed the can't .close XL problem by moving the binary read routine

to the I/O dispatch module so CONNECT and packet xfrs can share it, after

which XL/XC can be left open for every monitor (lncluding TSX) other than

RT-11XM.  This means if you use XL/XC anywhere besides XM, you can not then

deassign it without exiting the program.  This may be a pain, however it is

far better than no handler support at all for the SJ monitor..

From edit of 19-Mar-91:

1.  Cleaned up the RPACK (received packet) debug display, added <TIMOUT>

and now won't display <EOL> unless it's actually received (as opposed to

merely getting "length" number of bytes).

2.  Combined the TT and handler I/O modules into one segment in the low

memory overlay version to avoid zapping some handler parameters not kept

in the root.  Adds about 280 words..

3.  Cleaned up the shared XL read routine so as to not drop chars when

entering the CONNECT mode and when getting the first packet after exiting

it.

From edit of 20-Mar-91:

1.  Fixed DCD display for XL handler.

2.  Reorganized psects to get stuff out of root that doesn't need to be there.

3.  PRO auto assign of a CL unit now sets flag used to deassign same when

modifying the line or exiting Kermit.

4.  Added esc_char ^x to send control char "x" to the host ala VTCOM, tho

here it's done from within the CONNECT mode.  Useful under RT-11, TSX passes

everything directly.

From edit of 22-Mar-91:

1.  Restored the "stock" Kermit kludge of doing several twaits in the DTR drop

to dump extra .RSUMs queued up when in the CONNECT mode which cause it to fail

under XM (only) when one twait becomes a nop and sends XL to outer space..

Fixing this properly will require some thought about how best to proceed.  As

it is now the idle timer alone does a .RSUM every 3.1 secs, etc, etc..  In the

meantime, this program now runs reasonably well under SJ, XM and TSX and that

includes CONNECT from SJ through XL (with TIMER support) and even XL with TSX.

2.  Fixed transfer of control of the handler read spfun completion routine to

CONNECT (from anywhere else) to only pass unused data, which may be less than

the contents of the whole buffer.  This avoids displaying the last part of a

modem message when DIAL succeeds and jumps to the terminal emulator.

Installation and Release Notes	KRT V03.61			Appendix A

From edit of 23-Mar-91:

1.  Display take file opened message even when a device for same is specified

in the command string.

2.  Fixed problem with PRO auto-assign of XC where it would blow up and Kermit

would try to send a file to it as if it were a disk..

3.  Fixed error handling for strike any key abort during REDIAL, also added

some small delays to let modem settle between various stages of its abort.

From edit of 24-Mar-91:

1.  Fixed a problem where some modems (most notably a Telebit at 9600/19200)

would start the TSX port to which they were connected when exiting Kermit.

2.  Misc small timing changes made to accommodate the cheaper variety of "pc"

modems.

3.  Don't flush the handler read buffer while DCD is high, so any plain text

message sent by the remote after a file xfr can be displayed when CONNECTing

back to it.

From edit of 31-Mar-91:

0.  With this edit version X3.61 becomes Y3.61, as the program now actually

performs as claimed/expected/etc, and is relatively bug free..

1.  Cleaned up SET FILE TYPE so when one sets a specific type, the flag

controlling same is cleared forcing the SET value.  In the past the image

mode would be set when sending (and that attribute sent) if the file type

was found in the binary-type list.

SET FIL AUTO (default) now enables the binary test, SET FILE anything-else

locks Kermit in that particular mode.  SHOW FILE was fixed to reflect what's

really going on.

2.  A serious attempt was made to clean up the help text file, make it more

readable and render it completely accurate with respect to what this Kermit

actually does..

3.  Modified SET RETRY ala VMS Kermit in that now there is a separate

parameter for send or receive inits, INITIAL-CONNECTION, and PACKET for

the number of retries after the init succeeds.  See HELP SET RETRY.

4.  Added SET INCOMPLETE-FILE-DISPOSITION for incoming files ala VMS Kermit.

5.  Made the TSX priority boost when using a handler settable by adding

SET CL PRIORITY.  Legal values are 50 to 100.  This helps when chars are

dropped at high handler speeds..  71 is a suggested starting point for 9600

if most everything else runs at 50, and this must now go into KERMIT.INI if

one wants to retain the same priority boost as was the default in the past.

Installation and Release Notes	KRT V03.61			Appendix A

6.  Patched HELP so it nests in as far as possible when a [sub]topic error

occurs after one or more of the prior arguments matched, instead of going

back to the top and displaying the root list of topics no matter what..

From edit of 2-Apr-91:

1.  Fix calculation of buffer space to only allocate what's actually needed.

Also made init error reporting more explicit, including displaying memory

needed vs. memory obtained when it's not enough.

2.  Added SHO MEMORY to display free memory available to fetch handlers

under RT-11.  Also display same when signing on.

3.  Added a test for a running clock to the above init code..

4.  Use copyz macro to load phnum buffer to be sure it won't be overwritten.

From edit of 3-Apr-91:

1.  When the packet-length is reduced as Kermit attempts to make things work

when the first data packet fails, display its new values as it's modified.

2.  Extended the incomplete file disposition code to check for a discard

flagged in the EOF packet and use same accordingly.  That is, if you are

sending to another Kermit, or to this one from another which supports it,

when you abort on the send end (via ^X, ^Z, etc) the incomplete file left

on the receive side will be saved or discarded as it is then (locally) set.

3.  Flush TT input buffer when exiting GET, RECEIVE, REMOTE and SEND to eat

any lingering control chars.  A leftover ^Z will dump the program if it gets

through.

From edit of 8-Apr-91:  (Y08)

1.  Corrected some errors in the HELP for SET DIAL, and added its text to the

doc file.

2.  Add a warning message when same packet is ACKed more than once, as from

excessive delay in the path.

3.  Add a generic Hayes-MNP modem definition, supporting a locked DTE speed.

Also pulled two of the three Vadic definitions that were identical.  Use the

generic Vadic for all the VA212 variants..

4.  Fix error handling for LOGFILE command, also added support for LP as the

log file.  For this you MUST spool LP in the operating system, if you will

be working faster than your printer can print, which will likely be the case.

5.  Added a couple extra queue elements, in case one wants to send a break

amidst receiving data sufficiently fast to keep all previously available busy.

Installation and Release Notes	KRT V03.61			Appendix A

From edit of 15-Apr-91:  (Y13)

1.  Fixed failure to send abort string to modem when a call timed out or

didn't answer ..

2.  Sped up the CONNECT code, cleaned up its source file, etc..  This now

will hold its own with VTCOM regarding speed when receiving data from the

handler.  Writes from the terminal to the handler are currently only half

as fast, though it's only noticeable when using function keys which output

data at speed.  Also added a "Z" command to zap (100% hose and reset) the

handler.

From edit of 19-Apr-91:  (Y14)

1.  Cleaned up Kermit-11's emulation of ".PRINT" knocking another couple

blocks off the complied program size.

2.  Added an HPRINT facility to print help text as may be useful..

From edit of 21-Apr-91:  (Y15)

1.  Sped up CONNECT's sending of keyboard input to the remote.  This

involved a minor cleanup of the parity generation stuff, which is back to

using a lookup table as it's faster and only added 32 words to the root

after the mods were done.  This remains slightly slower than VTCOM, but

then VTCOM doesn't offer local echo, software generated parity, protection

from typing the TSX lead-in char to the terminal, etc, etc..

2.  Moved all of the WHATS.NEW file to the end of K11TSX.DOC, which is

where it will live from now on, other than keeping only the most recent

edits in it, as it was getting too big and not all that current..

3.  Minor tweak to SET MODEM to ensure DTR comes up and stays there on

PRO hardware.

From edit of 24-Apr-91:  (Y16)

1.  Fixed a problem unique to XM where after dialing four calls which

resulted in failure the program would trash RMON and crash.  See the

incsts subroutine in K11RT4 for details.  This also led to some further

cleaning up of the dialer module, which is now is slightly faster when

clearing/reseting/re-initting the modem.

From edit of 25-Apr-91:  (Y17)

1.  Combine program segments where possible in both versions.

From edit of 26-Apr-91:  (Y18)

1.  Added version checking for TSX so as not to crash when attempting to

use something that does not exist in a particular version.

Installation and Release Notes	KRT V03.61			Appendix A

2.  Sped up (slightly) CONNECT's writing data from TT to the handler.

3.  Cleaned up code for EXAMINE.

4.  Fixed DISMOUNT to work with just "LD" (no zero, no colon).

From edit of 29-Apr-91:  (Y19)

1.  Fixed dupe error messages bug in HANGUP, SET DTR and modem init.  Also

cleaned up error handling for SET MODEM USER-DEFINED.

2.  Modified suggested setup for Telebit T2500 to the non-MNP, non-V.42,

plain, ordinary operation.  The various T2500 definitions init it as needed.

Also changed S55 to 0 (no big deal as the escape char is also disabled) and

S58 to 3 (it was 7, an error).

3.  Modified SHO LINE to not run off the screen by moving DTR status down a

line, then display PARITY on a third line, etc..

From edit of 30-Apr-91:  (Y21)

1.  Fix reset/DTR-drop/etc for modem on exiting so it all happens without

screwing up the modem or starting the line to which it's attached.

From edit of 1-May-91:  (Y23)

1.  (Y23A) Fixed a problem where HPRINT would eat the index space padding.

2.  (Y23) Moved the getdk routine to K11INI from K11RTI, and thus farther

down stream in the init.  This avoids various types of what seem to be

hardware related/dependent crashes when calling same under TSX..

From edit of 3-May-91:  (Y24)

1.  Clean up TRANSMIT code, modify for more efficient echoing.

2.  Sped up modem SET/RESET/EXIT stuff to run as fast as possible, and

patched c$idle to restore SET speed before reset.

3.  (Y24A) Clean up comments and help to more accurately reflect mods made.

From edit of 7-May-91:  (Y25)

1.  Help text, comments, .DOC file clean up.

From edit of 10-May-91:  (Y26)

1.  Add support for non-EIS CPUs, made the divide macro double precision

(input only, same as EIS divide) and made calls to ddiv and $ddiv normal

divides, since double precision output is never needed (or at least used).

2.  Made the distributed K11RT5.SAV runnable on a non-EIS CPU ..

Installation and Release Notes	KRT V03.61			Appendix A

3.  Added a "BUG" command ala C-Kermit.

From edit of 12-May-91:  (Y27)

1.  Minor help/doc and K11DIA/K11SHO clean up..

From edit of 15-May-91:  (Y28)

1.  Remove crud from RX.XLE in K11ATR, and fixed typo in it where r4 wuz

being restored into r3.

2.  Made L$VAL (ascii number to integer) unsigned, and fixed/sped-up non-EIS

MUL code so it sets the carry bit when it overflows, allowing L$VAL's use of

same for error detection to function.  Also added comments to P$DIV..

3.  Fixed error handling in K11DIA so as not to send an unnecessary abort

string to the modem.

From edit of 19-May-91:  (Y29)

1.  Provide double precision output for seconds subroutine, so transfer

time display works ok past 18:12:15 (>65535 secs past midnight), my mistake!

2.  Added equivalent to SETSIZ K11TSX/T:64., helps on some TSX systems..

From edit of 29-May-91:  (Y30/Y31)

1.  Fixed a couple typos in the comments.

2.  Display an ACK/NAK error message from the remote on a new line.

3.  Extended Kermit's SL editor to advance/backup and delete/undelete

char/word/line, etc.  See HELP SL for details.

4.  (Y31) Minor SL editor cleanup..

From edit of 30-May-91:  (Y31A)

1.  Finish K11HLP.HLP for SL editor.

2.  Fix undel word/line cursor position error on function error.

From edit of 6-Jun-91:  (Y31B)

1.  Clean up misc typos and clarify comments in some modules.

2.  Clean out unused stuff from K11RTE.

3.  Display free memory for handlers in words, not bytes (K11RTI).

Installation and Release Notes	KRT V03.61			Appendix A

From edit of 9-Jun-91:  (Y31C)

1.  Fix SL char processing when inserting chars at end of line.

2.  Clean up reduced packet length due to TT message in K11RTI.

From edit of 11-Jun-91:  (Y32)

1.  Include tentative entries in space shown as free in DIR routine.

Some of may it be tied up at the time, but on the whole this produces

a result closer to what's really available.

2.  Clean up server's opening message, ala C-Kermit.

From edit of 13-Jun-91:  (Y33)

1.  Fix text for HELP SET PROMPT.

2.  Fix SL to backspace/space/backspace when deleting char at end of line.

From edit of 17-Jun-91:  (Y34)

0.  Serious maintenance update, clean up and etc..

1.  Added SET/SHO SETTLE, allows altering settling time between sending a

reset string to the modem and dumping its handler, which also drops DTR.

If your modem doesn't faithfully resume its auto-answer state when exiting

Kermit, increase this value, specified in decimal clock ticks (1/50-60 sec).

From edit of 21-Jun-91:  (Y35)

1.  Changed "SETTLE" to "SETTLE-TIME" ..

2.  Dump yet more unused stuff from program.

3.  Fix problem with ^C abort when a write to the handler is pending.

From edit of 24-Jun-91:  (Y36)

1.  Add settle-time to SHO MODEM.

2.  Extended the incomplete file disposition to work with error packets.

From edit of 28-Jun-91:  (Y37)

1.  Removed control of SL from terminal type, added SET SL ON/OFF/KED/NOKED.

SL no longer defaults ON, it must be explicitly SET on, as in the init file.

SL also clears the keypad application mode as required, instead of doing it

SET TERM..  Added support for PF2 in both NOKED/KED modes.  Reset keypad on

exiting the program.

Installation and Release Notes	KRT V03.61			Appendix A

2.  (Y37A) Fix problem with PF2 trying to open the help file when in HELP

and said file is already open.

From edit of 4-Jul-91:  (Y38)

1.  Fix display of control chars via "^X" notation to conform to DEC's

A..Z[\]~? keys to be struck whilst the control key is depressed.

2.  Dropped the "RT-11" prefix from SET BREAK, CREATE-SIZE, [NO]VOLUME.

3.  Added SHOW REPEAT-QUOTING, SERVER, break to SHOW CONSOLE, CREATE-SIZE

and [NO]VOL to SHOW FILE.

4.  Misc small stuff changed/consolidated/enhanced..

From edit of 7-Jul-91:  (Y39)

1.  Add support for Concord Data 224 modem.

2.  Make dialer echo results direct from modem, so above can type out "..."

and similar call progress stuff from other modems may be seen in real time.

From edit of 15-Jul-91:  (Y39B)

1.  Clean up HELP error handling, comments in K11DIA, other small stuff.

From edit  of 17-Jul-91:  (Y39C)

1.  Misc HELP text and comments clean-up.

From edit of 21-Jul-91:  (Y40)

1.  Conform display for SL when ^C is typed to that of the op system.

2.  More misc small comments/HELP clean-up.

From edit of 22-Jul-91:  (Y41)

1.  Display abort message when file is skipped due to SET FILE PROTECT

(that is, by an "X" or "Z" in the ACK packet) on the other Kermit.

From edit of 7-Aug-91:  (Y42)

1.  Fix a bogus "local" error message header in K11REC.

From edit of 25-Aug-91:  (Y43)

1.  Minor cleanup.

2.  Fix NOTAIL to work with pre-V6.3 TSX per Chester Wilson's TSCLIB and

include in distribution.

Installation and Release Notes	KRT V03.61			Appendix A

From edit of 5-Sep-91:  (Y44)

1.  Use file create date/time/protection in COPY.

From edit of 21-Sep-91:  (Y45)

1.  The rx.xle result in at$len is no longer overwritten by rx.len.  Added

send exact file length in bytes.  This allows sticking an odd-block-length

file into an exactly matching hole, whereas before an extra free block was

needed, due to 1Kb block-size rounding..

From edit of 6-Oct-91:  (Y46)

1.  Clean up pre-V3.49 Kermit-11 bad system attribute length fix.

2.  Query before overwriting an existing file when opening a log file.

3.  Misc. small stuff cleaned up.

From edit 19-Oct-91:  (Y47)

0.  Pre-release clean up.  Commented everything in all source modules,

even a few things not added by me.  Also found and removed a few leftover

or unnecessary things in the program portion of them.

1.  Fix send in remote mode to check for file (and report an error if not

found) before the delay to enable the receiving Kermit, and not to open TT

as the link until after the delay, allowing ^C to abort during said delay.

2.  Prevent PF2 from possibly eating an overlay when it loads HELP by only

letting it work from the main command line.

3.  Add call to .gtim before calling .date to ensure clock is rolled over.

From edit of 23-Oct-91:  (Y47A)

0.  More small stuff fixed, almost entirely concerning correctness of

comments in the source modules.

From edit of 25-Oct-91:  (Y48)

1.  Modified LOGFILE, SET LOGFILE and SET DEBUG commands to each parse an

additional argument for a file name or debug mode as appropriate, allowing

a log file, mode, etc. to all be handled in one single command line, whilst

retaining compatibility with the prior two command scheme of doing things.

See HELP for details.

2.  If modem is on-line when SET SPEED is used, try to re-init the modem.

Installation and Release Notes	KRT V03.61			Appendix A

From edit of 31-Oct-91, V3.61 Pre-Release:  (Y49)

1.  Modified the modem re-init just added to SET SPEED to check for carrier

and not do the re-init when it's present.

2.  Modify incoming file name buffer to accommodate SVID (255. byte) file

names.  Also corrected its definition to bytes instead of words.

3.  Modify SET CL LINE to work on any matching fraction of LINE.  This used

to require at least LIN.

4.  Modify the LOGFILE and SET LOGFILE commands to close an open logfile

if issued with no argument(s).

5.  Added HELP MILNET-TAC ..

From V3.61 Pre-Release, 3-Nov-91:  (Y50)

1.  Kill SL editor during queries added to LOGFILE command requiring a yes/no

response.  Scrolling through stored commands here eats the argbuf containing

an as yet unparsed file name..

2.   SET now does multiple args, separated by commas, ala RT-11, for any one

function.  Example: SET SL ON,KED now works..

From edit Y50A:

3.  Ditto for GET, SEND commands per item #2, just above.

From edit of 15-Nov-91:  (Y51)

0.  This is, barring any unforseen bugs, the V3.61 release edit.

1.  Added display of program type (XM=extended-memory overlays, SJ=low-memory

overlays) to SHO VERSION.

From edit Y51A:

1.  Fix SET PHONE NUMBER so an embedded comma in the argument string is

not eaten.

From edit of 23-Nov-91: (Y52)

1.  Final clean-up.

Installation and Release Notes	KRT V03.61			Appendix A

From edit of 28-Nov-91, V3.61:  (Y53)

1.  Force USR to re-read the directory segment, so floppies can be swapped.

From edit Y53A:

1.  Fix error handling for leading comma in GET/SEND args.

2.  Call binary files "BINARY" not "IMAGE" so MS-Kermit is happy.

3.  Ensure directory search channel is closed on sen.sw errors/aborts.

This it it, V03.61, released 3-Dec-91:

1.  Rename files with KRT prefix to avoid conflict with K11.. files.

A few were renamed as is more appropriate:

KRT     was  K11RT5		KRTERM  was  K11TSX

KRT.INI was  KERMIT.INI		KRTERR  was  K11RTE

KRTCON  was  K11RCO		KRTRMS  was  K11RT4

KRTDIR  was  K11RTD		KRTUTL  was  K11RTU

KRTHLP  was  K11HLP		KRTXL   was  K11PRT

KRTOSI  was  K11RTI

Installation and Release Notes	KRT V03.62			Appendix A

Edit 3.62:

From edit of 11-Jan-92, X03.62:  (third trial)

1.  Increase size of ERRTXT buffer to max prompt + max error message size

in KRTDAT.MAC, and conform its size in KRTDEB.MAC, by placing ERBFSIZ in

KRTMAC.MAC.

3.  In KRTDIA at DODIAL's 50$: add a one tick wait to conform to what happens

when waiting for the modem to echo.

4.  Move CREATE-SIZE and VOLUME-VERIFY to SET FILE, and conform HELP text.

Restore default en$siz=0 after successful use of SET FIL CRE nnn.

5.  Move BREAK-SIZE to SET CONSOLE, and conform HELP text.

6.  Make PRINTM write to log files too.  Ditto for ERROR.  Clean up DEBUG

modes, etc.  See SET$DEBUG and SHO$DEBUG, or use commands and observe the

improvements.  This causes transfer status and error messages to be included

in packet logging.

7.  In KRTRMS, save .ENTER's file size in the file data table.  Also report

same when opening a log file.  SH DEB now displays current block pointer and

size of the file.

8.  Report error when log file fills up (putcr0,putrec,putc).  Add LOGERR

routine in KRTPAK.MAC to cope with this stuff..  Previously the file full

error(s) were just ignored, at the expense of losing all overflowed data.

9.  Fixed C$TYPE to strip hi bit and print the lower 7 bits on terminals

<= VT100, as opposed to just tossing them as in the past.

10.  Dropped NONAME.TMP for a nfs .enter, return "bad file name" error.

11.  Check packet length in dskdmp, don't trap to 4 writing past end

of buffer due to line noise and/or modems retraining.

12.  Fix display formatting when debugging to TT during an extended reply.

Also cleaned up formatting of debug display, including as written to disk.

13.  Redo SET DIAL ECHO and INIT-ONCE as [NO]ECHO and [NO]INIT-ONCE..

14.  Fix a few very small typos and errors in KRTHLP.HLP, expand and

enhance help text for SET DIAL, particularly its commands.

15.  Fixed wildcard "%" match to no longer match on blanks - there must be

some valid char in the appropriate column.

16.  Add current-block-pointer/size-of file to ^A stats display.

17.  Rename SET DIAL PROMPT to WAKE-ACK, add BLIND, PULSE, TONE, DIAL-ACK,

INITIATE-PROMPT, CONFIRM-ACK, [NO]BINARY-RESPONSE (were missing)..

18.  SET SETTLE-TIME is now SET DIAL SETTLE-TIME.  This remains a global

function, and works with pre-defined modems as well.

Installation and Release Notes	KRT V03.62			Appendix A

19.  Added x.result (default "Hayes" xmode) and time.settle defaults,

and in "Hayes" and Telebit modem definitions.

From edit of 12-Jan-92:  (Y03.62.01)

1.  Rewrote and greatly expanded help text for SET DIAL.

From edit of 17-Jan-92:  (Y03.62.03)

1.  Write time of day and process time for each packet at end of same

when debugging packets.

2.  If DCD is high when exiting program, hang up modem.

From edit of 20-Jan-92:  (Y03.62.04)

1.  Modify RDATA to not NAK unknown packets (noise), and SDATA to wait

through bad ACK packets, noise, etc..

2.  Make BUFFIL limit test max-0 (was max-4), allows bigger packets.

From edit of 21-Jan-92:  (Y03.62.05)

1.  Don't log bogus data for timout.

From edit of 23-Jan-92:  (Y03.62-7)

1.  Misc small stuff cleaned up in previously added V03.62 items.

From edit of 3-Feb-92:  (Y03.62-9)

1.  Make ERROR send an error packet, use PRINTM elsewhere.

2.  Fix KRTREC.MAC and KRTSEN.MAC error handling, extensive mods..

From edit of 5-Feb-92:  (Y03.62-10)

1.  More liberal retry on noise during file xfrs scheme.

2.  Default TIME-OUT made 5 secs.

From edit of 6-Feb-92:  (Y03.62-11)

1.  Clean up above noise handling routines.

From edit of 11-Feb-92:  (Y03.62-12)

1.  Further clean up above noise handling routines.

2.  Don't modify SET time-out value no matter what remote's is.

3.  Display any difference between local and remote time-outs.

Installation and Release Notes	KRT V03.62			Appendix A

From edit of 20-Feb-92:  (X03.62-13)

1.  Add bells: up arrow when nothing saved yet, down arrow at end.

2.  Add/enforce SET SEND PACKET-LEN limit, which is now in KRTST1.MAC.

3.  Major cleanup and maintenance update.

4.  Conform to major sinit stuff update in KRTINI.MAC

5.  Numerous small fixes and removal of redundant code..

From edit of 21-Feb-92:  (X03.62-14)

1.  Move SET TIME-OUT to SET RECEIVE TIME-OUT and SET SEND TIME-OUT.

From edit of 21-Feb-92:  (X03.62-15)

1.  Drop time-outs display, other Kermit's value is not always received

before send or receive mode is selected, thus it's not always correct.

2.  Minor code cleanup.

From edit of 23-Feb-92:  (X03.62-16)

1.  Corrected suggested XL BUFSIZ to 256. (sorry about this!).

2.  Leave channels to comm handler open all the time after it has been

assigned the link device.

From edit of 27-Feb-92:  (X03.62-17)

1.  Move senbrk to KRTCON.MAC.

From edit of 27-Feb-92:  (X03.62-18)

1.  Reduce size of XM root.

From edit of 5-Mar-92:  (X03.62-19)

1.  Minor clean up.

From edit of 18-Mar-92:  (Y03.62-20)

1.  Another minor clean up..

From edit of 23-Mar-92:  (Y03.62-21)

1.  Include last number dialed in SH PH (if one exists).

2.  Time-outs are now 7 secs rx, 10 secs tx.  SET REC TIME-OUT 0 is a good

idea when downloading from Compuserve.

Installation and Release Notes	KRT V03.62			Appendix A

From edit of 24-Mar-92:  (Y03.62-22)

1.  Added selectable .csispc comma delimiter parsing, see HELP SET FILE CSI.

From edit of 25-Mar-92:  (Y03.62-23)

1.  Clean up and speed up the above .csispc mods..

From edit of 31-Mar-92:  (Y03.62-25)

1.  Speed up dir name match testing.

From edit of 1-Apr-92:  (Y03.62-27)

1.  Yet more code cleaned up..

From edit of 11-Apr-92:  (Y03.62-31)

1.  Increase efficiency of KRTDIR.MAC yet again.

2.  Increase size of fixfil stack buffer to SVID limit.

From edit of 3-May-92:  (Y03.62-33)

1.  Still more minor clean up work..

2.  Make TYPE append a LF to CR if CR not followed by one..

From edit of 9-May-92:  (Y03.62-37)

1.  Clean up various macros.

From edit of 25-May-92:  (Y03.62-39)

1.  Move sho$ve after kerini so xprint is cleared first.

2.  Add dummy file name for HPRINT via RT-11 V5.2/TSX V6.0 LP/LS.

3.  Fixed virtual segment 1 size in non-EIS XM version.

From edit of 26-May-92:  (Y03.62-40/41/42)

1.  (Y40) Keep XL open even for XM, to accommodate RT-11 V5.6.

2.  (Y41) Drop auto-assign of XC during PRO-300 startup, in light of above..

3.  (Y42) Increase MAXLNG (long-packet size) to 1920.

From edit of 27-May-92:  (Y43)

1.  Clean up above mods..

Installation and Release Notes	KRT V03.62			Appendix A

From edit of 29/30-May-92:  (Y44/45)

1.  (Y44)  HELP extended wildcarding in progress, otherwise this is a stable

version.

2.  (Y45)  Finish up display all subtopic text when wildcarding through nested

text.

From edit of 31-May-92:  (Y47)

1.  Clean up recent additions and edits.

From edit of 3-Jun-92:  (Y48)

1.  Reinstall support for KL handler.  Note the HELP text isn't yet done..

2.  Fix SHO VERSION so SJ doesn't zap its number, added (non-)EIS to display.

3.  Return correct error message for non-existent device.

From edit of 5-Jun-92:  (Y49)

1.  Add RT-11 version checking, bypass V5 calls so V4 will run.  This kills

doing file create date and protection from Kermit (ala received attributes)

and the abort I/O call to the comm handler (which is first flushed by looping

regardless).

2.  Expand V5 macros so everything will assemble under V4, even though most

of it will never run..

3.  Add support for 38.4kb.  This requires hardware capable of doing it..

From edit of 7/9-Jun-92:  (Y51..53)

1.  Extend, finish and clean up recent work.

From edit of 11-Jun-92:  (Y55)

1.  Speed up keyboard input processing at high data rates.

2.  Add SET CONSOLE PRIORITY for KL/XC/XL under TSX.

From edit of 13-Jun-92:  (Y56)

1.  Speed up CL/XL/etc reads from the modem.

2.  Add Telebit T3000 modem definitions.

From edit of 14-Jun-92:  (Y57)

1.  Double the settle-time value for the wait after dropping DTR, as

some modems (including the T3000) need more time to recover.  Note when

changing the modem type the new default will overwrite any previously

SET value.

Installation and Release Notes	KRT V03.62			Appendix A

From edit of 16-Jun-92:  (Y59)

1.  Dumped the concept of KL (more or less XL-compatible) and created a

new handler called KM ("Kermit Modem" driver) which is more TSX-like, and

is thus easier to unhang, smaller, hopefully faster and etc.  See HELP

INSTALL KM for details.  This handler does NOT support VTCOM nor is it

intended to be used with anything besides KRT Y03.62-59 or future edits.

From edit of 18-Jun-92:  (Y60)

1.  Set virtual bit in JSW for XM assembly, which inadvertently got dropped.

2.  Apply the final solution to the parity memory vector eating the version

data in block 0.  See top of KRTCMD.MAC for details..

From edit of 19-Jun-92:  (Y61)

1.  Add hardware (RTS/CTS) flow control to the KM handler, and support for

same to Kermit, including making the T3000 modem definition use it.  You

must explicitly SET FLOW-CONTROL RTS to enable this (as in the init file).

From edit of 21-Jun-92:  (Y63)

1.  Reduce MAXLNG to 600. for NONEIS assembly, so the program has some

hope of loading under FB..  This of course restricts the long-packet length

to one block, but is better than nothing, until and unless other reductions

can be made.  It was done with the NONEIS flag for convenience.  See MAXLNG

in KRTMAC.MAC if this is not to your taste..

From edit of 23-Jun-92:  (Y64)

1.  Reduce ALSIZE (phone # buffer) to 512. bytes and LN$CNT (SL # of

lines saved) to 3 under the non-EIS assembly, so now KRT will run on

anybody's FB system with room enough left to load a comm handler, etc.

This applies to KRT.SAV as distributed with this program.  You may feel

free to tweak these as needed in KRTMAC.MAC.

2.  Ensure I/O to block 0 is done when initting KM, XC, XL, so SJ queue

gets reset.  This was causing various problems with these handlers..

From edit of 29-Jun-92:  (Y65)

0.  Clean up recent edits..

From edit of 30-Jun-92:  (Y66)

1.  Moved GETREC to KRTRMS from KRTUTL, so HELP via PF2 can't ever crash..

From edit of 3-Jul-92:  (Y67)

1.  Add Jim Whitlock's VBGEXE fetch pointer fixes.

Installation and Release Notes	KRT V03.62			Appendix A

From edit of 9-Jul-92:  (Y69)

1.  Add DCDTST for status during file transfers, so one will know when

one's data path via a modem has died..

2.  Ensure hi byte of time-out value is cleared in RPACK$.

From edit of 11-Jul-92:  (Y70)

1.  Fix flipped RTS and CTS status display flags in KM handler, and

SET FLOW-CONTROL code problem getting back to the XOFF mode.

2.  Dumped SET DIAL INFO, as any undefined message defaults to it and

as this program gets bigger the memory recovered is more than worth it.

From edit of 12-Jul-92:  (Y71)

1.  Misc cuts in FB root size, saving another 387. words..

From edit of 19-Jul-92:  (Y73)

1.  Fix DIR error handling for non-initialized disk.

2.  Enhance display of take file errors, and fix some related bugs.

3.  Command line buffer length (LN$MAX) is now back to 80. bytes.

From edit of 23-Jul-92:  (Y75)

1.  Yet more cuts to the root.

From edit of 26-Jul-92:  (Y76/77)

1.  Even more cuts to the root, added passed buff length to rpack and

also cut a big piece out of the overlays too.

2.  Add SHOW EOF.

From edit of 29-Jul-92:  (Y78)

1.  Fixed titles of .MAC files so XREF gets the file names.

2.  Moved $limit into the root so it links ok under separate I&D space.

3.  Dropped ndf tests in macros as it blows up the .LST output.

From edit of 31-Jul-92:  (Y79)

1.  Skip .ABTIO for XC/XL under ALL RT-11 V5.6 monitors to avoid hang/race/?

From edit of 2-Aug-92:  (Y82)

1.  Clean up error handling.

Installation and Release Notes	KRT V03.62			Appendix A

From edit of 3-Aug-92:  (Y83)

1.  Document use of LINK's /Extend switch and KRTTSX's handler buffer size.

From edit of 7-Aug-92:  (Y85)

1.  $$trip hack to avoid "ndf/df" tests blowing up .LST output.

2.  Yet more cuts to the root for use under RT-11FB.

From edit of 9-Aug-92:  (Y87)

1.  More clean-up work..

From edit of 29-Aug-92:  (Y88)

1.  Yet more clean-up, fixed HOME which Y87 screwed up.

From edit of 17-Sep-92:  (Y89)

1.  Add 25% to rec'd length for text files from non RT-11/TSX systems,

as others don't include record terminators and etc in the passed length.

2.  Fix .LST output of macros when code not producing binary output is

inserted.

3.  Once again fix free core pointer stuff for VBGEXE and separate I&D space.

From edit of 16-Oct-92:  (Y90)

1.  Fix various spelling errors.

From edit of 23-Oct-92:  (Y91)

1.  Fix more spelling and grammar errors.

2.  Add SET FILE [NO]REPLACE.

3.  Indicate possible device not responding in handler error messages.

4.  Fix BUFEMP's error handling.

5.  Add SHOW CL.

From edit of 1-Nov-92:  (Y93)

1.  Display any possible contents of "X" packet, for Unix and C-Kermit.

From edit of 5-Nov-92:  (Y94)

1.  Make hd$fir=:6 so badly initted disks don't crash DIR.

2.  Unpack repeat-encoded chars when displaying data in an "X" packet.

Installation and Release Notes	KRT V03.62			Appendix A

From edit of 18-Nov-92:  (Y95)

1.  Fix error reporting when modem dies during a keyboard abort when dialing.

2.  Fix problem dispatching I/O via TT under RT-11.

From edit of 3-Dec-92:

1.  This is it, V03.62.

2.  V03.62-1 adds Jim Whitlock's separate I&D space LINK command files,

and fixes numerous spelling and syntax errors.  No functional change from

V03.62.

From edit of 19-Mar-93:  V03.62-2

1.  Fix problem with error handler overwriting APR6.

From edit of 30-Apr-93:  V03.62-3

1.  Use clock rate for 0.5 sec status checks in CONNECT mode.

2.  Include RT-11 V5.6 monitor changes in documentation.

3.  Clean up dialer module, misc other small things cleaned up as well,

including some small USER-DEFINED modem bugs.

4.  Add automatic 50Hz clock rate detection and support for watching more

than 34. lines to NOTAIL (KRTAIL), just in case anyone ever achieves this..

5.  Fixed check for indirect command file or detached job running program, for

error handling.

From edit of 31-May-93:  V03.62-5

1.  Add SET PHONE [NO]ANSWER and SET DIAL [NO]ANSWER to support call back

modems.  Currently in the Hayes, Telebit and User-Defined definitions, others

either go idle on their own or need a manually entered command after the

remote hangs up (consult your modem's documentation).

2.  Fixed error handling when Kermit is being run by a command file, IND

process, as a detached job, etc..  Also updated KRT.IND by removing things

Kermit itself can now do.

From edit of 4-Jul-93:  V03.62-7

1.  Fix problem introduced in V03.62-5 wherein XL couldn't be dumped on exit.

2.  Allow the server to run through a comm handler.

3.  Add everything missing to SHOW - now, if you can SET it, you can SHOW it.

4.  Started on individual on/off control of file attribute processing, but

stopped as it would overflow 3 RX02/I:1 floppies, adding a disk for just

about nothing.  This will be in the next release along with INPUT and OUTPUT

commands, etc. worth the fourth disk..

From edit of 27-Jul-93:  V03.62-8

1.  Fix missing psect in KRTRMS.MAC.

2.  Add note concerning "dma=no" error when building KM.  See HELP INSTALL KM.

3.  This is absolutely, positively it for V03.62 ..

Billy Y..

27-Jul-93

-eof-