SZ(1)              Omen Technology INC (OMEN)               SZ(1)

NAME

sz - ZMODEM file send

zcommand, zcommandi - ZMODEM command download

SYNOPSIS

sz [-+abdEefgIikLlMmNnoPpqtuvxyYZ] file ...

zcommand [-oqtv] COMMAND

zcommandi [-oqtv] COMMAND

sz -T

DESCRIPTION

This set of commands uses the ZMODEM, YMODEM or XMODEM error

correcting protocol to send files to a program logged into a

serial port.  (See NETWORK NOTES, below.)

This version of sz operates with Omen Technology products,

and is licensed only for use therewith.

Commercial sz and rz for use with any program supporting

XMODEM, YMODEM, or ZMODEM are available for purchase from

Omen Technology INC.

The program is not designed be called from cu, tip, kermit,

or any other communications program.  Unix flavors of Omen

Technology's Professional-YAM communications software are

available for dial-out applications.

Sz sends one or more files with ZMODEM protocol.

Zcommand sends a single COMMAND to a ZMODEM receiver for

execution.  Zcommand exits with the COMMAND return value.

If COMMAND includes spaces or characters special to the

shell, it must be quoted.

Zcommandi sends a single COMMAND to a ZMODEM receiver for

execution.  Zcommandi exits as soon as the receiver has

correctly received the command, before it is executed.

The sixth form (sz -T) outputs all 256 code combinations to

the terminal.  The hex value of the character, a space, and

the raw character are sent, 8 per line.  In you are having

difficulty sending files, this command lets you see which

character codes are being eaten or corrupted by the

operating system or network.  The terminal program must be

set to display all 256 codes in a distinct form.  Check that

Page 1                                          (printed 1/21/95)

SZ(1)              Omen Technology INC (OMEN)               SZ(1)

each code is sent, and that the 8th bit on codes 0x80 to

0xFF is set.

Sz -T then allows the caller to input characters which are

echoed in hex.  You should check that all control characters

can be input this way.

The meanings of the available options are:

\    (backslash) (VMS) Force the next option letter to upper

case.

+    Instruct the receiver to append transmitted data to an

existing file (ZMODEM only).

a    Instruct the ZMODEM receiver to convert text file

format as appropriate for the receiving system.  When

sending to a PC-DOS system, the receiving ZMODEM

program converts END OF LINE to a CR/LF pair.

b    (ZMODEM) Binary override: transfer file without any

translation.

bb   (bogus binary) On VMS reads disk files with an

alternate mode.  Suitable for a few strange file types

such as Lotus .wk3 spreadsheet files.

c    Instruct the receiver to change the pathname if the

destination file exists.

d    Change all instances of "." to "/" in the transmitted

pathname.  Thus, C.omenB0000 (which is unacceptable to

MSDOS or CP/M) is transmitted as C/omenB0000.  If the

resultant filename has more than 8 characters in the

stem, a "." is inserted to allow a total of eleven.

e    Escape all control characters; normally XON, XOFF, DLE,

CR-@-CR, and Ctrl-X are escaped.

E    Escape the 8th bit to allow operation over 7 bit paths.

Witnout the P option, data is compressed.

EP   Pack 4 data bytes into 5 bytes for operation over 7 bit

paths.  This is more efficient when the data is already

compressed.

f    Send Full pathname.  Normally directory prefixes are

stripped from the transmitted filename.

ff   Send Full VMS pathname including device name and

version number.

g    Send ESC-[-?-3-4-l to start DSZ from Kermit.

i    Make VMS receiver use Stream_LF record format to avoid

padding file with nulls.  This record format is

understood by many C programs.

I    Make VMS receiver use Variable length (mrs=510) image

record format to avoid padding file with nulls.  This

binary record format is understood by many Kermit

programs.

LN   Use ZMODEM sub-packets of length N.  A larger N (32 <=

N <= 1024) gives slightly higher throughput, a smaller

N speeds error recovery.  The default is 128 below 300

Page 2                                          (printed 1/21/95)

SZ(1)              Omen Technology INC (OMEN)               SZ(1)

baud, 256 above 300 baud, or 1024 above 2400 baud.

lN   Wait for the receiver to acknowledge correct data every

N (32 <= N <= 1024) characters.  This may be used to

avoid network overrun when XOFF flow control is

lacking.

m    Request MobyTurboTm throughput enhancement.

M    Lock out MobyTurboTm throughput enhancement.

n    (ZMODEM) Send each file if destination file does not

exist.  Overwrite destination file if source file is

newer than the destination file.

N    (ZMODEM) Send each file if destination file does not

exist.  Overwrite destination file if source file is

newer or longer than the destination file.

o    (ZMODEM) Disable automatic selection of 32 bit CRC.

p    (ZMODEM) Protect existing destination files by skipping

transfer if the destination file exists.

q    When sz starts up, sz counts the files to be sent and

totals their lengths.  This information allows the

receiver to display the progress in a multiple file

(batch) transmission.  When many files are specified

(as with wild cards) this takes some time.  The q

(quiet) option bypasses this count.

r    (ZMODEM) Resume interrupted file transfer.  If the

source file is longer than the destination file, the

transfer commences at the offset in the source file

that equals the length of the destination file.

rr   As above, but compares the files (the portion common to

sender and receiver) before resuming the transfer.

ttim Change timeout to tim tenths of seconds.

u    Unlink the file after successful transmission.  Users

should not manually abort or skip file transfers when

this option is used.

wN   Limit the transmit window size to N bytes (ZMODEM).

The sz program will interrogate the receiver and pause

if necessary to let the receiver catch up to within the

specified window size.  This option is useful when

sending long files over networks that time out if the

user does not enter commands frequently.

v    Verbose causes a list of file names to be appended to

/tmp/szlog (szlog.tmp on VMS).  More v's generate more

detailed output.

x    Count skipped files as errors.

y    Instruct a ZMODEM receiving program to overwrite any

existing file with the same name.

Y    Instruct a ZMODEM receiving program to overwrite any

existing file with the same name, and to skip any

source files that do have a file with the same pathname

on the destination system.

Z    Use ZMODEM file compression to speed file transfer.

DIAGNOSTICS

Exit status is as follows: 0 for successful transfers.  1 if

Page 3                                          (printed 1/21/95)

SZ(1)              Omen Technology INC (OMEN)               SZ(1)

unrecoverable errors are detected.  2 if syntax errors or

file access problems are detected.  3 if the program was

terminated by a caught interrupt.

EXAMPLES

ZMODEM File Transfer (Unix to DSZ/ZCOMM/Professional-YAM)

% sz -a *.c

This single command transfers all .c files in the current

Unix directory with conversion (-a) to end of line

conventions appropriate to the receiving environment.  With

ZMODEM AutoDownload enabled, Professional-YAM  and ZCOMM

will automatically receive the files after performing a

security check.

% sz -Yan *.c *.h

Send only the .c and .h files that exist on both systems,

and are newer on the sending system than the corresponding

version on the receiving system, converting Unix to DOS text

format.

$ sz "-Yan" file1.c file2.c file3.c foo.h baz.h (for VMS)

ZMODEM Command Download (Unix to Professional-YAM)

cpszall:all

zcommand "c:;cd /yam/dist"

sz -ya $(YD)/*.me

sz -yqb y*.exe

zcommand "cd /yam"

zcommandi "!insms"

This Makefile fragment uses zcommand to issue commands to

Professional-YAM to change current disk and directory.

Next, sz transfers the .me files from the $YD directory,

commanding the receiver to overwrite the old files and to

convert from Unix end of line conventions to PC-DOS

conventions.  The third line transfers some .exe files.  The

fourth and fifth lines command Pro-YAM to change directory

and execute a PC-DOS batch file insms . Since the batch file

takes considerable time, zcommandi form is used to allow sz

to exit immediately.

XMODEM File Transfer (Unix to Crosstalk)

% sx foo.c

ESC

rx foo.c

The above three commands transfer a single file from Unix to

a PC and Crosstalk.  This combination is much slower and

less reliable under stress than ZMODEM.

NOTES

ZMODEM greatly simplifies file transfers compared to XMODEM.

In addition to a friendly user interface, ZMODEM provides

Personal Computer and other users an efficient, accurate,

and robust file transfer method.

Page 4                                          (printed 1/21/95)

SZ(1)              Omen Technology INC (OMEN)               SZ(1)

ZMODEM provides complete END-TO-END data integrity between

application programs.  ZMODEM's 32 bit CRC catches errors

that sneak into even the most advanced networks.

Advanced file management features include AutoDownload

(Automatic file Download initiated without user

intervention), Display of individual and total file lengths

and transmission time estimates, Crash Recovery, selective

file transfers, and preservation of exact file date and

length.

The -y option instructs the receiver to open the file for

writing unconditionally.  The -a option causes the receiver

to convert Unix newlines or VMS variable length records to

the receiving system's standard text format (carriage

returns and linefeeds for PC-DOS).

On VMS and Unix systems, sz and sb send the length and date

of each file.  If the receiving program uses this

information, the transmitted file length controls the exact

number of bytes written to the output dataset, and the

modify time and file mode are set accordingly.

SERVER/NETWORK NOTES

Terminals on some timesharing systems are connected

indirectly to the host with TERMINAL SERVERS operating over

networks.  Terminal servers often interfere with file

transfers by "eating" control characters and/or losing data.

Accessing a distant system using telnet on a local Unix

system entails the same problems.  If telnet does not work

well, try rlogin with the -8 option.  Set the escape

character to ^P (control P) which is protected by ZMODEM.

When terminal servers interfere with file transfers, server

commands are often available to help the problem.  When

possible, enable the server and modems for hardware flow

control in and out.  Set the terminal server to telnet

transparent mode.  Some typical commands are "SET TERMINAL

DOWNLOAD", "set session passall", and "SET TERMINAL NO ESC",

but there is no standard set of commands suitable for all

terminal servers.  Normally these commands are given before

attempting file transfers, but one user has reported that

the command must be given after the file transfer is

started(!).  Until terminal server vendors improve the

quality of their product, you may have to get on their case

until they adequately support high performance file

transfers.

Escaping control characters with ZMODEM (sz -e) may help if

the transfer gets stuck in the same place on every attempt.

Page 5                                          (printed 1/21/95)

SZ(1)              Omen Technology INC (OMEN)               SZ(1)

Professional-YAM and ZCOMM allow selection of which control

characters need to be escaped.  In extreme cases 7-bit

transmission may be required (see the Professional-YAM/ZCOMM

manual).  If you encounter problems with control characters

you should identify which control characters are causing the

problem.  Please refer to the sz -T command in sz.doc for

more help.

Flow control between the server and modem is often

defective.  This rarely causes a problem in interactive

applications whose short bursts (a screenfull at most) fit

within the available memory buffers.  Streaming protocols

such as YMODEM-g, long packet SuperKermit, and ZMODEM can

overload the available buffering.  Some terminal servers

support commands to enable hardware flow control such as

"TERM FLOWCONTROL HARDWARE IN OUT"; this assumes the modem

attached to the server is properly configured.

If the terminal server is too brain damaged to support flow

control, ZMODEM lets you fake it by setting a window size.

Try

sz -w2048 file...

to enable software flow control.  Experiment with different

window sizes for best results.

Terminal servers are designed to accept keyboard input from

human operators.  They may lose data when a program sends

more data to the host faster than humans type.  Some servers

allow larger input buffers to support file uploads.  If you

can upload short files (less than 100 bytes) but not longer

files, try

sz -w1024 file...

with your uploading program.  (Not all programs support this

option; use ZCOMM or Professional-YAM.)

When both flow control and character transparency are

problems, use

sz -ew1024 file...

as a starting point.

Sometimes the terminal server software is defective and must

be corrected.  Please contact the network's vendor for

corrective procedures.  An alternative is to install direct

serial ports for users desiring to upload files.

We have also encountered problems when networking software

fails to pass data after a large amount of data has been

sent to the host.  This has happened on a PPP internet

connection which prevented uploads of large amounts of data

via either FTP or a ZMODEM upload (via TELNET).  The PPP

connection grinds to a standstill and not even PING works.

The same ZMODEM/TELNET combination easily uploaded ten times

Page 6                                          (printed 1/21/95)

SZ(1)              Omen Technology INC (OMEN)               SZ(1)

as much to a nearby machine connected by Ethernet.

PORT WATCHERS

Some systems enforce a timeout which disconnects a user

after several minutes of keyboard inactivity.  Long, error

free ZMODEM transfers do not generate keyboard activity

(this is why ZMODEM is so efficient).  Restricting the

window size with

sz -w16384 file...

option generates terminal input at regular intervals even

when no error correction is needed.  If problems persist,

try setting a packet length.

sz -l1024 file...

(The default ZMODEM packet length is the file length.)

SEE ALSO

Professional-YAM, rz(omen), crc(omen), sq(omen),

todos(omen), tocpm(omen), tomac(omen)

VMS NOTES

To use sz a DCL verb must be defined with syntax similar to

the following:

sz :== "$disk$user2:[username.subdir]sz.exe sz"

For example, if "dir sz" results in:

Directory SYS$SYSDEVICE:[CAF]

SZ.EXE;41

then you should define sz:

$sz :== "$sys$sysdevice:[CAF]vsz.exe sz"

Other verbs are:

$zcommand  :== "$sys$sysdevice:[CAF]vsz.exe zcommand"

$zcommandi :== "$sys$sysdevice:[CAF]vsz.exe zcommandi"

Wild cards are expanded within sz.

sz -a [*...]*.h sends all accessible .h files in the system.

VMS DCL forces command line arguments to upper case unless

they are enclosed by double quotes.  Upper case option

letters may be represented by preceding the letter with \.

When a VMS program exits back to DCL, it returns the results

to DCL in the global symbol $status.  A value of 1 indicates

normal completion, otherwise a value of 2c is returned.

This may be displayed with the command "show symbol

$status".

VMS C Standard I/O and RMS may interact to modify the file

contents.  File lengths are not accurately reported for some

VMS record formats; When files are sent with XMODEM or

YMODEM, file length is measured by counting the characters

therein.

Page 7                                          (printed 1/21/95)

SZ(1)              Omen Technology INC (OMEN)               SZ(1)

For efficient operation at high baud rates, the system

manager should "set term tta0:/altypeahd/perm" (choose the

appropriate tty port).  This must be done when the

particular terminal line is not logged in.  Without setting

altypeahd you may get excessive retries and/or lousy

throughput.

In addition the system manager can try increasing the SYSGEN

parameter TTY_TYPAHDSZ to, say, 2048 or more.

In order to get the maximum speed the process quotas

buffered I/O byte count should be 30000 or more and buffered

I/O limit should be 100 or more.

To see what you have type:

$ show process/quotas

To change this type:

$ set default sys$system

$ run authorize

modify/BIOlm=100/Bytlm=30000 caf       (or whatever your

username is)

exit

Now log out and log back in to get the new quotas.

UNIX NOTES

Iff sz is invoked with $SHELL set and iff that variable

contains the string rsh or rksh (restricted shell), sz

operates in restricted mode.  Restricted mode restricts

pathnames to the current directory and PUBDIR (usually

/usr/spool/uucppublic) and/or subdirectories thereof.

FILES

/tmp/szlog stores debugging output (sz -vv) (SZLOG.TMP on

VMS).

32 bit CRC code courtesy Gary S. Brown.

TESTING FEATURE

The command "sz -T file" exercises the Attn sequence error

recovery by commanding errors with unterminated packets.

The receiving program should complain five times about

binary data packets being too long.  Each time sz is

interrupted, it should send a ZDATA header followed by

another defective packet.  If the receiver does not detect

five long data packets, the Attn sequence is not

interrupting the sender, and the Myattn string in sz.c must

be modified.

Page 8                                          (printed 1/21/95)

SZ(1)              Omen Technology INC (OMEN)               SZ(1)

After 5 packets, sz stops the "transfer" and prints the

total number of characters "sent" (Tcount).  The difference

between Tcount and 5120 represents the number of characters

stored in various buffers when the Attn sequence is

generated.

NOTES

Sz is not designed be called from cu(1) or other outdial

communications programs.  Unix and VMS flavors of Omen

Technology's Professional-YAM communications software are

available for dial-out applications.

When using TrailBlazer or other buffered modems at high

speed, particular attention must be paid to flow control.

The modem and Unix must agree on the flow control method.

Sz on USG (SYS III/V) systems uses XON/XOFF flow control.

If flow control cannot be properly set up, Try a "-w2048"

option to enforce protocol level flow control.  Experiment

with different window sizes for best results.

Telebit modems must not be set to "spoof" UUCP, XMODEM, or

KERMIT.  Setting one of these spoofing modes interferes with

other protocols.

Circular buffering and a ZMODEM sliding window should be

used when input is from pipes instead of acknowledging

frames each 1024 bytes.  If no files can be opened, sz sends

a ZMODEM command to echo a suitable complaint; perhaps it

should check for the presence of at least one accessible

file before getting hot and bothered.

A few high speed modems have a firmware bug that drops

characters when the direction of high speed transmission is

reversed.  The environment variable ZNULLS may be used to

specify the number of nulls to send before a ZDATA frame.

Values of 101 for a 4.77 mHz PC and 124 for an AT are

typical.

BUGS

On at least one BSD system, sz would abend it got within a

few kilobytes of the end of file.  Using the "-w8192" flag

fixed the problem.  The real cause is unknown, perhaps a bug

in the kernel TTY output routines.

The test mode leaves a zero length file on the receiving

system.

LICENSE

Use of this software for commercial or administrative

purposes except when exclusively limited to interfacing Omen

Technology products requires a license from Omen Technology

Page 9                                          (printed 1/21/95)

SZ(1)              Omen Technology INC (OMEN)               SZ(1)

Inc.   Use of this code by inclusion, decompilation, reverse

engineering or any other means constitutes agreement to

these conditions and acceptance of liability to license the

materials and payment of any reasonable legal or

investigative costs necessary to enforce this license

agreement.

Copyright 1993 Omen Technology INC All Rights Reserved

Omen Technology INC

Post Office Box 4681

Portland OR 97208

503-621-3406

Page 10                                         (printed 1/21/95)