MEMPAT  X4-008

(Beware the Jabberwock...)

_Base address?

enable write

disable write

expression=		evaluate expression and display result

,address not changed

Reset stack

Zero base address	use current address as the value for Z

^			address = address - increment

^@			pop, address = saved_address

^@^			pop, address = saved_address - increment

@			push <address + increment>

,address = q_value

@*			push <address + increment>

,address = entry_address

@expression		push <address + increment>

,address = expression

<return>		address = address + increment

>			address = address + increment

>expression		address = address + expression

<			address = address - increment

<expression		address = address - expression

I			increment = default_increment

,address = address + increment

Iexpression		increment = expression

,address = address + increment

.		current address

q		last displayed value (contents)

z		current zero base value

%Xvalue		use hexadecimal radix (the default)

%Dvalue		use decimal radix

%Ovalue		use octal radix

%Bvalue		use binary radix

+		addition	}

-		subtraction	} all operations are on unsigned

*		multiplication	} longword integers

/		division	}

_		logical NOT

!		logical OR

&		logical AND

#		logical XOR