POSTSCRIPT FOR TECHNICAL DRAWINGS

PSPLOT: A FORTRAN-CALLABLE

POSTSCRIPT PLOTTING LIBRARY

USER'S MANUAL

Kevin E. Kohler

Nova Southeastern University

Oceanographic Center

8000 North Ocean Drive

Dania, Florida  33004

1.   INTRODUCTION

This manual is intended for users interested in generating two-dimensional technical drawings or graphics for technical journals in PostScript format.  The manual describes a library of Fortran-callable subroutines which can be combined in a calling program to produce PostScript plot files.  The purpose, syntax, and calling convention of each subroutine is presented , along with plotting examples.

PostScript is a very  robust graphics and typesetting language with wide-ranging capabilities.  Since the focus of the subroutines in this library is to produce technical drawings, many of the "artistic" features of PostScript have not been addressed or included here.

The name PostScript is a registered trademark of Adobe Systems Incorporated.  All instances of the name PostScript in this manual are references to the PostScript language as defined by Adobe Systems Incorporated.

Send questions, comments, suggestions to kevin@ocean.nova.edu.

2.   PLOTTING BASICS

This section will present the conventions used by the plotting subroutines in the PSPLOT plotting library .  This information can be used to create customized plotting subroutines.

A plotting session is a set of plotting instructions in a user's application program which produces hardcopy graphic display output.  The output can be either a single plot or graph or a set of graphs.  Every plotting session must begin with a call to subroutine PSINIT (with the exception of a call to NEWDEV) and end with a call to subroutine PLOTND.

Pen movement is occasionally mentioned throughout this manual.  While PostScript does not use an actual pen for graphic production, it is useful to visualize the plot commands as directing the movements of a pen of variable thickness to a specified (x,y) coordinate, with the pen being either up or down.  The plotting subroutines are simple, user-callable commands which direct the movements of an imagined pen upon a plotting sheet.

The initial coordinate origin is approximately .5 inches from the bottom and left paper edges.  The orientation of the page must be specified to be either portrait (short side horizontal) or landscape (long side horizontal) and is set by the call to PSINIT (which see).  The figure below shows the paper orientation and beginning plot origin for portrait and landscape modes.  The current plot origin can be relocated to other positions during the plotting session to provide new reference points for subsequent plotting commands.  When the next graph is started, the new origin should be placed far enough away to avoid overlapping the just-completed graph.

All plotting commands use an absolute plot coordinate system.  This means that all coordinates passed to plotting subroutines are expected to be in terms of distance from the current plot origin.  This is contrast to a relative plot coordinate system, in which coordinates are assumed to be in terms of distance from the current pen position.

In all plotting subroutines, plot coordinates, character heights, distances, etc. are measured in inches.

Unless otherwise noted, any subroutine argument which specifies an angle is stated in degrees relative to the X axis, with positive angles measured counterclockwise from the X axis.

Most character variables are specified as Hollerith, rather than character strings. The reason for this is primarily historical, combined with the fact that the enormous amount of code already written and in use at my site precluded a comprehensive revision.  This should not cause any problems, however.

The initial font of a plot session is Helvetica, with a size of 12 points.  This can be changed permanently in subroutine PSINIT.  Of course, the current font can always be changed with subroutine SETFNT. PSPLOT supports the standard 35 fonts found on most PostScript printers.

PSPLOT supports color, although all examples in this manual are shown in grayscale for the purposes of reproduction.  Color is specified in the relevant subroutines as red, green, and blue (RGB) values.

Continuation allows you to append a character string or number to the end of a previously plotted string or number.  The coordinates of the appended string are automatically calculated.  Whether or not a  subroutine supports continuation is stated in subroutine description in the next section.

Continuation is specified by setting the X and/or Y coordinate argument in calls to the subroutines listed above to 999, and may be applied to X and Y coordinates separately.  A subroutine call with continuation must immediately follow the previous plotting call.  Continuation is useful when plotting strings containing variable values, such that the resultant string length is not known beforehand.

For example,

ND=10

CALL KEKNUM (1., 2., .15, FLOAT(ND), 0., -1, 0)

CALL KEKSYM (999., 999., .15, 5H DAYS, 0. ,5, 0)

produces "10 DAYS".

3.   USER-LEVEL PLOTTING ROUTINES

This section describes the subroutines in the PSPLOT plotting library at the time of this writing.  These routines are Fortran-callable from an application program.  Below is a brief summary of the subroutines, followed by an alphabetical listing of each subroutine containing a more detailed description of its function, syntax, and calling arguments.

ARC	Draws an arc of a specified radius and center.

AROHED	Draws an arrowhead at a specified location.

ARROW	Draws an arrow including the shaft.

AXIS	Draws an axis of a graph, with user-specified axis titles.

BLKSTP	Strips out embedded blanks in a character string.

BORDER	Draws a rectangular border with tick marks.

CHOPIT	Logically closes  the current page and begins a new one.

CIRCLE	Draws a circle with specified center, radius and fill.

CLIP	Inserts the PostScript command clip into the output file.

CLIPBOX	Defines the current clipping area.

COLBOX	Fills regions with specified color.

CONCOLR	Draws colored contour plots of a 2-dimensional array of regularly-spaced data.

CONFILL	Draws grayscale contour plots of a 2-dimensional array of regularly-spaced data.

CONREC	Draws contour plots of a 2-dimensional array of regularly-spaced data.

CURVE	Draws a curve between 2 points with specified beginning and ending slopes.

DRWCRV	Connects data points with a solid line.

DRWTRI	Draws a triangle.

DSHCRV	Connects data points with a dashed curve.

DSHLIN	Draws a dashed line between two points.

FACTOR	Enlarges or reduces a plot by a specified ratio.

FAROHED	Draws a "fancy" arrowhead.

FILLBOX	Fills a shape with a specified grayscale level.

FILRGN	Same as FILLBOX, except it restores the current gray level after filling.

FILRGNC	Fills a shape with the current gray level or color.

GREST	Calls the PostScript operator grestore.

GRKSYM	Plots a Greek symbol.

GSAV	Calls the PostScript operator gsave.

HILITEC	Prints text surrounded by a rectangular box (color).

HILITEG	Prints text surrounded by a rectangular box (grayscale).

INTEGRAL	Plots an integral with upper and lower limits.

KEKEXP	Plots a floating point number in exponential format.

KEKFLT	Plots a floating point number in floating point format.

KEKNUM	Plots a floating point number.

KEKSYM	Plots a character string.

KEKSYMC	Plots a character string, with the text string being chararacter rather than Hollerith.

KEKSYMO	Plots a character string with the characters outlined instead of filled.

LENSTR	Computes the number of characters in a character string.

NEWDEV	Specifies the name of the output PostScript file.

NUMBER	Plots a floating point number.  Similar to KEKNUM, but justification and continuation are not supported.

ONEHLF	Draws the symbol ½.

OVERBAR	Draws a character string with an overbar.

OVRSBSP	Draws an overbar over a subscripted and superscripted character string.

OVRSBSPG	Draws an overbar over a subscripted and superscripted Greek symbol.

OVERSUB	Draws an overbar over a subscripted character string.

OVERSUBG	Draws an overbar over a subscripted Greek symbol.

OVERSUP	Draws an overbar over a superscripted character string.

OVERSUPG	Draws an overbar over a superscripted Greek symbol.

OVRGRK	Draws an overbar over a Greek symbol.

PLOT	Directs "pen" movement to a specified (x,y) point with the "pen" either up or down.

PLOTND	Closes the output PostScript file and terminates the plotting session.

PLSMIN	Draws the ± character.

PRIME	Draws the  '  character.

PSINIT	Called to begin a plotting session.  It also specifies portrait or landscape mode.

RECT	Draws a rectangle.

RECTFILC	Draws a rectangle and fills it with specified red, green, and blue color levels.

RECTFILG	Draws a rectangle and fills it with specified gray level.

ROTATE	Rotates the current coordinate system by a specified angle.  Essentially, ROTATE issues the PostScript rotate command.

RRECT	Draws a rectangle with rounded corners.  It can then fill the rectangle with the current graylevel or RGB values.

SETCOLR	Sets the current color values.

SETFNT	Sets the current font.

SETGRY	Sets the current gray level value.

SETLW	Sets the current line width.

SIGMA	Draws the symbol S, with upper and lower limits.

SLDCRV	Connects data points with a solid curve.

SLDLIN	Draws a solid line between two points.

SQRSGN	Draws the radical (square root) sign.

SQUARE	Draws a square.

STROKE	Calls the PostScript operator stroke.

SUBBER	Draws a subscript.

SUBBERSP	Draws a subscript of “special” characters.

SUBSUP	Draws a subscript and superscript.

SUBSUPSP	Draws subscripts and superscripts of “special” characters.

SUPER	Draws a superscript.

SUPERSP	Draws a superscript of “special” characters.

SYMBOL	Plots a character string.  Similar to KEKSYM, but justification and continuation are not supported.

The plotting subroutines in the PSPLOT plotting library are written in Fortran and are called by user-written application programs.  In the following subroutine descriptions, integer variables are denoted by arguments beginning with the letters I-N, as per Fortran convention.  All other variables are real (4 byte) floating point Fortran and are called by user-written application programs.  All of the subroutines are written in single precision.  Hence, if your program generates data in double precision, any arguments sent to the plotting library subroutines must first be converted to single precision.

ARC

PURPOSE	ARC draws an arc having a specified radius and arc center.

SYNTAX	CALL ARC (XC, YC, RAD, ANG1, ANG2)

ARGUMENTS

XC, YC	X,Y coordinates of the center of the arc.

RAD		Radius of arc.

ANG1		Angle of arc beginning.

ANG2		Angle of arc ending.

AROHED

PURPOSE	AROHED draws an arrowhead at a specified location.

SYNTAX	CALL AROHED (XPP, YPP, DIR, AROLNP, SPRANG, LOCXY )

ARGUMENTS

XPP,YPP	X,Y coordinates of the tip of the arrowhead.

DIR		Direction of arrowhead, measured east from north.

AROLNP	Length of arrowhead sides.

SPRANG	Half the angular spread of arrowhead.

LOCXY	Location of arrowhead point

LOCXY=1	XPP,YPP at arrowhead point  (most commonly used)

LOCXY=2	XPP, YPP at center of arrowhead.

LOCXY=3	XPP,YPP at tail of arrowhead.

ARROW

PURPOSE	ARROW draws an arrow, with vector length, at a specified location.

SYNTAX	CALL ARROW (XSS, YSS, XPP, YPP, AROLNP, SPRANG, LOCXY )

ARGUMENTS

XSS,YSS	X,Y coordinates of the vector length origin.

XPP,YPP	X,Y coordinates of the tip of the arrowhead.

AROLNP	Length of arrowhead sides.

SPRANG	Half the angular spread of arrowhead.

LOCXY	Location of arrowhead point

LOCXY=1	XPP,YPP at arrowhead point  (most commonly used)

LOCXY=2	XPP, YPP at center of arrowhead.

LOCXY=3	XPP,YPP at tail of arrowhead.

AXIS

PURPOSE	AXIS draws an axis of a graph, with user-specified axis titles.  Tick marks are drawn with a spacing of one inch.

SYNTAX	CALL AXIS (XPP, YPP, IBCD, NC, AXLEN, THETA, RMIN, DEL)

ARGUMENTS

XPP,YPP		X,Y coordinates of axis origin.

IBCD			Axis title (Hollerith).

NC		Number of characters in title.  Its sign is used to specify on which side of the axis the title is to appear: positive for the counterclockwise side of the axis, negative for the clockwise side.  Positive labeling is generally used for Y axes, negative for X axes.

AXLEN	Length of axis.

THETA	Angle of axis.  Generally, 0 for X axes, 90 for Y axes.

RMIN		The starting value  and annotation  of the  first tick mark.

DEL		The data value increment between tick marks.

BLKSTP

PURPOSE	BLKSTP strips out embedded blanks in a character string.  It returns the new character string and the number of characters in the new string.

SYNTAX	CALL BLKSTP (STR1, NDIM, STR2, NC)

ARGUMENTS

STR1		Input character string.

NDIM		Maximum length of STR1 as stated in calling program.

STR2		Output character string with embedded blanks removed.

NC		Number of characters in STR2.

BORDER

PURPOSE	BORDER draws a rectangular border with tick marks.  The lower left corner of the border is located at the current plot origin.

SYNTAX	CALL BORDER(XLEN, YLEN, ITIC, IBRD, MAJX, MINX,

MAJY, MINY)

ARGUMENTS

XLEN			Length of x-direction side of border.

YLEN			Length of y-direction side of border.

ITIC		Four digit number which determines which of the border sides will contain tick marks.   Each of the digits is either 0 or 1, and if set to 1, that side will contain tick marks.  The border sides are ordered as follows:

Left-vertical     Bottom     Right-vertical     Top

For example, if ITIC = 1011, all sides except the bottom  would contain tick marks.  Additionally, if ITIC<0, the tick marks will be drawn on the outside of the border than on the inside (default).

IBRD		IBRD is similar to ITIC except that it determines which sides of the border will be drawn.  Hence, you can have tick marks with no border, and vice versa.

MAJX		Number of major divisions in the x-direction.  A longer tick mark is drawn for these divisions.

MINX		Number of minor  divisions in the x-direction, i.e. the number of divisions per major division.  A shorter tick mark is drawn for these divisions.

MAJY		Same as MAJX in y-direction.

MINY		Same as MINX in y-direction.

CHOPIT

PURPOSE	CHOPIT logically closes the current graphics page and begins a new one.

SYNTAX	CALL CHOPIT (XPP, YPP)

ARGUMENTS

XPP,YPP	X,Y coordinates of the initial plot origin of the next plot. This allows you to begin all plots at the same origin on each page.

Note: XPP and YPP are coordinates independent of the current scaling factor.  That is, XPP and YPP represent actual inches, not scaled coordinates.  The current scaling factor is reinstated after the new coordinate origin is set.

Subroutine CHOPIT should not be confused with subroutine PLOTND, which is called once at the end of each plotting session and which closes the entire output PostScript file.

CIRCLE

PURPOSE	CIRCLE draws a circle.

SYNTAX	CALL CIRCLE (XC, YC, RAD, FILL)

ARGUMENTS

XC,YC		X,Y coordinates of the center of the circle.

RAD		Radius of the circle.

FILL		Logical value.  If FILL=.TRUE., circle is filled with the current color or gray scale; otherwise it is outlined.

CLIP

PURPOSE	CLIP inserts the PostScript command clip into the output file.

SYNTAX	CALL CLIP

ARGUMENTS

NONE.

Note:  Most uses of this command are bracketed by calls to GSAV and GREST.

CLIPBOX

PURPOSE	CLIPBOX defines a clipping region.

SYNTAX	CALL CLIPBOX (XPTS, YPTS, NPTS)

ARGUMENTS

XPTS		Array containing x-coordinates of region path.

YPTS		Array containing y-coordinates of region path.

NPTS		Number of points in region path.

COLBOX

PURPOSE	COLBOX fills a region with a specified color.

SYNTAX	CALL COLBOX (XPTS, YPTS, NPTS, RED, GREEN, BLUE)

ARGUMENTS

XPTS		Array containing x-coordinates of region path.

YPTS		Array containing y-coordinates of region path.

NPTS		Number of points in region path.

RED		Red value of specified color in RGB color model.

GREEN	Green value of specified color in RGB color model.

BLUE		Blue value of specified color in RGB color model.

Arguments RED, GREEN, and BLUE must lie between 0. and 1.0, inclusive.

CONCOLR

PURPOSE	CONCOLR draws color contour plots of a two-dimensional array of regularly spaced data.

SYNTAX	CALL CONCOLR 	(ARR, IMAX, IEXT, JEXT, XLEN, YLEN, CVAL,

COLOR, NVAL, IOFFP, SPVAL)

ARGUMENTS

ARR		Two-dimensional array containing regularly spaced data to be contoured. CONCOLR assumes ARR(1,1) is located at the lower left corner of the plot.  Data points ARR(1,1)  through  ARR(IEXT,JEXT) are contoured in an area XLEN x YLEN.

IMAX		The first dimension of ARR in the calling program.

IEXT		Number of points in x-direction of ARR to be contoured.

JEXT		Number of points in y-direction of ARR to be contoured.

XLEN		X-direction length of plotting area.

YLEN		Y-direction length of plotting area.

CVAL		Array  containing the values to be used for contour intervals (1 to NVAL).  CVAL must be dimensioned with a dimension of at least 1.  Unlike in subroutine CONREC, NVAL cannot equal 0.

COLOR	Array dimensioned (3,NVAL) containing the red, green, and blue values for each of the contour levels.  The red value is stored in (1,n), the green value in (2,n) and the blue value in (3,n), where n specifies the contour index corresponding to CVAL.  Regions less than or equal to the corresponding contour value CVAL are filled with the corresponding RBG values.

NVAL		Number of contour intervals.  NVAL must be less than or equal to 100, and unlike in CONREC, NVAL cannot equal 0.

IOFFP		Flag indicating that grid boxes whose vertices have the value SPVAL are to be ignored during contouring.

SPVAL	Special value denoting which grid boxes are to be ignored during contouring.

See coding example for CONFILL.

CONFILL

PURPOSE	CONFILL draws grayscale contour plots of a two-dimensional array of regularly spaced data.

SYNTAX	CALL CONFILL(ARR, IMAX, IEXT, JEXT, XLEN, YLEN, CVAL,

GRYLEV, NVAL, IOFFP, SPVAL)

ARGUMENTS

ARR		Two-dimensional array containing regularly spaced data to be contoured. CONFILL assumes ARR(1,1) is located at the lower left corner of the plot.  Data points ARR(1,1)  through  ARR(IEXT,JEXT) are contoured in an area XLEN x YLEN.

IMAX		The first dimension of ARR in the calling program.

IEXT		Number of points in x-direction of ARR to be contoured.

JEXT		Number of points in y-direction of ARR to be contoured.

XLEN		X-direction length of plotting area.

YLEN		Y-direction length of plotting area.

CVAL		Array containing the values to be used for contour intervals (1 to NVAL).  CVAL must be dimensioned with a dimension of at least 1.  Unlike subroutine CONREC, NVAL cannot equal 0.

GRYLEV	Array dimensioned (NVAL) containing the grayscale values for each of the contour levels.  Regions less than or equal to the corresponding contour value CVAL are filled with the corresponding grayscale values.  Grayscale values range from 0. (black) to 1.0 (white).

NVAL		Number of contour intervals.  NVAL must be less than or equal to 100, and unlike in CONREC, NVAL cannot equal 0.

IOFFP		Flag indicating that grid boxes whose vertices have the value SPVAL are to be ignored during contouring.

SPVAL	Special value denoting which grid boxes are to be ignored during contouring.

CONREC

PURPOSE	CONREC draws contour plots of a two-dimensional array of regularly spaced data.

SYNTAX	CALL CONREC(ARR, IMAX, IEXT, JEXT, XLEN, YLEN, CVAL, NVAL)

ARGUMENTS

ARR		Two-dimensional array containing regularly spaced data to be contoured.  CONREC assumes ARR(1,1) is located at the  lower left corner of the plot.  Data points ARR(1,1) through ARR(IEXT,JEXT) are contoured in an area XLEN x YLEN.

IMAX		The first dimension of ARR in the calling program.

IEXT		Number of points in x-direction of ARR to be contoured.

JEXT		Number of points in y-direction of ARR to be contoured.

XLEN		X-direction length of plotting area.

YLEN		Y-direction length of plotting area.

CVAL		Array containing the values to be used for contour intervals (1 to NVAL).  CVAL must be dimensioned with a dimension of at least 1.  If NVAL = 0, the individual contour values are automatically computed.

NVAL		Number of contour intervals.  If NVAL = 0, the number of contour intervals and the contour interval values are automatically calculated. This is helpful if the range of data values is not known beforehand.  NVAL must be less than or equal to 100.

Special features of CONREC:

If NVAL < 0, only high and lows are plotted instead of contours.

Common block CONPAR is used to control various characteristics of the contour plot  from the calling program.  The variables in CONPAR are shown below, along with their default values:

COMMON/CONPAR/	ISPEC, IOFFP, SPVAL, ILEGG, ILABB, NHII,

NDECCN, NLBLL, LSCAL, LDASH, HGTLAB

ISPEC		If 0, variables in CONPAR have no effect. Default = 0.

IOFFP		If 1, you can specify regions not to contour by using SPVAL.  Grid boxes with any corner values equal to SPVAL are not contoured.  Default = 0.

SPVAL	Special value to denote areas not to contour (usually set to 999.).  Default = 0.

ILEGG		If 0, legend below contour plot showing contour values, scale factors, etc. will not be printed.  Default = 1.

ILABB		If 0, contour labels are not plotted.  Default = 1.

NHII		If 0, H and  L mark relative highs and lows, with the data value plotted underneath the symbol.

If > 0, values are plotted at each array point.

If < 0, neither of the above is done.

Default = 0.

NDECCN	Number of digits to the right of the decimal point in the contour labels.  Default = 1.

NLBLL	Number of unlabeled contours between labelled contours.  Default = 3.

LSCAL	If LSCAL = 0, the contours are scaled such that 0 < labeled values <  100.  If LSCAL = 1, the contours are not scaled, i.e. the contour values are the actual data values.  Default = 0.

LDASH	Specifies whether contours are solid (LDASH = 0) or dashed (LDASH NE 0). If LDASH is not equal to 0, then LDASH specifies the type of dash line to use to draw the contours.  The numeric value of LDASH is the number of times the dashed pattern (solid line/blank space) is repeated per inch.  Default= 0.

HGTLAB	Specifies the height of contour labels. If HGTLAB=0, the height of the contours is .11 inches.

CURVE

PURPOSE	CURVE draws a curve between two points, given the beginning and ending curve slopes.

SYNTAX	CALL CURVE (X1, Y1, X2, Y2, X3, Y3, X4, Y4, CONTIN)

ARGUMENTS

X1,Y1		X,Y coordinates of the beginning of the curve.

X2,Y2		X, Y coordinates specifying the beginning curve slope.  The initial slope of the curve is given by (Y2-Y1)/(X2-X1).

X3,Y3		X,Y coordinates specifying the ending curve slope.  The end slope of the curve is given by (Y4-Y3)/(X4-X3).

X4,Y4		X,Y coordinates of the endpoint of the curve.

CONTIN	Logical value. If CONTIN=.false. then the curve drawn starts a newpath; otherwise, the curve is appended to the current path.

Note:  The curve produced by CURVE is not automatically stroked.

DRWCRV

PURPOSE	DRWCRV draws a solid curve.

SYNTAX	CALL DRWCRV (XARR, YARR, NPTS, THK, CLOSER)

ARGUMENTS

XARR		Array containing the x-coordinates of the points to be connected.

YARR		Array containing the y-coordinates of the points to be connected.

NPTS		Number of points in the curve.

THK		Thickness of the curve.  If THK = 0., the current linewidth is used.

CLOSER	Logical variable. If CLOSER = .TRUE., the last point in the data arrays is connected to the first point, i.e. the curve is closed; otherwise, the curve is open.

DRWTRI

PURPOSE	DRWTRI draws an equilateral triangle.

SYNTAX	CALL DRWTRI (XC, YC, SIDE, THK)

ARGUMENTS

XC, YC	X, Y coordinates of triangle center.

SIDE		Length of triangle side.

THK		Thickness of line used to draw the triangle.  If THK = 0., the current line width is used.

DSHCRV

PURPOSE	DSHCRV connects points in two data arrays using a line with a specified dash pattern.

SYNTAX	CALL DSHCRV (XARR, YARR, NPTS, IDSHPN, THK)

ARGUMENTS

XARR		Array containing the x-coordinates of the points to be connected.

YARR		Array containing the y-coordinates of the points to be connected.

NPTS		Number of points in the curve.

IDSHPN	Dashed pattern of the line used to connect the data points.  IDSHPN is specified as the number of times the dashed pattern (solid line/blank space) is repeated per inch.

THK		Thickness of the dashes in the curve.  If THK = 0., the current line width is used.

DSHLIN

PURPOSE	DSHLIN connects two points using a line with a specified dash pattern.

SYNTAX	CALL DSHLIN (X1, Y1, X2, Y2, IDSHPN, THK)

ARGUMENTS

X1,Y1		X,Y coordinates of first data point.

X2,Y2		X,Y coordinates of second data point.

IDSHPN	Dashed pattern of the line used to connect the two data points.  IDSHPN is specified as the number of times the dashed pattern (solid line/blank space) is repeated per inch.

THK		Thickness of the dashes in the connecting line.  If THK =  0., the current line width is used.

FACTOR

PURPOSE	FACTOR enlarges or reduces the size of a plot by a specified ratio.

SYNTAX	CALL FACTOR (FACT)

ARGUMENTS

FACT		The ratio of the desired plot size to the actual size.  FACT must be a positive floating point number.

For  example, to  enlarge a plot to twice its normal size:

CALL FACTOR (2.0)

FAROHED

PURPOSE	FAROHED draws a "fancy" arrowhead at a specified location.

SYNTAX	CALL FAROHED (XPP, YPP, DIR, AROLNP, SPRANG, LOCXY, FILL)

ARGUMENTS

XPP,YPP	X,Y coordinates of the tip of the arrowhead.

DIR		Direction of arrowhead, measured east from north.

AROLNP	Length of arrowhead sides.

SPRANG	Half the angular spread of arrowhead.

LOCXY	Location of arrowhead point

LOCXY=1	XPP,YPP at arrowhead point  (most commonly used)

LOCXY=2	XPP, YPP at center of arrowhead.

LOCXY=3	XPP,YPP at tail of arrowhead.

FILL		Logical variable.  If FILL= .TRUE., the arrowhead is filled; otherwise it is outlined.

FILLBOX

PURPOSE	FILLBOX fills a shape with a specified gray level.

SYNTAX	CALL FILLBOX (XARR, YARR, NPTS, GRY)

ARGUMENTS

XARR		Array containing the x-coordinates of the points forming the perimeter of the shape to fill.

YARR		Array containing the y-coordinates of the points forming the perimeter of the shape to fill.

NPTS		Number of points comprising the shape perimeter.

GRY		Gray level of filled shape.  GRY must lie between 0. (black) and 1. (white), inclusive.

Note:  The current gray level remains at value GRY after exiting FILLBOX.

FILRGN

PURPOSE	FILRGN is the same as FILLBOX, except it restores the current gray level after filling.

SYNTAX	CALL FILRGN (XARR, YARR, NPTS, GRY)

ARGUMENTS

XARR		Array containing the X coordinates of the points forming the perimeter of the shape to fill.

YARR		Array containing the Y coordinates of the points forming the perimeter of the shape to fill.

NPTS		Number of points comprising the shape perimeter.

GRY		Gray level of filled shape.  GRY must lie between 0.(black) and 1. (white), inclusive.

FILRGNC

PURPOSE	FILRGNC fills a region with the current gray level or color.

SYNTAX	CALL FILRGNC (XARR, YARR, NPTS)

ARGUMENTS

XARR		Array containing the X coordinates of the points forming the perimeter of the shape to fill.

YARR		Array containing the Y coordinates of the points forming the perimeter of the shape to fill.

NPTS		Number of points comprising the shape perimeter.

GREST

PURPOSE	GREST  calls the PostScript operator grestore. It is useful after establishing and using a clipping region when it is no longer needed.

SYNTAX	CALL GREST

ARGUMENTS

NONE

GREST is used to restore the graphics state of a PostScript page.  It is called after a clipping region has been established and is no longer needed.  GREST is used in conjunction with GSAV.

GRKSYM

PURPOSE	GRKSYM plots a Greek symbol.

SYNTAX	CALL GRKSYM (XPP, YPP, HEIGHT, ICH, ANG, NCHAR, MJUS)

ARGUMENTS

XPP,YPP	X,Y coordinates of the Greek symbol to be plotted.  Plotting may be continued from the end of a previously plotted character when used  in conjunction with any of the subroutines which support continuation.

HEIGHT	Height of Greek symbol to be plotted.

ICH		Integer value between 1 and 50 corresponding to the desired Greek symbol (see table below).

ANG		Angle, measured counterclockwise from the X-axis, at which the character is to be plotted.

NCHAR	Number of characters to be plotted (should  be set to 1).

MJUS		Controls the  justification of the character to be plotted.

If MJUS = 0,  (XPP,YPP) denotes the position of the lower left corner of the plotted character.

If MJUS =  1, (XPP,YPP) denotes the position of the center of the plotted character.

If  MJUS = 2,  (XPP,YPP) denotes the position of the lower right corner of the plotted character.

GSAV

PURPOSE	GSAV  calls the PostScript operator gsave. It is useful before establishing and using a clipping region.

SYNTAX	CALL GSAV

ARGUMENTS

NONE

GSAV is used to save the graphics state of the PostScript page prior to defining a clipping region.  After the clipping region is used, GREST is called to restore the original graphics state.

HILITEC

PURPOSE	HILITEC prints text surrounded by a rectangular box (color).

SYNTAX	CALL HILITEC (XPP, YPP, HEIGHT, CSTR, ANG, EDG, JUSX, JUSY,							FRED, FGREEN, FBLUE, BRED, BGREEN, BBLUE)

ARGUMENTS

XPP,YPP	X,Y coordinates of the box surrounding the text.

HEIGHT	Height of text to be plotted.

CSTR		Character string text to be plotted.

ANG		Angle, measured counterclockwise from the  X-axis, at which the text is to be plotted.

EDG		Fraction of the text height to use as an edge border.

JUSX		Justification in the x-direction.

JUSX=0  is left justification.

JUSX=1 is centered.

JUSX=2 is right justification.

JUSY		Similar to JUSX, but in the y-direction.

JUSY=0 is bottom justification.

JUSY=1 is centered.

JUSY=2 is top justification.

FRED, FGREEN, FBLUE	Foreground text RGB values.  Must lie between 0. and 1.0, inclusive.

BRED, BGREEN, BBLUE	Background box RGB values.  Must lie between 0. and 1.0, inclusive.

See code example for HILITEG.

HILITEG

PURPOSE	HILITEG prints text surrounded by a rectangular box (grayscale).

SYNTAX	CALL HILITEG (XPP, YPP, HEIGHT, CSTR, ANG, EDG, JUSX, JUSY,							FGRY, BGRY)

ARGUMENTS

XPP,YPP	X,Y coordinates of the box surrounding the text.

HEIGHT	Height of text to be plotted.

CSTR		Character string text to be plotted.

ANG		Angle, measured counterclockwise from the  X-axis, at which the text is to be plotted.

EDG		Fraction of the text height to use as an edge border.

JUSX		Justification in the x-direction.

JUSX=0  is left justification.

JUSX=1 is centered.

JUSX=2 is right justification.

JUSY		Similar to JUSX, but in the y-direction.

JUSY=0 is bottom justification.

JUSY=1 is centered.

JUSY=2 is top justification.

FGRY		Graylevel of the (foreground) text. FGRY must lie between 0. and 1.0, inclusive.

BGRY		Graylevel of the (background) surrounding box.  BGRY must lie between 0. and 1.0, inclusive.

INTEGRAL

PURPOSE	INTEGRAL plots an integral with upper and lower limits.

SYNTAX	CALL INTEGRAL (XPP, YPP, HEIGHT, ANG, LOWER, NL, LUPPER,

NU)

ARGUMENTS

XPP,YPP	X,Y coordinates of the lower left edge of the integral.

HEIGHT	Height of the integral.

ANG		Angle, measured counterclockwise from the  X-axis, at which the integral is plotted.

LOWER	Character string of the lower limit (Hollerith).

NL		Number of characters in the lower limit text.

LUPPER	Character string of the upper limit (Hollerith).

NU		Number of characters in the upper limit text.

KEKEXP

PURPOSE	KEKEXP plots a floating  point  number in exponential format.

SYNTAX	CALL KEKEXP (XPP, YPP, HEIGHT, FNUM, ANG, NDEC, MJUS)

ARGUMENTS

XPP,YPP	X,Y coordinates of the  number to be plotted.  Plotting may be continued from the end of a previously plotted character when used  in conjunction with any of the subroutines which support continuation

HEIGHT	Height of number to be plotted.

FNUM		Floating point number to be plotted.

ANG		Angle, measured counterclockwise from the X-axis, at which the number is to be plotted.

NDEC		Controls the plotted precision of FNUM.

If NDEC > 0, it specifies the number of digits to the right of the decimal point, after rounding.

If  NDEC = 0, only the number's integer portion and a decimal point are plotted, after rounding.

If  NDEC =  -1, only the number's integer portion is plotted, after rounding.

MJUS		Controls the justification of the number to be plotted.

If MJUS = 0, (XPP,YPP) denotes the position of the lower left corner of the plotted number.

If MJUS = 1, (XPP,YPP) denotes the position of the center of the plotted number.

If MJUS  = 2, (XPP,YPP) denotes the position of the lower right corner of the plotted number.

KEKFLT

PURPOSE	KEKFLT is the same as KEKNUM.

SYNTAX	CALL KEKFLT (XPP, YPP, HEIGHT, FNUM, ANG, NDEC, MJUS)

ARGUMENTS

Same as KEKNUM.

KEKNUM

PURPOSE	KEKNUM plots a floating point number in floating point format.

SYNTAX	CALL KEKNUM (XPP, YPP, HEIGHT, FNUM, ANG, NDEC, MJUS)

ARGUMENTS

XPP,YPP	X,Y  coordinates of the  number to be plotted.  Plotting may be continued  from the end of a previously plotted character when  used  in  conjunction  with any of the subroutines which support continuation.

HEIGHT	Height of number to be plotted.

FNUM		Floating point number to be plotted.

ANG		Angle, measured counterclockwise from the X-axis, at which the number is to be plotted.

NDEC		Controls the plotted precision of FNUM.

If NDEC > 0, it specifies the number of digits to the right of the decimal point, after rounding.

If NDEC = 0, only the number's integer portion and a decimal point are plotted, after rounding.

If NDEC =  -1, only the number's integer portion is plotted, after rounding.

MJUS		Controls the justification of the number to be plotted.

If MJUS = 0, (XPP,YPP) denotes the position of the lower left corner of the plotted number.

If MJUS = 1, (XPP,YPP) denotes the position of the center of the plotted number.

If MJUS  = 2, (XPP,YPP) denotes the position of the lower right corner of the plotted number.

KEKSYM

PURPOSE	KEKSYM plots a character string.

SYNTAX	CALL KEKSYM (XPP, YPP, HEIGHT, IBCD, ANG, NCHAR, MJUS)

ARGUMENTS

XPP,YPP	X,Y coordinates of the character string to be plotted.  Plotting may be continued from the end of a previously plotted character when used in conjunction  with any of the subroutines which support continuation.

HEIGHT	Height of character string to be plotted.

IBCD		Character string to be plotted (Hollerith).

ANG		Angle, measured counterclockwise from the X-axis, at which the character string is to be plotted.

NCHAR	Number of characters in the string to plot.

MJUS		Controls  the  justification  of  the  character string to  be plotted.

If MJUS = 0, (XPP,YPP) denotes the position of the lower left corner of the character string.

If MJUS = 1, (XPP,YPP) denotes the  position of the center of the character string.

If  MJUS = 2, (XPP,YPP) denotes the position of the lower right corner of the character string.

Special feature of KEKSYM:

You can plot special characters, for example the characters in fonts Symbol and Zapf Dingbats, by setting NCHAR=-999 and IBCD  equal to the octal code of the character you want to plot.

KEKSYMC

PURPOSE	KEKSYMC plots a character string. Same as KEKSYM, but text is input as a character string rather than Hollerith.

SYNTAX	CALL KEKSYMC (XPP, YPP, HEIGHT, CSTR, ANG, NCHAR, MJUS)

ARGUMENTS

XPP,YPP	X,Y coordinates of the character string to be plotted.  Plotting may be continued from the end of a previously plotted character when used in conjunction  with any of the subroutines which support continuation.

HEIGHT	Height of character string to be plotted.

CSTR		Character string to be plotted (Hollerith).

ANG		Angle, measured counterclockwise from the X-axis, at which the character string is to be plotted.

NCHAR	Number of characters in the string to plot.

MJUS		Controls  the  justification  of  the  character string to  be plotted.

If MJUS = 0, (XPP,YPP) denotes the position of the lower left corner of the character string.

If MJUS = 1, (XPP,YPP) denotes the  position of the center of the character string.

If  MJUS = 2, (XPP,YPP) denotes the position of the lower right corner of the character string.

Special feature of KEKSYMC:

You can plot special characters, for example the characters in fonts Symbol and Zapf Dingbats, by setting NCHAR=-999 and CSTR  equal to the octal code of the character you want to plot. CSTR must be in single quotes.

KEKSYMO

PURPOSE	KEKSYMO plots a character string; however, the characters are outlined rather than filled (solid).

SYNTAX	CALL KEKSYMO (XPP, YPP, HEIGHT, IBCD, ANG, NCHAR, MJUS)

ARGUMENTS

XPP,YPP	X,Y coordinates of the character string to be plotted.  Plotting may be continued from the end of a previously plotted character when used in conjunction with any of the subroutines which support continuation.

HEIGHT	Height of character string to be plotted.

IBCD		Character string to be plotted (Hollerith).

ANG		Angle, measured counterclockwise  from the X-axis, at which the character string is to be plotted.

NCHAR	Number of characters in the string to plot.

MJUS		Controls the justification of the character string to be plotted.

If MJUS = 0, (XPP,YPP) denotes the position of the lower left corner of the character string.

If MJUS = 1, (XPP,YPP) denotes the position of the center of the character string.

If MJUS = 2, (XPP,YPP) denotes  the position of the lower right corner of the character string.

Special feature of KEKSYMO:

You can plot special characters, for example the characters in fonts Symbol and Zapf Dingbats, by setting NCHAR=-999 and IBCD  equal to the octal code of the character you want to plot.

LENSTR

PURPOSE	LENSTR is a function whose returned value is the number of characters in the character string argument after trailing blanks have been removed.

SYNTAX	LS = LENSTR(STRING, NDIM)

ARGUMENTS

STRING		Character string variable.

NDIM			Maximum length of STRING as stated in calling program.

LS		Returned length of the character string STRING after trailing blanks have been removed.

NEWDEV

PURPOSE	NEWDEV specifies a non-default filename which will contain the application program's output PostScript plot file.

SYNTAX	CALL NEWDEV (FLNAME, NCHAR)

ARGUMENTS

FLNAME		Character  string or  FORTRAN character  variable which contains  the name of the output Postscript file.

NCHAR		Number of characters in the filename.

For example,

CALL NEWDEV ('MYPLOT.DAT', 10)

NEWDEV is called only once per plotting session and must be called prior to PSINIT.  If NEWDEV is not called, the name of the output PostScript file is psplot.ps.

NUMBER

PURPOSE	NUMBER is the same as KEKNUM with left justification.

SYNTAX	CALL NUMBER (XPP, YPP, HEIGHT, FNUM, ANG, NDEC)

ARGUMENTS

Same as KEKNUM.

ONEHLF

PURPOSE	ONEHLF plots the symbol ½ .

SYNTAX	CALL ONEHLF (XPP, YPP, HEIGHT, ANG, MJUS)

ARGUMENTS

XPP,YPP	X,Y coordinates of the  symbol to be plotted.

HEIGHT	Height of symbol to be plotted.

ANG		Angle, measured counterclockwise from the X-axis, at which the symbol is to be plotted.

MJUS		Controls the justification of the character string to be plotted.

If MJUS = 0, (XPP,YPP) denotes the position of the lower left corner of the character string.

If MJUS = 1, (XPP,YPP) denotes the position of the center of the character string.

If MJUS = 2, (XPP,YPP) denotes the position of the lower right corner of the character string.

OVERBAR

PURPOSE	OVERBAR plots a character string with an overbar.

SYNTAX	CALL OVERBAR (XPP, YPP, HEIGHT, IBCD, ANG, NCHAR, MJUS)

ARGUMENTS

XPP,YPP	X,Y coordinates of the  character string to be plotted.  Plotting may be continued from the end of a previously plotted character when used  in conjunction with any of the subroutines which support continuation.

HEIGHT	Height of number to be plotted.

IBCD		Character string to be plotted (Hollerith).

ANG		Angle, measured counterclockwise from the X-axis, at which the character string is to be plotted.

NCHAR	Number of characters in the string to plot.

MJUS		Controls the justification of the character string to be plotted.

If MJUS = 0,  (XPP,YPP) denotes the position of the lower left corner of the character string.

If MJUS = 1, (XPP,YPP) denotes the position of the center of the character string.

If MJUS = 2, (XPP,YPP) denotes the position of the lower right corner of the character string.

OVERSBSP

PURPOSE	OVERSBSP plots a subscripted and superscripted character string with an overbar. The overbar extends to cover both the subscripts and superscripts.

SYNTAX	CALL OVERSBSP (XPP, YPP, HEIGHT, IBCD, ANG, NCHAR,

MJUS,ISUB,NSUB, ISUP, NSUP)

ARGUMENTS

XPP,YPP	X,Y coordinates of the character string to be plotted.  Plotting may be continued from the end of a previously plotted character when used in conjunction with any of the subroutines which support continuation.

HEIGHT	Height of character string to be plotted.

IBCD		Character string to be plotted (Hollerith).

ANG		Angle, measured counterclockwise from the X-axis, at which the character string is to be plotted.

NCHAR	Number of characters in the string to plot.

MJUS		Controls  the  justification  of  the  character string to  be plotted.

If MJUS = 0, (XPP,YPP) denotes the position  of the lower left corner of the character string.

If MJUS = 1, (XPP,YPP) denotes the position of the center of the character string.

If MJUS  = 2, (XPP,YPP) denotes the position of the lower right corner of the character string.

ISUB		Hollerith character(s) comprising the subscript.

NSUB		Number of characters in the subscript string.

ISUP		Hollerith character(s) comprising the superscript.

NSUP		Number of characters in the superscript string.

OVERSBSPG

PURPOSE	OVERSBSPG plots a subscripted and superscripted Greek symbol with an overbar. The overbar extends to cover both the subscripts and superscripts.

SYNTAX	CALL OVERSBSPG	(XPP, YPP, HEIGHT, ICH, ANG, CHAR, MJUS,

ISUB, NSUB, ISUP, NSUP)

ARGUMENTS

XPP,YPP	X,Y coordinates of the  character string to be plotted.  Plotting may be continued  from the end of a previously plotted character when  used in conjunction with any of the subroutines which support continuation.

HEIGHT	Height of symbol to be plotted.

ICH		Integer value between 1 and 50 corresponding to the desired Greek symbol (see table below Subroutine GRKSYM).

ANG		Angle, measured counterclockwise from the X-axis, at which the character string is to be plotted.

NCHAR	Number of characters in the string to plot (should be set to 1).

MJUS		Controls the justification of  the  character string to be plotted.

If MJUS = 0, (XPP,YPP) denotes the position of the lower left corner of the character string.

If MJUS = 1, (XPP,YPP) denotes the position of the center of the character string.

If MJUS = 2, (XPP,YPP) denotes the position of the lower right corner of the character string.

ISUB		Hollerith character(s) comprising the subscript.

NSUB		Number of characters in the subscript string.

ISUP		Hollerith character(s) comprising the superscript.

NSUP		Number of characters in the superscript string.

OVERSUB

PURPOSE	OVERSUB plots a subscripted character string with an overbar. The overbar extends to cover the subscripts.

SYNTAX	CALL OVERSUB (XPP, YPP, HEIGHT, IBCD, ANG, NCHAR, MJUS,

ISUB, NSUB)

ARGUMENTS

XPP,YPP	X,Y coordinates of the character string to be plotted.  Plotting may be continued from the end of a previously plotted character when used  in conjunction with any of the subroutines which support continuation.

HEIGHT	Height of character string to be plotted.

IBCD		Character string to be plotted (Hollerith).

ANG		Angle, measured counterclockwise from the X-axis, at which the character string is to be plotted.

NCHAR	Number of characters in the string to plot.

MJUS		Controls the justification of the character string to be plotted.

If MJUS = 0, (XPP,YPP) denotes the position of the lower left corner of the character string.

If MJUS = 1, (XPP,YPP) denotes the position of the center of the character string.

If MJUS = 2, (XPP,YPP) denotes the position of the lower right corner of the character string.

ISUB		Hollerith character(s) comprising the subscript.

NSUB		Number of characters in the subscript string.

OVERSUBG

PURPOSE	OVERSUBG plots a subscripted Greek symbol with an overbar. The overbar extends to cover the subscript.

SYNTAX	CALL OVERSUBG(XPP, YPP, HEIGHT, ICH, ANG, NCHAR,

MJUS,ISUB,NSUB)

ARGUMENTS

XPP,YPP	X,Y coordinates of the  character string to be plotted.  Plotting may be continued from the end of a previously plotted character when  used in conjunction with any of the subroutines which support continuation.

HEIGHT	Height of the Greek symbol to be plotted.

ICH		Integer  value between 1 and 50 corresponding  to the desired Greek symbol (see table below Subroutine GRKSYM).

ANG		Angle, measured counterclockwise from the X-axis, at which the symbol is to be plotted.

NCHAR	Number of characters in the string to plot (should be set to 1).

MJUS		Controls the justification of the character string to be plotted.

If MJUS = 0, (XPP,YPP) denotes the position of the lower left corner of the symbol.

If MJUS = 1, (XPP,YPP) denotes the position of the center of the symbol.

If MJUS = 2, (XPP,YPP) denotes the position of the lower right corner of the symbol.

ISUB		Hollerith character(s) comprising the subscript.

NSUB		Number of characters in the subscript string.

OVERSUP

PURPOSE	OVERSUP plots a superscripted character string with an overbar. The overbar extends to cover the subscript.

SYNTAX	CALL OVERSUP	(XPP, YPP, HEIGHT, ICH, ANG, NCHAR,

MJUS,ISUP, NSUP)

ARGUMENTS

XPP,YPP	X,Y coordinates of the character string to be plotted.  Plotting may be continued from the end of a previously plotted character when used  in conjunction with any of the subroutines which support continuation.

HEIGHT	Height of character string to be plotted.

IBCD		Character string to be plotted (Hollerith).

ANG		Angle, measured counterclockwise from the X-axis, at which the character string is to be plotted.

NCHAR	Number of characters in the string to plot.

MJUS		Controls the justification of the character string to be plotted.

If MJUS = 0, (XPP,YPP) denotes the position of the lower left corner of the character string.

If MJUS = 1, (XPP,YPP) denotes the position of the center of the character string.

If MJUS = 2,  (XPP,YPP) denotes the position of the lower right corner of the character string.

ISUP		Hollerith character(s) comprising the superscript.

NSUP		Number of characters in the superscript string.

OVERSUPG

PURPOSE	OVERSUPG plots a subscripted Greek character with an overbar. The overbar extends to cover the superscript.

SYNTAX	CALL OVERSUPG(XPP, YPP, HEIGHT, ICH, ANG, NCHAR, MJUS,

ISUP, NSUP)

ARGUMENTS

XPP,YPP	X,Y coordinates of the Greek symbol to be plotted.  Plotting may be continued from the end of a previously plotted character when used in conjunction with any of the subroutines which support continuation.

HEIGHT	Height of the Greek symbol to be plotted.

ICH		Integer  value between 1 and 50 corresponding to the desired Greek symbol (see table below Subroutine GRKSYM).

ANG		Angle, measured counterclockwise from the X-axis, at which the symbol is to be plotted.

NCHAR	Number of characters in the string to plot (should be set to 1).

MJUS		Controls the justification of the character string to be plotted.

If MJUS = 0, (XPP,YPP) denotes the position of the lower left corner of the symbol.

If  MJUS = 1, (XPP,YPP) denotes the position of the center of the symbol.

If MJUS = 2, (XPP,YPP) denotes the position of the lower right corner of the symbol.

ISUP		Hollerith character(s) comprising the superscript.

NSUP		Number of characters in the superscript string.

OVRGRK

PURPOSE	OVRGRK plots a Greek symbol with an overbar.

SYNTAX	CALL OVRGRK (XPP, YPP, HEIGHT, ICH, ANG, NCHAR, MJUS)

ARGUMENTS

XPP,YPP	X,Y coordinates of the Greek symbol to be plotted.  Plotting may be continued from the end of a previously plotted character when  used  in conjunction with any of the subroutines which support continuation.

HEIGHT	Height of symbol to be plotted.

ICH		Integer value between 1 and 50 corresponding to the desired Greek symbol (see table below Subroutine GRKSYM).

ANG		Angle, measured counterclockwise from the X-axis, at which the symbol is to be plotted.

NCHAR	Number of characters in the string to plot (should be set to 1).

MJUS		Controls the justification of the symbol to be plotted.

If MJUS = 0, (XPP,YPP) denotes the position  of the lower left corner of the symbol.

If MJUS = 1, (XPP,YPP) denotes the position of the center of the symbol.

If MJUS = 2, (XPP,YPP) denotes the position of the lower right corner of the symbol.

PLOT

PURPOSE	PLOT is the most fundamental user-level plotting command.  It gives you direct control of pen movement (to any X,Y coordinate position) and pen status (up or down).  Additionally, it allows you to re-define the current plotting origin.

SYNTAX	CALL PLOT (XPP, YPP, IPEN)

ARGUMENTS

XPP,YPP	X,Y coordinates of the position to which the pen is to be moved.  An origin may be established anywhere on or off the plotting surface, as  explained below for negative IPEN values.

IPEN		A signed integer which controls pen status (up/down) and origin definition.

If IPEN = 2, the pen is down during movement, thus drawing a visible line.

If IPEN = 3, the pen is up during movement, thus changing the pen's current position only.

If IPEN = -2  or -3, a new origin is defined at the position (XPP,YPP) after the movement is completed as if IPEN were positive.  The logical X,Y coordinates of the new pen position are set to (0,0), so that all subsequent pen movements use this position as a reference point.

If IPEN = 999, the call to PLOT closes the output file.  Thus, a call to PLOT with IPEN = 999 may be used only once in a given plotting session, and if used, must be the last plotting command in the plotting  session.  Calling PLOT with IPEN  = 999 is identical to calling PLOTND (which see).

PLOTND

PURPOSE	PLOTND closes the output PostScript plot file to terminate the current plot defined by the user application program.

SYNTAX	CALL PLOTND

ARGUMENTS

None

You must call PLOTND (or  PLOT ( 0., 0., 999) ) as the last plotting call in the application program.

PLSMIN

PURPOSE	PLSMIN plots the symbol ±.

SYNTAX	CALL PLSMIN (XPP, YPP, HEIGHT, ANG, MJUS)

ARGUMENTS

XPP,YPP	X,Y coordinates of the symbol to be plotted.  Plotting may be continued from the end of a previously plotted character when  used  in conjunction with any of the subroutines which support continuation.

HEIGHT	Height of symbol to be plotted.

ANG		Angle, measured counterclockwise from the X-axis, at which the symbol is to be plotted.

MJUS		Controls the justification of the character string to be plotted.

If MJUS = 0, (XPP,YPP) denotes the position of the lower left corner of the character.

If MJUS = 1, (XPP,YPP) denotes the position of the center of the character.

If MJUS = 2, (XPP,YPP) denotes the position of the lower right corner of the character.

PRIME

PURPOSE	PRIME plots the symbol  ‘  .

SYNTAX	CALL PRIME (XPP, YPP, HEIGHT, ANG, MJUS)

ARGUMENTS

XPP,YPP	X,Y coordinates of the symbol to be plotted.  Plotting may be continued from the end of a previously plotted character when used in conjunction with any of the subroutines which support continuation.

HEIGHT	Height of symbol to be plotted.

ANG		Angle, measured counterclockwise from the X-axis, at which the symbol is to be plotted.

MJUS		Controls the justification of the symbol to be plotted.

If MJUS = 0, (XPP,YPP) denotes the position of the lower left corner of the character.

If MJUS = 1, (XPP,YPP) denotes the position of the center of the character.

If MJUS  = 2, (XPP,YPP) denotes the position of the lower right corner of the character.

PSINIT

PURPOSE	PSINIT is called to begin a plotting session.  It must be called before any other plotting command, with the exception of NEWDEV.

SYNTAX	CALL PSINIT (PRTRT)

ARGUMENTS

PRTRT		Logical variable indicating the paper orientation.  If PRTRT=.TRUE., the paper is oriented in portrait mode, i.e. long side of page is vertical.  If PRTRT=.false., the paper is oriented in landscape mode, i.e. long side of page is horizontal.

RECT

PURPOSE	RECT draws a rectangle (or square).

SYNTAX	CALL RECT (XX1, YY1, XX2, YY2, HEIGHT)

ARGUMENTS

XX1,YY1	X,Y coordinates of lower left corner of rectangle.

XX2,YY2	X,Y coordinates of lower right corner of rectangle.

HEIGHT	Height of rectangle.

RECTFILC

PURPOSE	RECTFILC draws a rectangle (or square) and fills it with a specified red, green, and blue color levels.

SYNTAX	CALL RECTFILC( XX1, YY1, XX2, YY2, HEIGHT, RED, GREEN BLUE)

ARGUMENTS

XX1,YY1	X,Y coordinates of lower left corner of rectangle.

XX2,YY2	X,Y coordinates of lower right corner of rectangle.

HEIGHT	Height of rectangle.

RED, GREEN, BLUE	RGB values of the fill color. Must lie between 0. and 1., inclusively.

RECTFILG

PURPOSE	RECTFILG draws a rectangle (or square) and fills it with a specified graylevel.

SYNTAX	CALL RECTFILG (XX1, YY1, XX2, YY2, HEIGHT, GRYLEV)

ARGUMENTS

XX1,YY1	X,Y coordinates of lower left corner of rectangle.

XX2,YY2	X,Y coordinates of lower right corner of rectangle.

HEIGHT	Height of rectangle.

GRYLEV	Graylevel to use for filling rectangle.  GRYLEV must lie beween 0. (black) and 1. (white), inclusive.

ROTATE

PURPOSE	ROTATE rotates the current coordinate system by a specified angle.  Essentially, ROTATE issues the PostScript rotate command.

SYNTAX	CALL ROTATE ( ANG )

ARGUMENTS

ANGLE	Angle, measured counterclockwise, to rotate the current coordinate system.

RRECT

PURPOSE	RRECT draws a rectangle (or square).with rounded corners.  It can then fill the rectangle with the current graylevel or RGB values.

SYNTAX	CALL RECT (XX1, YY1, WIDTH, HEIGHT, RAD, ANG, FILL)

ARGUMENTS

XX1,YY1	X,Y coordinates of lower left corner of rectangle.

WIDTH	Width of rectangle.

HEIGHT	Height of rectangle.

RAD		Radius of rectangle corner curves.

ANG		Angle of rectangle rotation.

FILL		Logical value.  If FILL=.true., then the rectangle is filled with the current graylevel or RGB values; otherwise, the rectangle is outlined.

SETCOLR

PURPOSE	SETCOLR sets the current red, green, and blue color levels.

SYNTAX	CALL SETCOLR (RED, GREEN, BLUE)

ARGUMENTS

RED		Red color saturation level, between 0. and 1.0.

GREEN	Green color saturation level, between 0. and 1.0.

BLUE		Blue color saturation level, between 0. and 1.0.

SETFNT

PURPOSE	SETFNT sets the current font.

SYNTAX	CALL SETFNT (NFONT)

ARGUMENTS

NFONT		Number of desired font between 1 and 35 (see table below).

The fonts sets 29 (Symbol) and 35 (ZapfDingbats) require the octal code of the character to be plotted.  See the tables on the following pages for the character sets and corresponding octal code values for these two fonts.

SETGRY

PURPOSE	SETGRY sets the current gray level.

SYNTAX	CALL SETGRY (GRYLVL)

ARGUMENTS

GRYLVL	Gray level between 0. (black) and 1. (white).

SETLW

PURPOSE	SETLW sets the current linewidth.

SYNTAX	CALL SETLW (RLW)

ARGUMENTS

RLW		Line width.

After calling SETLW, RLW remains the current linewidth until SETLW is called again.

SIGMA

PURPOSE	SIGMA draws the symbol S, with upper and lower limits.

SYNTAX	CALL SIGMA ( XPP, YPP, HEIGHT, ANG, LOWER, NL, LUPPER, NU)

ARGUMENTS

XPP, YPP	X, Y coordinates of the lower left edge of the symbol.

HEIGHT	Height of the symbol.

ANG		Angle, measured counterclockwise from the X-axis, at which the symbol is to be plotted.

LOWER	Character string specifying the lower limit (Hollerith).

NL		Number of characters in the lower limit character string.

LUPPER	Character string specifying the upper limit (Hollerith).

NU		Number of characters in the upper limit character string.

If NL (NU) is set to -999, then LOWER (LUPPER) is the octal code of the character in the font SYMBOL.

SLDCRV

PURPOSE	SLDCRV connects points in two data arrays using a line with a specified thickness.

SYNTAX	CALL SLDCRV (XARR, YARR, NPTS, THK)

ARGUMENTS

XARR		Array containing the X coordinates of the data points to be plotted.

YARR		Array containing the Y coordinates of the data points to be plotted.

NPTS		Number of points in the curve.

THK		Specifies the thickness of the curve.  If THK = 0., the current linewidth is used.

SLDLIN

PURPOSE	SLDLIN connects two points using a line with a specified thickness.

SYNTAX	CALL SLDLIN (X1, Y1, X2, Y2, THK)

ARGUMENTS

X1,Y1		X,Y coordinates of first data point.

X2,Y2		X,Y coordinates of second data point.

THK		Specifies the thickness of the connecting line.  If THK=0., the current linewidth is used.

SQRSGN

PURPOSE	SQRSGN draws a radical (square root) sign.

SYNTAX	CALL SQRSGN (XPP, YPP, HEIGHT, RLEN)

ARGUMENTS

XPP,YPP	X,Y coordinates of lower left corner of the radical.

HEIGHT	Height of the radical.

RLEN		Length of the top of the radical.

SQUARE

PURPOSE	SQUARE draws a centered square.

SYNTAX	CALL SQUARE (XC, YC, SIDE)

ARGUMENTS

XC,YC		X,Y coordinates of the center of the square.

SIDE		Length of the sides of the square.

STROKE

PURPOSE	STROKE calls the PostScript operator stroke (which paints or draws the current path).

SYNTAX	CALL STROKE

ARGUMENTS

None

SUBBER

PURPOSE	SUBBER draws a subscript.

SYNTAX	CALL SUBBER (ISUB, NSUB, SIZE, ANG)

ARGUMENTS

ISUB		Hollerith variable specifying the subscript character string.

NSUB		Number of characters in the subscript.

SIZE		Height of the variable to be subscripted.  It is not the height of the subscript characters themselves.

ANG		Angle of the subscripted variable.

Note:		SUBBER must be called immediately after the call to create the subscripted variable.

SUBBERSP

PURPOSE	SUBBERSP draws a subscript of “special” characters.

SYNTAX	CALL SUBBERSP (NSET, NFNT, ITITLE, NCHR, HEIGHT, ANG

ARGUMENTS

NSET		Number of different font sets needed.

NFNT		Array holding the font numbers 1 to NSET.

ITITLE		ITITLE holds the octal codes for characters of fonts 29 or 35; otherwise, it hold the text characters themselves.

NCHR		Number of characters of a given font. Usually one, but can be greater than one for fonts other than 29 and 35.

HEIGHT	Height of the variable to be subscripted.  It is not the height of the subscript characters themselves.

ANG		Angle of the subscripted variable.

Note:		SUBBERSP must be called immediately after the call to create the subscripted variable.

SUBSUP

PURPOSE	SUBSUP draws a subscript and superscript.

SYNTAX	CALL SUBSUP (ISUB, NSUB, ISUP, NSUP, SIZE, ANG)

ARGUMENTS

ISUB		Hollerith variable specifying the subscript character string.

NSUB		Number of characters in the subscript.

ISUP		Hollerith variable specifying the superscript character string.

NSUP		Number of characters in the superscript.

SIZE		Height of the variable to be subscripted and superscripted.  It is not the height of the subscript and superscript characters themselves.

ANG		Angle of the subscripted and superscripted variable.

Note:	SUBSUP must be called immediately after the call to create the subscripted and superscripted variable.

SUBSUPSP

PURPOSE	SUBSUPSP draws subscripts and superscripts  of “special” characters.

SYNTAX	CALL SUBSUPSP (NSUB, NFNTSB, ITITLESB, NCHRSB, NSUP,

NFNTSP,ITITLESP, NCHRSP, HEIGHT, ANG)

ARGUMENTS

NSUB, NSUP	Number of different font sets needed for subscripts and superscripts, respectively.

NFNTSB, NFNTSP	Arrays holding the font numbers 1 to NSET for subscripts and superscripts, respectively.

ITITLESB, ITITLESP	Arrays holding the octal codes for characters of fonts 29 or 35; otherwise, holding text characters themselves.

NCHRSB, NCHRSP	Number of characters of a given font for subscripts and superscripts, respectively. Usually one, but can be greater than one for fonts other than 29 and 35.

HEIGHT		Height of the variable to be subscripted.  It is not the height of the subscript characters themselves.

ANG			Angle of the subscripted variable.

Note:  SUBSUPSP must be called immediately after the call to create the subscripted variable.

SUPER

PURPOSE	SUPER draws a superscript.

SYNTAX	CALL SUPER (ISUP, NSUP, SIZE, ANG)

ARGUMENTS

ISUP		Hollerith variable specifying the superscript character string.

NSUP		Number of characters in the superscript.

SIZE		Height of the variable to be superscripted.  It is not the height of the subscript characters themselves.

ANG		Angle of the superscripted variable.

Note:	SUPER must be called immediately after the call to create the superscripted variable.

SUPERSP

PURPOSE	SUPERSP draws a superscript of “special” characters.

SYNTAX	CALL SUPERSP (NSET, NFNT, ITITLE, NCHR, HEIGHT, ANG)

ARGUMENTS

NSET		Number of different font sets needed.

NFNT		Array holding the font numbers 1 to NSET.

ITITLE		ITITLE holds the octal codes for characters of fonts 29 or 35; otherwise, it hold the text characters themselves.

NCHR		Number of characters of a given font. Usually one, but can be greater than one for fonts other than 29 and 35.

HEIGHT	Height of the variable to be superscripted.  It is not the height of the superscript characters themselves.

ANG		Angle of the superscripted variable.

Note:  SUPERSP must be called immediately after the call to create the superscripted variable.

SYMBOL

PURPOSE	SYMBOL plots a character string. It is similar to subroutine KEKSYM, except that justification and continuation are not supported.

SYNTAX	CALL SYMBOL (XPP, YPP, HEIGHT, IBCD, ANG, NCHAR)

ARGUMENTS

XPP,YPP	X,Y coordinates of the lower left corner of the first character of the string to be plotted.

HEIGHT	Height of character string to be plotted.

IBCD		Character string to be plotted (Hollerith).

ANG		Angle, measured counterclockwise from the X-axis, at which the character string is to be plotted.

NCHAR	Number of characters in the string.